3GPP TSG-RAN WG2 Meeting #122		 R2-2206042

Incheon, Korea: 22nd May – 26th May 2023

Source:	Qualcomm Incorporated

Title:	Data collection for MRO for inter-system handover for voice fallback

Agenda Item:	7.13.2 MRO for inter-system handover for voice fallback

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

This paper discusses UE data collection for MRO for inter-system handover voice fallback.

# Discussion

2.1	MRO enhancement for inter-system handover voice fallback

Inter-system handover for voice fallback is defined as a mobility procedure where the network initiates a handover procedure to move a UE in RRC_CONNECTED state to cell using other RAT, e.g. E-UTRA, UTRA-FDD [2]. The handover, i.e., the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell.

UE accesses the target cell indicated in the inter-RAT message following the specification of the target RAT. Upon the successful completion of Mobility from NR, the NR RRC connection is released. Upon the failure of NR from mobility command, if the voiceFallbackIndication is included in the MobilityFromNRCommand, UE attempts to select an E-UTRA cell. If a suitable E-UTRA cell is selected, UE goes to the IDLE state and establishes the connection to the E-UTRA cell. Otherwise, UE performs connection re-establishment.

In the RAN2#119bis-emeeting [3], RAN2 made the following agreements:

Agreements:

1	An explicit indication is included in RLF-report when mobility from NR fails and the corresponding MobilityFromNRCommand includes voiceFallbackIndication

2	The below content is included in RLF-report when reestablishment procedure is initiated due to mobility From NR failure.

a. reestablishmentCellID

During the RAN2#121bis-emeeting online discussion [4], RAN2 made the following agreements and captured FFS items.

Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

### 2.1.1	Discussion on SON/MDT enhancements for mobility from NR procedures for emergency service fallback

Note that in Rel-16/Rel-17, UE performs RRC connection Establishment to an acceptable cell only for emergency services. In rel-16/rel-17, UE does not report information about acceptable cells, e.g.,

- UE does not log any measurement in logged MDT report when UE is in "camped on any cell state"

- UE does not log in MHI acceptable cell information and time spent on the acceptable cell

- In the RLF report, UE reports reconnectCellId in which the UE comes back to connected after connection failure and after failing to perform reestablishment. Therefore, in our understanding, it should not be an acceptable cell.

Furthermore, reporting the acceptable cell info as the reconnectCellId does not have a clear optimization objective, as the handover cannot be performed to the acceptable cell.

Observation 1: In rel-16/rel-17, UE does not report information about acceptable cells in SON/MDT reports, for example,

- UE does not log any measurement in logged measurement report when UE is in "camped on any cell state"

- UE does not log in Mobility History Information (MHI) acceptable cell information and time spent on the acceptable cell

- In the RLF report, UE reports reconnectCellId in which the UE comes back to connected after connection failure and after failing to perform reestablishment. Therefore, in our understanding, it should not be an acceptable cell.

Observation 2: As handover cannot be performed to the acceptable cell, there is no clear optimization objective of reporting acceptable cell information in the RLF report.

Proposal 1: For the emergency service fallback, UE does not log acceptable cell information in the RLF report as reconnectCellId when "HoF failure happens, no suitable EUTRA cell is found, and UE selects an acceptable cell for an emergency call".

Proposal 2: When mobility from NR procedure is performed for emergency service fallback, the UE reports the following information in the RLF report:

- If a suitable EUTRA cell is found after HOF, UE includes EUTRA cell ID as reconnectCellId.

- If a suitable EUTRA cell is not found after HOF and UE selects an acceptable cell for emergency service, UE does not log reconnectCellId and reestablishmentCellId.

- If both suitable and acceptable EUTRA cells are not found after HOF, UE logs the reestablishmentCellId.

### 2.1.2	Discussion on SON/MDT enhancements for mobility from NR procedures for voice fallback

For the normal voice fallback scenario, if the suitable EUTRA cell is not found, UE reverts to the configuration used in the source PCell. Furthermore, UE initiates a connection re-establishment procedure. Therefore, If a suitable EUTRA cell is found after HOF, UE includes EUTRA cell ID as reconnectCellId. Otherwise, if both suitable and acceptable EUTRA cells are not found after HOF, UE logs the reestablishmentCellId.

Proposal 3: When mobility from NR procedure is performed for voice fallback, the UE reports the following information in the RLF report:

- If a suitable EUTRA cell is found after HOF, UE includes EUTRA cell ID as reconnectCellId.

- If both suitable and acceptable EUTRA cells are not found after HOF, UE logs the reestablishmentCellId.

# Conclusion

Observation 1: In rel-16/rel-17, UE does not report information about acceptable cells in SON/MDT reports, for example,

- UE does not log any measurement in logged measurement report when UE is in "camped on any cell state"

- UE does not log in Mobility History Information (MHI) acceptable cell information and time spent on the acceptable cell

- In the RLF report, UE reports reconnectCellId in which the UE comes back to connected after connection failure and after failing to perform reestablishment. Therefore, in our understanding, it should not be an acceptable cell.

Observation 2: As handover cannot be performed to the acceptable cell, there is no clear optimization objective of reporting acceptable cell information in the RLF report.

Proposal 1: For the emergency service fallback, UE does not log acceptable cell information in the RLF report as reconnectCellId when "HoF failure happens, no suitable EUTRA cell is found, and UE selects an acceptable cell for an emergency call".

Proposal 2: When mobility from NR procedure is performed for emergency service fallback, the UE reports the following information in the RLF report:

- If a suitable EUTRA cell is found after HOF, UE includes EUTRA cell ID as reconnectCellId.

- If a suitable EUTRA cell is not found after HOF and UE selects an acceptable cell for emergency service, UE does not log reconnectCellId and reestablishmentCellId.

- If both suitable and acceptable EUTRA cells are not found after HOF, UE logs the reestablishmentCellId.

Proposal 3: When mobility from NR procedure is performed for voice fallback, the UE reports the following information in the RLF report:

- If a suitable EUTRA cell is found after HOF, UE includes EUTRA cell ID as reconnectCellId.

- If both suitable and acceptable EUTRA cells are not found after HOF, UE logs the reestablishmentCellId.

# Reference

[1] RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

[2] TS 38.331, “Radio Resource Control (RRC) protocol specification (Release 17)”

[3] RAN2#119bis-emeeting report

[4] RAN2#121bis-emeeting report