3GPP TSG-RAN WG2 Meeting #122		R2-2306043

Incheon, Korea: 22nd May – 26th May 2023

Source:	Qualcomm Incorporated

Title:	Discussion on NR-U Related Enhancements

Agenda Item:	7.13.5 SON for NR-U

Document for:	Discussion and Decision

# Introduction & Background

Rel-18, SON/MDT WID [1] has the following as one of the objectives,

Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PSCell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

This paper discusses SON/MDT enhancements for the NR-U (MRO and UL MLB).

# Discussion

2.1 NR-U MRO

- Logging of additional information when consistent LBT failures happen at the UE

In RAN2#121 meeting [2], RAN2 made the following agreements,

Agreements:

1: 	Log the last successful RA procedure related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

Observation 1: In the RAN2#121 meeting [2], RAN2 agreed to log the last successful RA procedure-related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

At the MAC layer, consistent uplink failure is detected per UL BWP by counting the LBT failure indication. Note that when consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select. During the RAN2#121 meeting, some companies argued to report the BWP information such that the network can optimize the configured BWPs at the UE. Note that a BWP is identified by locationAndBandwidth and subcarrierSpacing information.

Observation 2: A BWP is identified by the locationAndBanwidth and subcarrierSpacing.

Proposal 1: UE provides only the BWP information (I.e., locationAndBandwidth and subcarrierSpacing information) for previously failed RA procedures (when multiple consistent LBT failures happen).

- Logging of LBT information (per attempt)

During the RAN2#121 meeting, RAN2 discussed two options for logging the LBT-related information as the following,

a)	A random access is considered as attempted whenever PHY tries to transmit a preamble, irrespective of whether the LBT is successful or not

b)	A random-access attempt is considered as attempted only if the PHY layer actually transmitted the preamble, i.e., successful LBT

During the RAN2#121 meeting, this was further discussed through the email discussion, and the email discussion summary was captured in R2-2304200. In our view, UE should not be required to implement two different LBT failure-related information reporting methods for the last BWP and other previous bandwidth parts. Therefore, we believe that Proposal 2 and Proposal 5 in the email discussion summary report (in R2-2304200) should be discussed together.

Proposal 2	On how to represent the preamble transmission attempts blocked by LBT, RAN2 to selected one of the following solutions:

a.	Introduce a field that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching

b.	For each preamble transmission attempt included in the perRAAtttemptInfoList, include a flag indicating whether transmission failures experienced before this successful preamble transmission attempt and a flag indicating transmission failures experienced right before beam switching

Proposal 5	For the other BWPs in which the UE experienced the consistent LBT failure, the UE logs the number of LBT failures experienced in each BWP during the RA.

In our understanding, RAN2 should first discuss whether the LBT failure-related counter should only be reported for the last BWP or all previous BWPs. We further believe that for the uniformity of the solution for reporting the count of LBT failures on each BWP, we can introduce a field that counts the number of preamble transmissions blocked by LBT per RA procedure. In Proposal 2 (in R2-2304200), the objective of reporting a flag indicating transmission failures experienced right before beam switching is not clear.

Observation 3: UE should not be required to implement two different LBT failure-related information reporting methods for the last BWP and other previous bandwidth parts.

Proposal 2: RAN2 should discuss Proposal 2 and Proposal 5 in the email summary TDOC R2-2304200 together.

Proposal 3: RAN2 should first discuss whether the LBT failure-related counter should only be reported for the last BWP or all previous BWPs.

Proposal 4: If the number of LBT failures experienced in each BWP during RA is reported, then to represent the preamble transmission attempts blocked by LBT, introduce a field that counts the number of preamble transmissions blocked by LBT per RA procedure.

Proposal 5: RAN2 should further discuss the objective of reporting a flag indicating transmission failures experienced right before beam switching.

- On logging RSSI and EDT information

Note that channel access procedure, UE senses the channel and determines if the channel is free. The channel access procedure depends on sensed energy at the channel. While the measured signal energy is quantized to form the received signal strength indicator (RSSI), the channel access procedure is not directly correlated to the RSSI value. Furthermore, note that to optimize the transmission opportunity UE may configure the max configured value for the energy detection threshold (EDT).

Observation 4: Actual LBT evaluation is performed based on sensed energy on the channel, i.e., the channel access procedure is not directly correlated to the RSSI value.

Observation 5: In a practical implementation UE may use the max EDT configured at the UE, which is a cell-specific configuration and should be known to the network.

Proposal 6: The RSSI and EDT are not reported in the RA report.

Note that in the rel-16/rel-17 SON/MDT report, UE logs the measurements already available at the RRC or MAC layer. Furthermore, retrieving the information from the physical layer causes UE overheads, therefore for SON/MDT reports, UE should not be required to retrieve the information from the physical layer.

Observation 6: In the rel-16/rel-17 SON/MDT report, UE logs the measurements already available at the RRC or MAC layer.

Observation 7: Retrieving the information from the physical layer causes overheads at the UE.

Proposal 7: For SON/MDT reports, UE should not retrieve measurement information from the physical layer.

# Conclusion

Observation 1: In the RAN2#121 meeting [2], RAN2 agreed to log the last successful RA procedure-related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

Observation 2: A BWP is identified by the locationAndBanwidth and subcarrierSpacing.

Proposal 1: UE provides the BWP information (I.e., locationAndBandwidth and subcarrierSpacing information) for previously failed RA procedures (when multiple consistent LBT failures happen).

Observation 3: UE should not be required to implement two different LBT failure-related information reporting methods for the last BWP and other previous bandwidth parts.

Proposal 2: RAN2 should discuss Proposal 2 and Proposal 5 in the email summary TDOC R2-2304200 together.

Proposal 3: RAN2 should first discuss whether the LBT failure-related counter should only be reported for the last BWP or all previous BWPs.

Proposal 4: If the number of LBT failures experienced in each BWP during RA is reported, then to represent the preamble transmission attempts blocked by LBT, introduce a field that counts the number of preamble transmissions blocked by LBT per RA procedure.

Proposal 5: RAN2 should further discuss the objective of reporting a flag indicating transmission failures experienced right before beam switching.

Observation 4: Actual LBT evaluation is performed based on sensed energy on the channel, i.e., the channel access procedure is not directly correlated to the RSSI value.

Observation 5: In a practical implementation UE may use the max EDT configured at the UE, which is a cell-specific configuration and should be known to the network.

Proposal 6: The RSSI and EDT are not reported in the RA report.

Observation 6: In the rel-16/rel-17 SON/MDT report, UE logs the measurements already available at the RRC or MAC layer.

Observation 7: Retrieving the information from the physical layer causes overheads at the UE.

Proposal 7: For SON/MDT reports, UE should not retrieve measurement information from the physical layer.

# Reference

- RP-221825, “Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity)”

- RAN2#121 meeting report

- TS 38.321, “Medium Access Control (MAC) protocol specification”