3GPP TSG-RAN WG2 Meeting #122	R2- 2306045

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:		7.16.2.1

Source:		Futurewei Technologies

Title:		Discussion on the architectural and general aspects of AI/ML

Document for:		Discussion

# Abstract

In this contribution we summarized the discussions so far in both RAN1 and RAN2 on the selected topics related to the architectural and general aspects of the AI/ML SI, and provided our views on functional framework, model/functionality identification, as well as model selection/activation/deactivation/ switching/fallback.

# Functional Framework

Both RAN1 and RAN2 discussed the functional framework during RAN1 Meeting #112bis-e [1][2] and RAN2 Meeting #121bis-e, with the same proposal below.

Figure 1. Functional Framework diagram discussed at Meeting #121bis-e (no agreement)

In RAN2 discussion, the main debate was whether model storage should be in the figure (it is not in the RAN3 functional framework, which is widely considered as the starting point of RAN1 framework). At the end, no conclusion was reached on the functional framework diagram, but the group did agree on the following statement, which includes model storage as one of the components of the functional framework.

The general AI/ML framework consist of, (i) Data Collection, (ii) Model Training, (iii) Model Management, (iv) Model Inference, and (v) Model Storage.

During RAN1 at-meeting email discussion, RAN2 online discussion results of functional framework came in and affected the RAN1 discussion; it was considered as the agreement of RAN2.

In the at-meeting email discussions that followed, the following topics have been the focuses.

- Whether RAN1 should continue to discuss the function framework given RAN2 is working on it.

- Whether the storage block should be in the diagram.

- Whether new links need to be added between blocks.

Our opinions below.

First, we need to avoid parallel discussions between RAN1 and RAN2 on the same topics, otherwise we may need to spend more time addressing the discrepancies between different agreements coming out from the two groups. For the functional framework, in particular, we believe RAN1 should handle it due to the following reasons.

- The use cases involved are RAN1 use cases so RAN1 knows the situation better, such as the need for functions and signaling exchanges.

- RAN1 started the discussion much earlier than RAN2 so RAN1 participants might have better understanding of the topic.

Our feeling is that RAN1 is not far away from reaching agreements on functional framework as it has been discussing this in multiple meetings, therefore we can leave this work to RAN1.

Second, regarding whether there should be a Model Storage block, we are fine to include it because we think without such as block it is hard to present the actions of model transfer/delivery/development. Therefore, if we decide to continue the discussion of functional framework, we suggest having the model storage block in the diagram. Note that depending on the implementation and where model storage is located, the arrows between model training/inference and model storage may not be needed. For example, when model storage collocates with model training at gNB, there is no need for the delivery of trained/updated model to the model storage after training; it stays at the same entity (the gNB) where training happens.

Lastly, regarding whether additional links are needed for newly added functional blocks or existing blocks, we think additional links are necessary for newly added blocks such as model storage. We will show these new links in our proposed functional framework diagram below.

Based on the above discussions, we present our proposal of the functional framework.

Proposal 1: RAN2 to leave the definition of functional framework to RAN1.

Proposal 2: Adopt the functional framework in Figure 2 as the starting point for the AI/ML study item.

- The dash lines in the diagram indicate the operation and/or data/signaling transfer may or may not be needed depending on where the model is stored in the network.

Figure 2. Proposed AI/ML functional framework

# Model and Functionality Identifications

## Background

In this section, current activities and status of both RAN1 and RAN2 on the subjects will be presented as the background information to facilitate the discussion. We hope this information is helpful for our colleagues who did not participate in RAN1 discussions.

### RAN1 Agreements and Conclusions

Model identification and functionality identification were the two major topics that have been heavily discussed in RAN1 both during the meetings and after the meetings in emails.

Meeting #112bis-e

During Meeting #112bis-e[1][2], RAN1 achieved the following agreements on the architecture and general aspects.

Agreement: On definitions of functionality identification and model identification and related LCMs.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

  - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

- For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models:

  - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

  - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda.

  - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s).

  - FFS: relationship between functionality-based LCM and model-ID-based LCM

- Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

Agreement: On UE reporting of applicable functionalities and UE part/UE-side models

- Study necessity, mechanisms, after functionality identification, for UE to report updates on applicable functionality(es) among [configured/identified] functionality(es), where the applicable functionalities may be a subset of all [configured/identified] functionalities.

- Study necessity, mechanisms, after model identification, for UE to report updates on applicable UE part/UE-side model(s), where the applicable models may be a subset of all identified models.

Conclusion: On logical model ID

  - From RAN1 perspective, it is clarified that an AI/ML model identified by a model ID may be logical, and how it maps to physical AI/ML model(s) may be up to implementation.

  - When distinction is necessary for discussion purposes, companies may use the term a logical AI/ML model to refer to a model that is identified and assigned a model ID, and physical AI/ML model(s) to refer to an actual implementation of such a model.

Meeting #112

During Meeting #112[3][4], RAN1 achieved the following agreements on the topic.

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Meeting #111

In meeting #111, the group agreed to study two types of LCM procedures, functionality-based procedure, and model-ID-based procedure. Two working assumptions of the terms “Model identification” and “Functionality identification” have also been made. However, both terms were defined as processes or methods. The “ID” itself has not been defined; in other words, they have been defined for their usage (a process/method), but not about what they are. The agreements and working assumptions are listed below.

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality.

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Functionality

For model-ID-based LCM procedure, indication of model selection/activation/deactivation/ switching/fallback based on individual model IDs

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

### RAN2 Agreements and Conclusions

In RAN2 meeting #121bis-e, the following have been agreed on topics related to the general aspects.

On Model ID

Model ID can be used to identify model or models for the following LCM purposes:

- model selection/activation/deactivation/switching (or identification, if that will be supported as a separate step).

- (e.g. for so called “model ID based LCM”)

If model transfer/delivery is supported, model ID can be used for model transfer/delivery LCM purpose.

How to achieve globality of the Model ID is FFS.

- Initial discussion in RAN2: the following global unique model ID definition directions can be considered as a starting point:

- Direction1: Pre-defined/hard-coded global unique model ID

- Direction3: Assigned global unique model ID via specific ID management node.

- Note: Other global unique model ID definition is not precluded.

- Model ID structure, if any, is FFS.

In meetings before RAN2 meeting 121bis-e, RAN2 agreed on the following about model ID.

- R2 assumes that a model is identified by a model ID. Its usage is FFS.

- R2 assumes that from Management or Control point of view mainly some meta info about a model may need to be known, details FFS.

- RAN2 assumes that Model ID is unique “globally”, e.g., in order to manage test certification each retrained version need to be identified.

## The Definition of Model ID

Model ID has not been defined in either RAN1 or RAN2. Our view is that a model ID is a unique index/number that differentiates one model from other models within a network, in a way just like a phone number.

“Globally Unique” is a desirable feature to have for model ID but it may be difficult to obtain and may also have some disadvantages. For example, some companies proposed to use UUID, which is 128-bit long with multiple variants. For model identification purpose, 128 bits may be too long as it brings extra overhead for model LCM. Therefore, local ID should also be supported. For the use of local ID, the network boundary within which model ID is unique can be flexible. For example, it could be one carrier’s nation-wide network, a metropolitan network, or even smaller networks for smaller operators. Within the same network, a model ID can unambiguously identify an AI/ML model for the common understanding between the NW and the UE, achieving the goal of model identification.

Proposal 3: A model ID is a unique index that differentiates one model from other models within a network. The model IDs may or may not be globally unique.

## Meta information

Model ID alone does not tell us everything about a model; a model needs to be associated with some supplemental information, which is called meta information in both RAN1 and RAN2. Meta information about the model can be provided during model registration (to be defined later) and/or model identification processes. The examples of meta information could be applicable sub-use-case(s) and conditions, supported features/functionalities, version number, parameter information etc. Meta information can be stored within any entity that it is needed.

One of the important functions of meta information is to keep the model ID simple; by keeping all other information in meta information, model ID can be a simple, structure-less, index number. As we expect model ID will be used and exchanged among different entities more frequently than other information about the model, this will save on signaling overhead.

Proposal 4: Each model ID should be associated with a list of meta information that describes the functionalities, associated features, and other characteristics etc. of the model.

## IDs for two-sided models

During the discussion after meeting #112, an issue with IDs for two-sided models was raised. The basic question was, in the case of two-sided model, when one side of the model is updated and assigned a new ID, should the other side be updated with the same new ID too (in this case model on the other side has not changed)?

A further related question was, if the model on the other side is still compatible with the updated model, should both sides use the same new ID, or the other side can keep its old ID?

In our view, for two-sided models, whenever an update happens on either side of the model, the changes need to be made known to the other side before the operation of the updated models; that is, all information associated with the model in use needs to be synced.

Observation 1: There are two cases for the development of two-sided models.

- In the first case, each side has only one model. In this case, the relationship is one-to-one, and the two sides may share the same model ID.

- In the second case, one NW-side model may be matched to multiple UE-side models, or one UE-side model may be matched to multiple NW-side models. In this case it is not easy, if not impossible, for the two sides to share the same model ID.

## Functionality Identification

During meeting #112bis-e, the group reached the following agreement related to functionality identification.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models:

  - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability.

  - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

    - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities

    - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level

    - FFS: Other aspects that may constitute Functionality

FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda.

Based on this definition, functionality is essentially AI/ML-enabled Feature/FG which can be configured through UE capability.

Going back to meeting #112, the group agreed that for AI/ML functionality identification, reuse legacy 3GPP framework of Features as a starting point for discussion. 3GPP TR 38.822 [5] provides the list of UE features for NR (also specified in 3GPP TS 38.306[6]). It is our understanding that AI/ML related features will be part of the list in the future. Therefore, functionality identification is a process that identifies which of the Feature(s)/FG(s) in the feature list an AI/ML model supports.

Figure 4. Partial feature list for Mobility Enhancement in [3]

Take the feature list for Mobility Enhancement from [5] for example (as shown in Figure 4), the feature of Mobility Enhancement has an index of 21. This feature has multiple Feature Groups, and its first feature group has an index of 21-1a, which, in turn, contains multiple components. In the original table, each feature group has other information too but is not shown here due to the limit of the page width.

We envision that for the AI/ML related features, such a feature list (or a similar one) is also necessary. That implies all the features and their related information need to be defined offline (e.g., in 3GPP) and clearly listed in the table before a model can identify its functionality using this table.

In meeting 112bis-e, one of the aspects that has not been decided is whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level.  These are considered the dynamic parts of the functionality that may be too early to decide at this stage.

The request of pre-defined AI/ML related features in the standard and the fast pace of the development in the field of AI/ML, making it difficult to ensure the standard to be future proof. We envision that there may be a need to define some placeholders for TBD features and/or user-defined feature so that new features can be added between standard releases and be considered standard compatible at some point.

But even with the measures of future-thinking, there are still many aspects related to functionality ID that are not clear to us and, based on our observations, would take the group huge effort to sort it out (if possible). Some of our concerns are listed here.

- Although the group decided to use legacy 3GPP framework of Features as a starting point for the study of functionality identification, there is not a well-defined “3GPP framework of Features”.

- There is not clear understanding of the relationship between model Functionality and the Feature/FG. For example, how to link a Functionality to multiple UE Features/FGs? Note the agreement only says that “functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG”. Then, can we link one Functionality ID to multiple Features/FGs?

- It would be hard to define Functionality without the definitions of each Feature if Functionalities will be based on Features.

Based on this thinking and given that we don’t have much time left for the SI phase, we would suggest the group focusing on model identification first and defer the study of functionality identification.

Proposal 5: RAN2 to focus on model identification in the SI phase and defer the study of functionality identification details till Rel-19 work item phase.

# Model selection/activation/deactivation/switching/fallback

In RAN2 meeting #121bis-e, the following agreements were reached on model selection/activation/ deactivation/switching/fallback.

For the CSI compression and beam management use cases, model/function selection/(de)activation/switching/fallback can be UE-initiated or gNB-initiated.

    - FFS how the different cases are different (e.g., applicability to UE-sided vs network sided model).

For the positioning use case, model/function selection/(de)activation/switching/fallback can be UE-initiated or LMF-/gNB-initiated.

    - FFS how the different cases are different (e.g., applicability to UE-sided vs network sided model).

In RAN1 meeting #110bis-e, the following agreement was reached.

RAN1 #110bis-e Agreement

For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms:

- Decision by the network

  - Network-initiated

  - UE-initiated, requested to the network

- Decision by the UE

  - Event-triggered as configured by the network, UE’s decision is reported to network

  - UE-autonomous, UE’s decision is reported to the network

  - UE-autonomous, UE’s decision is not reported to the network

FFS: for network sided models

FFS: other mechanisms

As we can see from the above, both RAN1 and RAN2 have discussed the topic.

- RAN2 concluded on which side can initiate the action(s), for all inference types (NW-side model, UE-side mode and two-sided models)

- RAN1 concluded not only on which side can initiate the action(s), but also which side can make the decisions, but only for UE sided models and two-sided models (whether it is for NW-side model is FFS).

In meetings after RAN1 #110bis-e, there were a good number of proposals require UE to either report the operation decision to NW to get approval before executing the action, or to limit the actions UE can execute if UE make the decision solely on itself. The concerns were that, if the UE makes its own decision and executes it without the approval from the NW, there could be following issues.

- The UE may not be aware of all aspects impacting the AI/ML model operation so the decision may not be right.

- The NW could suffer unidentified network performance fluctuation/loss due to UE’s decision, especially if the UE does not report to the NW what it has done.

- The NW is not aware of the change of the model input/output, which may result in mismatched RS configurations and/or mismatched content/payload size of the expected UE report.

Further discussion on this topic is currently on hold in RAN1. We expect debates on whether UE can make its own decisions and execute it in RAN1’s future meetings. Therefore, it is better to postpone RAN2’s discussion and wait for RAN1’s decision to avoid conflict decisions.

Proposal 6: RAN2 to wait for RAN1’s decisions on whether UE can decide on the operation of model selection/activation/deactivation/switching/fallback and execute it without NW’s approval.

# Conclusions

In this contribution, we continue to present our views on architectural and general aspects. Based on the discussions in the previous sections, our proposals and observations are as follows.

Proposal 1: RAN2 to leave the definition of functional framework to RAN1.

Proposal 2: Adopt the functional framework in Figure 2 as the starting point for the AI/ML study item.

- The dash lines in the diagram indicate the operation and/or data/signaling transfer may or may not be needed depending on where the model is stored in the network.

Figure 2. Proposed AI/ML functional framework

Proposal 3: A model ID is a unique index that differentiates one model from other models within a network. The model IDs may or may not be globally unique.

Proposal 4: Each model ID should be associated with a list of meta information that describes the functionalities, associated features, and other characteristics etc. of the model.

Observation 1: There are two cases for the development of two-sided models.

- In the first case, each side has only one model. In this case, the relationship is one-to-one, and the two sides may share the same model ID.

- In the second case, one NW-side model may be matched to multiple UE-side models, or one UE-side model may be matched to multiple NW-side models. In this case it is not easy, if not impossible, for the two sides to share the same model ID.

Proposal 5: RAN2 to focus on model identification in the SI phase and defer the study of functionality identification details till Rel-19 work item phase.

Proposal 6: RAN2 to wait for RAN1’s decisions on whether UE can decide on the operation of model selection/activation/deactivation/switching/fallback and execute it without NW’s approval.

# References

R1-2304168, Session notes for 9.2 (Study on AI/ML for NR air interface), RAN1 Meeting #112bis-e

R1-2304052, Summary of General Aspects of AI/ML Framework, Feature Lead, RAN1#112bis-e.

R1-2302063, Session notes for 9.2 (Study on AI/ML for NR air interface), Meeting #112

R1-2210661, Summary of General Aspects of AI/ML Framework, Feature Lead, RAN1#110bis-e.

3GPP TS 38.822, NR User Equipment (UE) feature list (Release 16).

3GPP TS 38.306, User Equipment (UE) radio access capabilities (Release 17)