3GPP TSG-RAN WG2 Meeting #122	R2-2306047

Incheon, May 22 – 26, 2023

Agenda item:	7.11.2.1

Source:	Sharp

Title:	Notification of Multicast session deactivation/temporary no data in enhanced group paging message

WID/SID:	NR_MBS_enh-Core

Document for:	Discussion and Decision

1	Introduction

In this discussion paper, we considered how to notify UE about the session deactivation or temporary no data.

2 Discussion

In RAN2 #121b meeting, RAN2 has agreed that enhanced group paging is used to indicate session activation or data transmision resumed. And after receiving the enhanced group paging, UE starts monitoring corresponding G-RNTI if the MBS session is indicated as activated or data transmission resumed and the PTM coniguration is avaliable [1] :

Rel-18 UE can stay in RRC_INACTIVE and start monitoring corresponding G-RNTI upon an enhanced group paging (e.g., upon session activation or data transmission resumed). Details FFS.

Upon events like session activation/data transmission resumed, if PTM configuration is not available to UE, UE initiates RRC connection resumption.

Considering the enhanced group paging indicating either activation or data transmission resumed just triggerring UE to start monitoring the G-RNTI of the MBS session. From UE perspective, there is no difference between the indication of activation and data transmission resumed. So, the enhanced group paging carries an indication to trigger UE to start monitoring the G-RNTI for the MBS session actviated or data transmssion resumed in a case that the PTM configuration is avaiable but does not need to indicate exactly the activation or data transmission resumed for the MBS session.

Observation 1 : The enhanced group paging does not need to differentiate the activation or data transmission resumed for MBS session.

And for the case of session deactivation or temporary no data, it was agreed that UE should stop monitoring the MBS session that indicated as deactivation or temporary no data [1]:

For one UE already in RRC_INACTIVE, it can stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

Similar to activation/data transmission resume case, upon receiving either session deactivation or temporary no data indication, UE just stops monitoring G-RNTI of the MBS session. From UE perspective, there is no difference between the indication of deactivation and temporary no data. So, the indication used trigger UE to stop monitoring the G-RNTI for the MBS session deactviated or temporary no data does not need to indicate exactly session deactivation or temporary no data for the MBS session.

Observation 2 : The indication used to trigger UE to stop monitoring the G-RNTI for the MBS session deactviated or temporary no data does not need to indicate exactly session deactivation or temporary no data for the MBS session.

About how to indicate the deactivation or temporary no data for the MBS session is still FFS, but two options are provided:

Option 1 : Indicate the deactivation or temporary no data in enhanced group paging.

Option 2 : Indicate the deactivation or temporary no data in enhanced MCCH.

If option 2 is used, UE should always monitor MCCH for the deactivation/temporary no data of MBS session, which is not a power efficient way. And how to carry the indication via MCCH also need to be further discussed, for example, carried in DCI or MCCH message.

Considering that we anyway need to design the indication for the case of activation/data transmission resume. It is straightforward and a unified way to carry the indication in the enhanced group paging to trigger UE to stop monitoring the G-RNTI for the MBS session deactivated or temporary no data. Based on observation 1&2, the indication used to indicate activation/data transmission resume can also be used to indicate deactivation/temporary no data by setting a different value.

Proposal 1: Group paging caries an indication to trigger UE to stop monitoring the G-RNTI for the MBS session deactviated or temporary no data.

3 Summary

In this section, we summarized our proposals and observation as showed below:

Observation 1 : The enhanced group paging does not need to differentiate the activation or data transmission resumed for MBS session.

Observation 2 : The indication used to trigger UE to stop monitoring the G-RNTI for the MBS session deactviated or temporary no data does not need to indicate exactly session deactivation or temporary no data for the MBS session.

Proposal 1: Group paging carries an indication to trigger UE to stop monitoring the G-RNTI for the MBS session deactviated or temporary no data.

4 References

[1] RAN2#121bis-e chairman’s notes.