3GPP TSG-RAN WG2 Meeting #122	R2-2306051

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item:	7.4.2.1

Source: 	Fujitsu

Title:	Failure detection and fast recovery

Document for:	Discussion and Decision

- Introduction

In the RAN2#121bis-e meeting, the following agreements were made [1]

- LTM cell switch is supervised by a timer Following behaviors of LTM supervisor timer are agreed: - - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) - LTM supervisor timer is RRC layer timer. - At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution.

This document discusses some detail on failure detection and fast recovery.

- Discussions

2.1	Support of fast recovery

In the RAN2#121bis-e meeting, it was discussed whether the UE can perform LTM execution to the selected cell at RLF or LTM execution failure, if the UE performs cell selection and the selected cell is an LTM candidate cell [2]. This approach is beneficial for recovering RRC connection at RLF or LTM and similar approach has been introduced in CHO. We propose that RAN2 should confirm support fast recovery to a candidate cell at RLF and LTM execution failure.

Proposal 1: RAN2 confirms to support fast recovery to a candidate cell at RLF and LTM execution failure.

However, details of fast recovery need to be studied. We discuss some details in the following sub clauses.

2.2	Fast recovery high level procedures

One of details RAN2 needs to discuss is how to avoid key stream reuse issue in fast recovery at LTM execution failure. For Rel-16 CHO, key stream reuse issue on HOF handling has been pointed out and discussed [3]. The conclusion of the discussion was captured in TS 38.331 as follows.

#### 5.3.7.3 Actions following cell selection while T311 is running - … NOTE 1: It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

This means, e.g., The network always sets masterKeyUpdate in condRRCReconfig if attemptCondReconfig is set in ConditionalReconfiguration [3].

Since RAN2 already agreed that no security update support in Rel-18 with LTM, the same approach cannot be applied for LTM execution failure case.

Observation 1: In CHO based recovery, security key update is used for avoiding key stream reuse issue. But the same approach cannot be reused for LTM fast recovery.

However, the main problem for occurring key stream reuse in CHO based recovery is the UE reverts back to the source PCell configuration at the HOF. When the UE reverts back to the source PCell configuration the COUNT values are also reverted if CBRA was used for the failed handover, and if the selected cell is one of the candidate cells, the UE transmits RRCReconfigurationComplete message with the reverted COUNT value if the security key is not updated. This results in key stream reuse issue.

Observation 2: The main problem for occurring key stream reuse in CHO based recovery is when the UE reverts back to the source PCell configuration at the HOF, the COUNT value is also reverted if CBRA is used for the failed handover.

Therefore, if the UE can avoid reverting the COUNT values at LTM execution failure, the potential key stream reuse issue at fast recovery will be solved. We propose that after LTM execution failure, the UE performs cell selection without reverting back to the source configuration, i.e., with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE reverts back to the source PCell configuration and prepares RRCReestablishmentRequest message transmission

Proposal 2: After LTM execution failure, the UE performs cell selection without reverting back to the UE configuration used in the source PCell, i.e., with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE reverts back to the source PCell configuration and prepares RRCReestablishmentRequest message transmission

For RLF, the UE originally keeps current configuration (if attemptCondReconfig is configured) and performs cell selection. Therefore, we propose that after RLF, the UE performs cell selection with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE prepares RRCReestablishmentRequest message transmission

Proposal 2a: After RLF, the UE performs cell selection with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE prepares RRCReestablishmentRequest message transmission.

2.3	Latency reduction for fast recovery

Since the objective of LTM is mobility latency reduction, RAN2 may need to study latency reduction for fast recovery. According to current CHO based recovery mechanism, RRC re-establishment procedure is initiated after an RRC LTM timer expires, and cell selection is performed in accordance with the cell selection process as specified in TS 38.304. The cell selection process may increase the service interruption time. Therefore, an optimized mechanism or a new mechanism of the cell selection for LTM fast recovery may need to be studied.

Observation 3: The current cell selection process may increase the service interruption time.

Proposal 3: RAN2 to discuss whether an optimized mechanism or a new mechanism of the cell selection for LTM fast recovery will be necessary.

One possible optimized mechanism will be the UE evaluates the candidate cells first, and if no suitable cell is found form the candidate cells then evaluate other cells. With this, the UE has higher possibility to perform LTM execution to the candidate cell and thus the interruption time incurred by normal cell selection process can be reduced.

Also, for a possible new mechanism, a simple solution will be that the UE performs LTM execution to a cell in the candidate configurations. Some condition can be specified for the candidate cell, e.g., the quality of the candidate cell is better a configured threshold or is better than the source cell. Considering there can be interference for UL transmission, a configured threshold is a feasible option.

Proposal 4: If Proposal 3 is agreed, the following optimized/new mechanisms should be studied:

- Optimized mechanism: candidate cell has higher priority than cells without candidate cell configuration

- New mechanism: If L1-RSRP for a candidate cell is available and is better than a threshold, the UE performs LTM execution on the candidate cell

If LTM fast recovery procedure has big difference from CHO based recovery, it may be better to define a new procedure than reuse the RRC re-establishment procedure. Therefore, we propose that RAN2 to discuss whether the existing RRC re-establishment procedure is reused or a new procedure is defined for the LTM fast recovery.

Proposal 5: RAN2 to discusses whether the existing RRC re-establishment procedure is reused or a new procedure is defined for the LTM fast recovery.

2.4	Applicable scenarios/Triggers for fast recovery

For CHO, the CHO based recovery is applied not only for CHO failure but also for normal handover failure. Since RAN2 agreed that the UE could also execute any L3 handover command sent by the network while configured with LTM candidate cells, RAN2 needs to discuss whether the fast recovery to a candidate cell will be applied after normal HOF (reconfiguration with sync failure).

Proposal 6: RAN2 discusses whether the fast recovery to a candidate cell will be applied after normal HOF (reconfiguration with sync failure) like CHO based recovery case.

LTM cell switch is supervised by an RRC layer timer. If the RRC layer timer for MCG expires, as agreed, the UE considers LTM failure. The LTM supervisor timer is started upon reception of the LTM cell switch MAC CE and is stopped upon successful completion of LTM cell switch. After receiving the LTM cell switch MAC CE, the UE may perform validity check, which triggers re-establishment. If fast recovery mechanism is introduced, we need to discuss whether it applies to validity check failure.

Proposal 7: RAN2 discusses whether the fast recovery to a candidate cell will be applied at the validity check failure of the target cell if the validity check is performed after receiving LTM cell switch MAC CE.

Regarding RLF, it is natural that all RLF triggers applies. One question is whether early RACH procedure on candidate cell for TA acquisition triggers the RLF. If current RACH procedure with RAR receiving from serving cell is used for early TA acquisition, RA problem may occur. However, if the serving cell works well, it is not reasonable to trigger RLF due to RA problem on candidate cell. In this case, the radio link quality on the serving cell is good enough for data transmission but RA problem on candidate cell occurs. This should be discussed in RAN2.

Proposal 8: RAN2 to discuss how to handle the RACH failure on candidate cell.

The straightforward method is the MAC will not indicate RA problem to RRC if RA failure towards a candidate cell occurs. Alternatively, RLF is triggered by RA problem on serving cell rather than candidate cell.

In addition, RLM on candidate cell can be discussed considering that RLF on candidate cell may occur immediately after LTM cell switch, which is not expected.

Proposal 9: RAN2 to discuss whether RLM on candidate cell is needed.

- Conclusion

Observation 1: In CHO based recovery, security key update is used for avoiding key stream reuse issue. But the same approach cannot be reused for LTM fast recovery.

Observation 2: The main problem for occurring key stream reuse in CHO based recovery is when the UE reverts back to the source PCell configuration at the HOF, the COUNT value is also reverted if CBRA is used for the failed handover.

Proposal 2: After LTM execution failure, the UE performs cell selection without reverting back to the UE configuration used in the source PCell, i.e., with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE reverts back to the source PCell configuration and prepares RRCReestablishmentRequest message transmission

Proposal 2a: After RLF, the UE performs cell selection with keeping the current configuration, and:

- if selected cell is one of PCell of the candidate configurations, the UE replaces the current configuration with the target complete configuration and perform LTM cell switch.

- else, the UE prepares RRCReestablishmentRequest message transmission.

Observation 3: The current cell selection process may increase the service interruption time.

Proposal 3: RAN2 to discuss whether an optimized mechanism or a new mechanism of the cell selection for LTM fast recovery will be necessary.

Proposal 4: If Proposal 3 is agreed, the following optimized/new mechanisms should be studied:

- Optimized mechanism: candidate cell has higher priority than cells without candidate cell configuration

- New mechanism: If L1-RSRP for a candidate cell is available and is better than a threshold, the UE performs LTM execution on the candidate cell

Proposal 5: RAN2 to discusses whether the existing RRC re-establishment procedure is reused or a new procedure is defined for the LTM fast recovery.

Proposal 6: RAN2 discusses whether the fast recovery to a candidate cell will be applied after normal HOF (reconfiguration with sync failure) like CHO based recovery case.

Proposal 7: RAN2 discusses whether the fast recovery to a candidate cell will be applied at the validity check failure of the target cell if the validity check is performed after receiving LTM cell switch MAC CE.

Proposal 8: RAN2 to discuss how to handle the RACH failure on candidate cell.

Proposal 9: RAN2 to discuss whether RLM on candidate cell is needed.

# References

- RAN2#121bis-e Chair Note EOM

- R2-2304214, “Summary of [AT121bis-e][018][eMob] Procedure Consolidation (Huawei)”

- R2-2101900, “Report of [Post112-e][254][R16 MOB] Issue on failure handling of handover without key change for the UE configured with attemptCondReconfig (Sharp)”