# 3GPP TSG RAN WG2 Meeting #122                          	    R2-2306057
Incheon, Korea, May 22-26, 2023

Agenda item:	7.15.6

Source:	Qualcomm Incorporated

Title:	Discussion on SL CA

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss design consideration for sidelink carrier aggregation (SL CA).

# Discussion

For NR sidelink CA, it’s scoped that only LTE sidelink CA features are supported (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, power control for simultaneous sidelink TX, packet duplication) for resource allocation mode 2.

Based on LTE sidelink CA design, the following agreements were made at RAN2 #121-bis e-meeting [1].

Agreement:

Proposal 2. Support one independent HARQ entity per carrier used for NR sidelink communication and one transport block is generated per carrier.

Proposal 3. Support that each transport block and its retransmissions are mapped to a same single carrier.

Agreement:

Proposal 3:	For groupcast/broadcast, as in LTE SL CA, the carrier(s) that can be used for transmitting data are configured by V2X layer for the L2 destination. FFS on backwards compatibility issue.

Agreement

Proposal 8: Packet duplication for NR sidelink is performed at the PDCP layer. The duplicated PDCP PDUs of the same PDCP entity are submitted to two different RLC entities and associated to two different sidelink logical channels respectively.

Proposal 9: RAN2 agrees that LCH mapping restriction shall be defined such that the duplicated PDCP PDUs of the same PDCP entity are only allowed to be transmitted on different NR sidelink carriers.

Agreement:

Proposal 16: For NR sidelink PDCP duplication, reuse the hard-coded way for paired sidelink LCID to identify duplicated sidelink LCHs (i.e., for a unified design for all Bcast/Gcast). The specific SL LCID values occupied are left to Stage-3. FFS on Unicast case.

Agreement:

Proposal 10: For TX carrier (re)selection triggers in NR sidelink CA, reuse the triggers for TX carrier (re)selection per sidelink process in LTE sidelink CA as follows at least for GC/BC, if the resource (re)selection is triggered with the sidelink process. If there is no sidelink grant associated with the sidelink process on any carrier allowed for the STCH as indicated by upper layers (i.e., RRC layer and V2X layer).

FFS on unicast case.

Agreement:

Proposal 7	For LCP, only allow the LCHs having a priority whose associated CBR threshold for reselection is no lower than the CBR of the carrier when the carrier is (re-)selected. FFS on how to determine the per-carrier CBR at least for GC/BC.

FFS on unicast case.

Agreement:

Proposal 5	NR SL CA TX carrier (re)selection follows LTE CA solution, i.e., define 1) per-carrier-per-priority CBR threshold for carrier (re)selection, and 2) per-carrier-per-priority CBR threshold for carrier keeping. And final carrier selection is done based on the lowest CBR value across carriers. Where the priority is the LCH priority.

FFS on unicast case.

Agreement:

Proposal 1: Based on observation that section 6.1.2.12 of TS 24.587-v18.0.0 has captured V2X layer can be provisioned with service to frequency mapping for unicast. RAN2 assume it is applicable to PC5 unicast SL CA after link has been established. RAN2 notify SA2 this assumption and ask their input on identified questions.

In this contribution, we further discuss design considerations for sidelink CA.

## Sidelink carrier allocation for PC5-S and PC5-RRC

It was agreed that carrier(s) for transmitting data are configured by V2X layer for the L2 destination for broadcast and groupcast, and carrier(s) provisioned with service for unicast may also be assumed for unicast after PC5 link establishment. However, for PC5-S and PC5-RRC messages carried on SCCH, the mapping with carrier(s) needs to be discussed with different cases.

For the initial message (e.g., DCR message) for PC5 connection establishment procedure, a UE may transmit the initial message with a default or common carrier which may be pre-configured or configured or with one of the carriers indicated from the upper layer based on the mapping with a unicast service. Alternatively, to ensure all UEs (supporting or not supporting sidelink CA) participating in a unicast service, a UE may transmit the initial message on both carriers which are mapped with the unicast service.

During the PC5 connection establishment or after the PC5 connection establishment, UEs may exchange messages on carrier(s) to be used for PC5 RRC messages, for example, based on UE’s capability of carrier(s) or UE’s preferred carrier(s).

Proposal 1. RAN2 needs to discuss how to allocate an SCCH transmission on a carrier.

## PDCP duplication

For LTE sidelink CA, PDCP duplication is based on if the ProSe Per-Packet Reliability (PPPR) value is lower than the configured PPPR threshold threshSL-Reliability. Similarly, for NR sidelink CA, PDCP duplication may be based on NR PC5 QoS parameter for reliability. For example, Packet Error Rate (PER) as a QoS parameter for reliability may be considered for PDCP duplication.

Proposal 2. Following LTE sidelink CA framework, support that Packet Error Rate (PER) is used for PDCP duplication.

## Backward compatibility issue

Since a Rel-16/Rel-17 UE do not support sidelink carrier aggregation and can operate with only one carrier, the Rel-16/Rel-17 UE may not be able to receive all TB transmissions which are broadcasted or groupcasted by a Rel-18 UE using sidelink CA with more than one sidelink carrier. How to ensure that the Rel-16/Rel-17 UE will not miss any transmission broadcasted or groupcasted from Rel-18 UE needs to be addressed at RAN2. For example, the framework for supporting Rel 17 sidelink backward compatibility may be adopted for sidelink CA backward compatibility, i.e., via the indication in Tx Profile.

Proposal 3. RAN2 needs to discuss how to support backward compatibility for sidelink CA.

# Conclusion

In this contribution, we discussed RAN2 scope on sidelink carrier aggregation (CA) and concluded with the following proposals.

Proposal 1. RAN2 needs to discuss how to allocate an SCCH transmission on a carrier.

Proposal 2. Following LTE sidelink CA framework, support that Packet Error Rate (PER) is used for PDCP duplication.

Proposal 3. RAN2 needs to discuss how to support backward compatibility for sidelink CA.

# References

[1] Session chair’s meeting note, “RAN2-121bis-e_R16_NR SL_R17&R18_NR SL enh_20230425-EOM_r1”, 3GPP TSG RAN2 #121-bis e-meeting, April 17 – 26, 2023.