# 3GPP TSG RAN WG2 Meeting #122                           	    R2-2306058
Incheon, Korea, May 22-26, 2023

Agenda item:	7.15.7

Source:	Qualcomm Incorporated

Title:	Discussion on SL coexistence for LTE V2X and NR V2X

WID/SID:	NR_SL_enh2 – Release 18

Document for:	Discussion and Decision

# Introduction

The objective of co-channel coexistence for LTE sidelink and NR sidelink was updated at RAN #99 [1]

as the follows.

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this contribution, we discuss possible RAN2 work to support co-channel coexistence for LTE V2X and NR V2X per the latest objective of the co-channel coexistence on sidelink.

# Discussion

Based on the updated objective of co-channel coexistence for LTE sidelink and NR sidelink, resource selection for NR sidelink needs to consider co-existence with LTE to minimize the impact to LTE V2X.

## NR transmissions without HARQ feedback

With the same numerology, e.g., 15kHz sub-carrier space (SCS), LTE V2X and NR V2X have the same slot boundary, thus the current slot based resource selection for an NR V2X transmission or retransmission(s) without HARQ feedback may still work while co-existing with LTE V2X in a slot.

Proposal 1. For the numerology of 15kHz SCS, no RAN2 work is needed on resource selection for NR V2X transmission or retransmission(s) without HARQ feedback while co-existing with LTE V2X in a slot.

With different numerologies, e.g., 15kHz SCS for LTE and 30kHz SCS for NR, LTE V2X and NR V2X have the same subframe boundary but different slot boundaries. For example, there are two NR slots for each LTE slot, and thus only the odd number NR slots (e.g., the first, third, fifth, and so on) are aligned with LTE slots. In this regard, to meet the following requirement described in the objective, RAN2 needs to discuss how to support NR V2X transmission or retransmission(s) without HARQ feedback while co-existing with LTE V2X in a subframe.

Proposal 2. For the numerology of 30kHz SCS, RAN2 needs to discuss how to support NR V2X transmission or retransmission(s) without HARQ feedback while co-existing with LTE V2X in a subframe.

## NR transmissions with HARQ feedback

LTE V2X supports only blind retransmissions, but NR V2X supports both blind retransmissions and HARQ feedback retransmissions. To minimize NR V2X HARQ feedback transmission impact to an LTE V2X transmission (e.g., the PSFCH transmitted at the end of a slot shared with an LTE V2X transmission), the following requirement is outlined in the objective also.

To avoid NR V2X HARQ feedback transmission impact to an LTE V2X transmission, a simple solution is that the HARQ feedback is disabled for an NR V2X retransmission while sharing a slot with an LTE V2X transmission.

Proposal 3. RAN2 needs to discuss how to disable NR V2X HARQ feedback for retransmission(s) if the feedback transmission overlaps with an LTE V2X transmission in a slot.

If NR V2X HARQ feedback cannot be dynamically disabled based on that a HARQ feedback overlaps with an LTE V2X transmission, then resource selection needs to consider how to allocate the resources for NR initial transmission and associated HARQ feedback based retransmission(s) to avoid the overlapping between an NR V2X HARQ feedback transmission and an LTE V2X transmission in a slot.

Proposal 4. if NR V2X HARQ feedback cannot be dynamically disabled, RAN2 needs to discuss how to avoid the overlapping between an NR HARQ feedback transmission and an LTE transmission in a slot.

# Conclusion

In this contribution, we discussed possible RAN2 work to support co-channel coexistence for LTE V2X and NR V2X and concluded with the following proposals.

Proposal 1. For the numerology of 15kHz SCS, no RAN2 work is needed on resource selection for NR V2X transmission or retransmission(s) without HARQ feedback while co-existing with LTE V2X in a slot.

Proposal 2. For the numerology of 30kHz SCS, RAN2 needs to discuss how to support NR V2X transmission or retransmission(s) without HARQ feedback while co-existing with LTE V2X in a subframe.

Proposal 3. RAN2 needs to discuss how to disable NR V2X HARQ feedback for retransmission(s) if the feedback transmission overlaps with an LTE V2X transmission in a slot.

Proposal 4. if NR V2X HARQ feedback cannot be dynamically disabled, RAN2 needs to discuss how to avoid the overlapping between an NR HARQ feedback transmission and an LTE transmission in a slot.

# References

[1] RP-230077 “WID revision: NR sidelink evolution”, 3GPP TSG RAN Meeting #99, Rotterdam, Netherlands, March 20 – 23, 2023.