3GPP TSG-RAN WG2 Meeting #122	R2-2306068

Incheon, Korea, 22 - 26 May, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on SSB-less SCell operation

Agenda Item:	7.3.3

Document for:	Discussion and decision

Introduction

During the Study Item phase, inter-band SSB-less was discussed. RAN2 agreed that the SCell without SSB in intra-band CA is considered as baseline. The feasibility is left to RAN1, but RAN2 did not identify any blocking point during the Study Item phase. The following descriptions were captured in the 3GPP TR 38.864 [1]:

The SCell without SSB in intra-band CA is considered as baseline, i.e., for a serving cell without transmission of SS/PBCH blocks, a UE acquires time and frequency synchronization with the serving cell based on receptions of SS/PBCH blocks on the SpCell or the SCell, of the cell group. More detailed discussion on higher layer procedures for RAN2 may be needed in WI phase according to the other WGs input. Feasibility of this solution is in RAN1 scope.

In the RAN #98e meeting, the Work Item on NES was approved, including the following objective on SSB-less operation [2]:

- Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2]

In RAN4 #106bis, SSB-less SCell operation was discussed, and the following agreements were achieved:

- Agreements - The following scenarios are considered for inter-band SSB-less SCell requirements definition - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell. - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis - Agreements - Conditions for SSB-less inter-band CA SCell operation for scenario-1, scenario-2 and scenario-2a - Received time difference (RTD) between the SSB-less SCell and the FR1 inter-band active serving cell - Option 1-1: 260ns - Option 1-2: 3us - Option 1-3: smaller than 3us - Other values are not precluded - The difference of the reception power with the FR1 inter-band active serving cell - Option 2-1: 6dB - Option 2-2: 6dB+offset - Other values are not precluded - FFS: TCI state (using TCI state information of inter-band co-located SSB less SCell from active serving cell) - FFS: frequency domain separation between the SSB-less SCC and the FR1 inter-band active serving CC; or Certain band combinations. - FFS if requirements will be defined subject to feasibility analysis - Note: different conditions and values may be applicable scenario-1, scenario-2 and scenario-2a

In this paper, we provide our considerations based on RAN4 progress.

Discussion

The following scenarios are identified by RAN4 for inter-band SSB-less SCell requirements definition:

    - Scenario 1: No SSB but with TRS transmission configured on the SSB-less SCell

      - FFS whether to consider requirements based on TRS transmission during and/or after SCell activation

    - Scenario 2: No SSB and no TRS transmission configured on the SSB-less SCell

    - Scenario 2a: No DL transmission but with UL reception at the NW side on the SSB-less SCell.

    - FFS if requirements will be defined for all or a subset of scenarios subject to feasibility analysis

From RAN2 perspective, Temporary RS (TRS) for efficient SCell activation was introduced in R17 LTE_NR_DC_enh2-Core.

The corresponding TRS is configured by RRC signalling and triggered by Enhanced SCell Activation/Deactivation MAC CE:

---- Cited from 38.331 v17.4.0: CSI-MeasConfig ::= SEQUENCE { [omitted] [[ sCellActivationRS-ConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17 OPTIONAL, -- Need N sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL -- Need N ]] } SCellActivationRS-Config-r17 ::= SEQUENCE { scellActivationRS-Id-r17 SCellActivationRS-ConfigId-r17, resourceSet-r17 NZP-CSI-RS-ResourceSetId, gapBetweenBursts-r17 INTEGER (2..31) OPTIONAL, -- Need R qcl-Info-r17 TCI-StateId, ... } ---- Cited from 38.321 v17.4.0: The MAC entity shall for each configured SCell: 1> if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell: 2> if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is indicated for this SCell: 3> indicate to lower layers the information regarding the TRS. 2> if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or 2> if the SCell is configured with sCellState set to activated upon SCell configuration: [omitted]

Regarding the Scenario 1 mentioned by RAN4 (i.e. No SSB but with TRS transmission configured on the SSB-less SCell), it can already be supported by RAN2 framework with minimum spec changes, and avoiding SSB transmission can lead to obvious network energy saving gains. If Scenario 1 is supported, TRS is configured by RRC signalling as in R17, and whether TRS is used during SCell activation and/or after SCell activation is pending on RAN4 further discussion on the requirements.

Proposal 1: Scenario 1 (i.e. No SSB but with TRS transmission configured on the SSB-less SCell) is feasible from RAN2 perspective with minimum spec change.

Regarding the Scenario 2 mentioned by RAN4 (i.e. No SSB and no TRS transmission configured on the SSB-less SCell), we are open on this issue but we think this is not in RAN2 scope. TRS can be useful in e.g. time tracking after SCell activation to guarantee PDCCH/PDSCH performance, and it is still in RAN4 discussion whether inter-band SSB-less SCell is feasible without TRS.

Regarding the Scenario 2a mentioned by RAN4 (No DL transmission but with UL reception at the NW side on the SSB-less SCell), this is a new use case to RAN2. The motivation of this scenario is to further enlarge network energy saving by shutting down the DL transmitter of the NW completely. Under such use case, from NW perspective, there is no PDCCH/PDSCH but there can be UL reception in the SSB-less SCell. From our point of view, Scenario 2a has no further RAN2 impacts, since the UE still relies on PCell for RLM and RRM, and cross-carrier scheduling can be used. However, Scenario 2a requires all the UEs have no DL reception requirements, which is a corner case, and can be considered as an extreme example of Cell DTX where the SCell is always in the non-active duration, so the discussion is either initiated in Cell DTX/DRX session or not considered by RAN2.

Conclusion

In this paper, we discussed the SSB-less SCell techniques, and made the following proposal:

Proposal 1: Scenario 1 (i.e. No SSB but with TRS transmission configured on the SSB-less SCell) is feasible from RAN2 perspective with minimum spec change.

References

- RP-222821, 3GPP TR 38.864 V1.0.0, Study on network energy savings for NR.

- RP-223540, New WID: Network energy savings for NR

- R4-2306260, Topic summary for [106-bis-e][229] Netw_Energy_NR, Huawei

- TS 38.331 v17.4.0

- TS 38.321 v17.4.0