3GPP TSG-RAN WG2 Meeting #122                                        R2-2306078

Inchon, Korea, May 22th – May 26th, 2023

Source:         ZTE Corporation

Title:            Discussion on sidelink positioning

Agenda item:    7.2.2

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID[1] of sidelink positioning has been achieved as follows:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

RAN2 is mainly responsible for the protocol design between UEs, or between UE and LMF. Based on above, this contribution provides discussion and proposals for SLPP protocol and procedure design.

- Discussion

  - Anchor UE (re)selection

In RAN2#121[2], the following agreement regarding to the SL positioning procedure is agreed as follows:

Agreement: With respect to the overall signaling procedure for PC5-only positioning (including at least IC and OOC; FFS if there are differences for PC), it is proposed to agree that the sidelink positioning procedure comprises the following series of steps as a baseline, between the LMF/positioning server UE/NG-RAN/candidate Anchor UE(s) and Target UE(s): 1.Triggering event 2. Sidelink positioning capability exchange 3. Sidelink positioning assistance data transfer 4. SL Positioning Request Location Information 5. Measurement of SL-PRS 6. Location calculation 7. SL Positioning Provide Location Information Some steps may have dependencies on SA2 and can be revisited in this light. The order is subject to further discussion. FFS if discovery and selection of anchor UEs and/or server UE are part of the positioning layer in RAN2 scope. LS to SA2 to ask for confirmation and guidance on the SA2 aspects.

The procedure basically follows the LPP signaling from RAN2 perspective depicting a UE-to-UE flow. For the FFS part on the discovery/anchor UE (re)selection, note that there is relay UE discovery/(re)selection procedure already specified in TS 24.554[3] and TS 38.331[4], describing relay (re)selection in AS layer and relay discovery in NAS layer, respectively. We think the anchor UE discovery and (re)selection can take the existing procedure as baseline for further discussion.

- Discovery filtering is from NAS layer perspective, it is to compare whether the the service level requirement for discovery is satisfied or not(i.e., the NAS layer criteria) as specified in TS24.554, and the AS layer criteria should be determined by RAN2. Regarding to the issue how the anchor UE is considered good enough to be (re)selected (the AS layer criteria of anchor UE (re)selection), at least the AS layer criteria of relay UE (re)selection is suitable to be reused, since SL positioning also needs good channel condition at PC5 interface. To be specific, target UE determines whether the UE can be an anchor UE is, whether anchor UE’s transmission RSRP is larger than a threshold. The transmission RSRP can be SL signaling RSRP or SL PRS RSRP, depending on the case of first time selection or reselection.

- In addition, the anchor UE’s attribute should be taken into consideration as another kind of AS layer criteria when determining whether the anchor UE should be chosen as a final anchor UE. For example, target UE would like to have interactions with the anchor UE that can transmit SL-PRS with the SCS of 30kHz, etc.

- Proposal 1: Support to take relay UE (re)selection criteria as the baseline of anchor UE (re)selection criteria, i.e., the criteria that target UE can determine the UE can be an anchor UE is, whether anchor UE’s transmission RSRP is larger than a threshold. FFS on which RSRP, SL signaling RSRP or SL PRS RSRP.

- Proposal 2: Support to take anchor UE’s capability into consideration when target UE performs anchor UE (re)selection. I.e., the capability transfer can be broadcasted or groupcasted to assist anchor UE (re)selection before a SL positioning session is set up.

According to SA2’s specification TS23.586[5], both mode A and mode B discovery are supported. In RAN2#121-bis[6] meeting, the following agreement is achieved:

Agreements: RAN2 confirms that discovery messages will be used to carry information for targeted discovery and candidate selection of SL positioning UEs, including at least the indication of anchor UE, target UE. and server UE roles. FFS how much information is indicated about anchor UEs (e.g., knowledge of location). The UE role information is indicated in the discovery SLPP metafield. FFS if this applies to both discovery modes and which messages.

Mode A discovery is for anchor UEs to broadcast ‘I can be discovered’ to the neighbor UEs. So it is natural to contain UE role in the Mode A discovery announce message. Mode B discovery is for one UE to request other UE’s discovery response, so the request message should contain UE roles as ‘I am a target UE, and I would like to find the anchor UE with some positioning requirements’, and if anchor UE is interested at the service, it can make the discovery message response, but it is not necessary to contain UE role in the discovery response message, since the response message itself already denotes the UE’s role as anchor UE.

Here gives example procedures for mode A and mode B discovery and anchor UE (re)selection in figure 1 and figure 2, respectively:

Figure 1. Procedure of mode A anchor UE discovery

- Step 1: Anchor UE(s) broadcast discovery announce message, which contains its UE role as anchor UE.

- Step 2: target UE sends request capability message via SLPP broadcast/groupcast.

- Step 3: anchor UEs send their SL positioning capability to the target UE via broadcast/groupcast/unicast.

- Step 4: target UE determines final anchor UEs which satisfies both AS layer criteria(including anchor UE’s capability) and NAS layer criteria.

- Step 5: target UE may send the acknowledgement to the selected anchor UEs in step 4, e.g., to set up SL positioning session.

- Step 6: Target UE and selected anchor UEs begin to perfrom SL positioning (further capability if any/assistance data/SL-PRS transfer/location information transfer).

Figure 2. Procedure of mode B anchor UE discovery

- Step 1: Target UE sends the discovery request message which contain its UE role as target UE.

- Step 2: anchor UE monitors the discovery request message to see if it satisfies the NAS layer criteria of the discovery request.

- Step 3: Target UE receives the discovery response of several candidate anchor UEs which satisfy the NAS layer criteria.

- Step 4: target UE sends request capability message via SLPP broadcast/groupcast.

- Step 5: anchor UEs send their SL positioning capability to the target UE via broadcast/groupcast/unicast.

- Step 6: Target UE selects final anchor UEs based on the AS layer criteria(including anchor UE capability) from the several candidate anchor UEs.

- Step 7: target UE may send the acknowledgement to the selected anchor UEs in step 6, e.g., to set up SL positioning session.

- Step 8: Target UE and selected anchor UEs begins to perfom SL positioning (capability if any/assistance data/SL-PRS transfer).

- Proposal 3: Support target UE to discover candidate anchor UEs according to NAS layer criteria, then select final anchor UEs according to AS layer criteria.

- Proposal 4: Support UE roles containing in mode A discovery announce message and mode B discovery request message. Take figure 1 and figure 2 for mode A and mode B discovery procedure as baseline.

  - Stage-2 procedure

There are 3 kinds of scenarios that will impact stage 2 procedure:

Scenario 1: target UE is in coverage while anchor UEs are in coverage or out of coverage; (legacy IC/PC scenario)

Scenario 2: target UE is out of coverage while one of the anchor UE is in coverage; (legacy PC scenario)

Scenario 3: target UE is out of coverage while all the anchor UEs are out of coverage.(legacy OOC scenario)

Scenario 1 and scenario 2 can be used for the procedure with LMF involvement, and scenario 3 can be used for the procedure without LMF involvement. From specification writing perspective, we think at least the above 3 procedure figures should be captured in the stage-2 specification.

Proposal 5: Capture at least the following 3 procedure figures in the stage-2 specification:

- Procedure 1: with LMF’s involvement, target UE is in coverage while anchor UEs in in coverage or out of coverage; (legacy IC/PC scenario)

- Procedure 2: with LMF’s involvement, target UE is out of coverage while one of the anchor UE is in coverage; (legacy PC scenario)

- Procedure 3:without LMF’s involvement, target UE is out of coverage while all the anchor UEs are out of coverage.(legacy OOC scenario)

With LMF involvement

For the procedures that with LMF’s involvement, SA2 TS23.586 has already agreed following definitions:

- TS23.586: - 5.5.2 Network assisted SL Positioning of UE with NAS connection A UE with a NAS signalling connection is in CM-Connected state. The UE enters CM-Connected state by performing UE triggered Service Request for 5GC-MO-LR or performing Network triggered Service Request for 5GC-NI-LR or 5GC-MT-LR. (omitted) - 5.5.3 Network assisted SL positioning without NAS connection When Target UE cannot establish the NAS connection with AMF due to the Target UE being out of coverage, for 5GC-MO-LR or pending 5GC-MT-LR (e.g. deferred 5GC-MT-LR), the following principles are applied: (omitted)

This contains 2 cases: target UE is in coverage(TS23.586, section 5.5.2), or target UE is out of coverage while one of the anchor UE is in coverage(TS23.586, section 5.5.3). If network-involved SL positioning is enabled in partial coverage scenario, there can be two procedure flows for target UE in coverage, and target UE out of coverage while one of the anchor UE in coverage. The examples are given as follow:

Figure 3. target UE in coverage when network assisted SL positioning(with LMF involvement) is performed

Step 1. LMF receives the location service request of target UE for MT-LR, MO-LR or NI-LR.

Step 2. LMF decides the target UE to perform SL positioning and send SL positioning trigger message via LPP RequestLocationInformation message.

Step 3. Interaction between LMF and NG-RAN nodes to negotiate SL-PRS configuration if necessary. Note that there is a case that there are more than one UEs (adopting scheme 1 SL-PRS transmission) but with different gNB’s coverage. The SL-PRS configuration given by gNB should be non-overlapped to avoid interference.

Step 4. Serving gNB of the target UE provides SL-PRS configuration for scheme 1 target UE via RRC signaling.

Step 5. target UE triggers discovery and anchor UE selection procedure according to section 2.1 above.

Step 6. at this step SLPP session is established. Anchor UEs may send SL-ProvideAssistanceData SLPP message to target UE including anchor UE’s location and/or anchor UE’s SL-PRS configuration; target UE may send the SL-PRS configuration to the anchor UEs.

Step 7. anchor UEs and target UE transmit SL-PRS to each other.

Step 8. anchor UEs and target UE measure SL-PRS and make SL positioning measurements(e.g., SL-RSTD) respectively.

Step 9. anchor UEs provide their SL positioning measurements to the target UE via SL-ProvideLocationInformation SLPP message.

Step 10. target UE includes all the SL positioning measurements in a container and transmit to LMF via LPP message.

Step 11. LMF takes all the UE’s SL positioning measurements (or with uu positioning measurements) and calculate target UE’s location.

Step 12. LMF returns the location estimate to the service request source.

Figure 4. target UE out of coverage while one of the anchor UE in coverage when network assisted SL positioning(with LMF involvement) is performed

Step 1. Target UE has a MO-LR service request for acquiring its own location.

Step 2. target UE may acquire SL-PRS configuration via pre-configuration.

Step 3. target UE triggers discovery and anchor UE selection procedure according to section 2.1 above.

Step 4: anchor UE that has NAS connection can trigger a MO-LR request to the AMF to get its own location or to transmit/receive target UE’s location in the following steps.

Step 5: AMF forwards the anchor UE’s MO-LR request to LMF.

Step 6. at this step SLPP session is established. Anchor UEs may send SL-ProvideAssistanceData SLPP message to target UE including anchor UE’s location and/or anchor UE’s SL-PRS configuration; target UE may send the SL-PRS configuration to the anchor UEs.

Step 7. anchor UEs and target UE transmit SL-PRS to each other.

Step 8. anchor UEs and target UE measure SL-PRS and make SL positioning measurements(e.g., SL-RSTD) respectively.

Step 9. target UE provides the SL positioning measurements to the anchor UE in step 4 via SL-ProvideLocationInformation SLPP message.

Step 10. anchor UE includes all the SL positioning measurements (of target UE and/or itself) in a container and transmit to LMF via LPP message.

Step 11. LMF takes all the UE’s SL positioning measurements (or with uu positioning measurements) and calculate target UE’s location.

Step 12. LMF returns the location estimate of the target UE to the AMF.

Step 13: AMF sends MO-LR response containing target UE’s location to the anchor UE.

Step 14. anchor UE forwards the target UE’s location to the target UE via SL-ProvideLocationInformation SLPP message.

Proposal 6: Support to take the figure 3: target UE in coverage when network assisted SL positioning(with LMF involvement) is performed, and figure 4:  target UE out of coverage while one of the anchor UE in coverage when network assisted SL positioning(with LMF involvement) is performed, for the LMF-involved procedure as stage-2 baseline.

Without LMF involvement

For the SL positioning procedure without LMF involvement, the basic procedure flow is given as follows:

Figure 5. SL positioning procedure that target UE and all anchor UEs are out of coverage without LMF’s involvement

Step 1. Target UE has a MO-LR service request for acquiring its own location.

Step 2. target UE may acquire SL-PRS configuration via pre-configuration.

Step 3. target UE triggers discovery and anchor UE selection procedure according to section 2.1 above.

Step 4. at this step SLPP session is established. Anchor UEs may send SL-ProvideAssistanceData SLPP message to target UE including anchor UE’s location and/or anchor UE’s SL-PRS configuration; target UE may send the SL-PRS configuration to the anchor UEs.

Step 5. anchor UEs and target UE transmit SL-PRS to each other.

Step 6. anchor UEs and target UE measure SL-PRS and make SL positioning measurements(e.g., SL-RSTD) respectively.

Step 7. anchor UEs provide the SL positioning measurements to the target UE via SL-ProvideLocationInformation SLPP message.

Step 8. target UE performs lcoation calculation based on the gathered SL positioning measurements.

Proposal 7: Support to take the figure 5 for the SL positioning procedure out of coverage without LMF’s involvement as stage-2 baseline.

Server UE in the procedure

In RAN2#121-bis-e meeting, the following agreement is achieved:

Agreement: Anchor UE and target UE roles can be shown in the sidelink positioning procedures in stage 2. Server UE can be further discussed at least for the case that the server UE is separate from the target and anchor.

Companies raised concern on whether to have an separate server UE and whether the separate server UE should be shown in the stage-2 procedure figure. So far the target UE and anchor UE have clear definition that target UE is the UE needs to perform positioning, and anchor UEs are the UEs interacting SL-PRS or SLPP signaling with the target UE to assist positioning. However server UE is only agreed with several functions, such as method determination, anchor UE selection and location calculation.

In SA2’s definition, the server UE is ‘a UE offering location calculation for Sidelink Positioning and Ranging based service. It interacts with other UEs over PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported.’ That is to say that, the UE that satisfies one of the server UE functions (location calculation) can be called a server UE. Then the other two server UE functions (method determination and anchor UE selection) are not strictly performed by server UE if target UE or anchor UE has the corresponding capability. The stage-2 procedure flow should cover as much cases as possible, so it will be too detailed to include the server UE(which may even have only one function) in the figure. Instead, the description of server UE can be included in the step description or a note saying that if target UE or anchor UE do not have such capability, server UE should perform the function.

Proposal 8: Not to introduce server UE in the stage-2 procedure figure. If the procedure involving server UE is necessary, it can be described in the step description or note.

  - SLPP session containing multiple target UEs

In RAN2#121, SA2 has sent an LS R2-2301928[7] to RAN2 asking the support of multiple target UEs containing in one SL positioning session:

1. Overall Description: SA2 would like to inform RAN2 that during the development of the Ranging/SL Positioning support (as in TS 23.586), use cases had been identified where multiple Target UEs are involved in the same positioning session. In these cases, it is desirable to allow the Ranging/SL Positioning control to support multiple target UEs, e.g. with broadcast or groupcast support. SA2 understand that RAN2 is currently defining the Ranging/SL Positioning Protocol (the SLPP in RAN2’s context). Therefore, SA2 would like to ask RAN2 whether the SLPP would support multiple Target UEs in the same signalling session, and whether there is possibility of signalling the positioning results of multiple Target UEs in the same message. 2. Actions: To RAN2 WG: ACTION: SA2 would like to ask RAN2 to provide feedback on the support of multiple Target UEs in the same Ranging/SL Positioning session.

- And in RAN2#121-bis-e, RAN2 has agreed the following working assumption:

- Agreements: - LS on group positioning is postponed for a reply from next meeting. - WA: RAN2 understand that group positioning is to acquire location estimates of multiple target UEs (absolute positioning) or multiple UE pairs (Ranging/relative positioning) per LCS request, in line with the guidance already received from SA2. - WA: At least part of the group management for group positioning is performed at upper/application layer.

- In the uu interface, a LPP positioning session contains only one target UE for absolute positioning. Each LPP positioning session is associated with a service request. If multiple target UEs are to be contained in a same positioning session, there should be requirements on the multiple target UE’s measurements, for example, requiring multiple UEs to measure simultaneously, or the multiple UEs should perform positioning with the same positioning purpose (for example, ranging, two UEs are associated to be positioned). one way to achieve this is that: the service trigger should include multiple target UE’s service request at the very first beginning. For example, for SL-MO-LR service request, one SL-MO-LR service request generated by one target UE can contain other target UEs’ location request. For SL-MT-LR service request, one SL-MT-LR service request from network to a target UE can contain other target UE’s’ location request. For example in figure 6, there is a MT-LR request with multiple target UEs’ location request, and one LPP session is associated with a SLPP session that contains target UE1, target UE2 and target UE3.

- Figure 6. Example of MT-LR request with multiple target UEs in a SLPP session

- Proposal 9: For multiple target UEs in a same positioning session, support the service trigger(MO-LR, MT-LR, etc.) to include multiple target UE’s service request.

- Regarding the question ‘whether there is possibility of signalling the positioning results of multiple Target UEs in the same message’, we think it is also possible. This is to allow one SL-ProvideLocationInformation SLPP message to contain multiple UE’s SL-PRS measurements/location estimate, each with the corresponding UE identity. This is useful when forwarding measurements between UEs in a positioning session, or when forwarding measurements to the LMF via LPP container.

- Proposal 10: Support one SL-ProvideLocationInformation SLPP message to include multiple UE’s location measurements/estimates, each with corresponding UE identity.

  - SLPP specification design

In RAN2#121-bis-e meeting, SLPP specification design has been discussed. In which, all the need code(need M, need S, need N, need R) and toaddmodlist/toreleaselist are called delta signaling and they are not sure whether to be introduced in the SLPP specification. Delta signaling is useful for forward compatibility. Note that SLPP signaling can be broadcasted/groupcasted/unicasted, they are different from PC5-RRC message which can only be unicasted. For example, need M and toaddmodlist/toreleaselist can not be applied to broadcast or groupcast since UE may not able to receive the corresponding field before. To decide the delta signaling use, RAN2 should first decide whether to separate broadcast/groupcast ASN.1 signaling and unicast ASN.1 signaling as current RRC (SIBx and dedicate signaling) in SLPP specification, or to have joint ASN.1 signaling that can be applied to all broadcast/groupcast/unicast. If they are present separately, unicast dedicate signaling can have all the need code and toaddmodlist/toreleaselist, broadcast/groupcast signaling can only have need S, need R and need N; if they are present jointly, all the need code can be applied, need M should be clarified that only applied to unicast mode. Further, toaddmodlist/toreleaselist should not be introduced in the joint signaling.

To reduce the work of specification design, we slightly prefer to have joint signaling since there are almost same IEs of unicast and broadcast/groupcast.

Observation 1: Delta signaling (including all the need code and toaddmodlist/toreleaselist) is useful for forward compatibility.

Proposal 11: For SLPP specification design, support joint ASN.1 signaling of broadcast/groupcast and unicast, all the need code(need M, need R, need S, need N) can be applied, need M should be clarified that only applied to unicast mode. Further, toaddmodlist/toreleaselist should not be introduced in the joint signaling.

  - Protocol between UE and LMF

RAN2#119-b[8] has achieved the following agreement on the signaling support of hybrid or PC5-only positioning:

- Agreement: - Protocol options between UE and LMF for hybrid PC5+Uu positioning and PC5-only positioning in-coverage are studied and RAN2 will down-select during normative work. - Extension of LPP, whereby new signaling shall be defined to support hybrid Uu and PC5 based positioning, i.e. extend the existing LPP to support sidelink based positioning between UE and LMF - Enhancement of LPP whereby SLPP/RSPP signaling can be transported within LPP transparently, i.e. use the newly defined SLPP/RSPP to support sidelink based positioning and use the existing LPP to support Uu based positioning; and the SLPP/RSPP is carried as a container in LPP - Use of SLPP/RSPP between the UE and the LMF

Based on the rough classification, 1st and 2nd bullet are SLPP+LPP signaling between UE and LMF, and 3rd bullet is SLPP between UE and LMF. The proponents of 3rd bullet argue that there is a case that a UE can not support LPP but want to communicate with LMF for calculating/scheduling. From our point of view, LPP should be the basic protocol for a positioning UE. In addition, a unified design of PC5-only UE and Uu+PC5 UE is desired in order to reuse the legacy approach (i.e., LPP) as much as possible. We should avoid to design a new end-to-end protocol only for a special kind of UE (i.e., only support PC5 positioning, do not support Uu positioning) . Moreover, the PC5-only UE does not compulsively need the 3rd bullet to achieve the sidelink positioning. For example, the UEs that only support PC5 positioning can adopt at least two solutions: 1) these UEs adopt UE based calculating without network’s involvement; 2) if they want to perform LMF based positioning, these UEs can find a UE (maybe the SL positioning server UE or others) that supports LPP, then send RSPP/SLPP measurement to the LPP UE, then forward to the LMF.

When communicating with LMF, the difference between 1st bullet and 2nd bullet is that, 1st bullet is to set SLPP message as explicit signaling in LPP, i.e., add new IEs with concrete content in the existing LPP protocol; 2nd bullet is to set SLPP message as container in LPP, in which the RSPP/SLPP message only follows the LPP PDU transmission rules, but what content to be embedded in the container should follow SLPP module. From our point of view, either 1st bullet and 2nd bullet can be supported, and 2nd bullet is slightly preferred.

Observation 2: A unified design of PC5-only UE and Uu+PC5 UE is desired in order to reuse the legacy approach (i.e., LPP) as much as possible.

Proposal 12: Support to use the RSPP/SLPP+LPP between UE and the LMF, i.e., SLPP is designed as a container in the LPP.

LPP’s structure is depicted as below:

Figure 7.legacy LPP Message structure

Based on above, two structures of the SLPP container can be designed, i.e., add several SLPP containers per LPP message type (see figure 8), or add one SLPP container sharing the same sequence number, transaction ID within one LPP Message (see figure 9). Both structures are session-based approach, i.e., the SLPP container follows the establish/release/abort procedure of the attached LPP session. If the latter is applied, each LPP transaction performs a single operation as SLPP message. What to contain in one SLPP message is based on LMF’s implementation, but the scope of LMF’s decision is within the SLPP module and IEs. We think these two structures can be further considered and down-selected by RAN2.

Figure 8. Several SLPP containers per LPP message type

Figure 9. One SLPP container per LPP Message

Further, the detailed SLPP message of capability transfer, assistance data transfer and location measurement information transfer should be discussed by RAN2, considering the difference between the SLPP message in the SLPP container between UE&LMF and SLPP message between UEs, if exists.

Proposal 13: RAN2 to discuss and down-select the two structures of the SLPP container in LPP: add several SLPP containers per LPP message type, or add one SLPP container within one LPP Message.

  - Joint SL and Uu positioning methods and measurements

According to RAN2#119[9] in SI phase, both SL-only-based positioning and joint SL & Uu positioning are studied and recommended for normative work:

- Agreement - Proposal 2: Study the architecture and signaling procedures to enable at least the following two operation scenarios: - - Operation Scenario 1: PC5-only-based positioning. - - Operation Scenario 2: Combination of Uu- and PC5-based positioning.

DL-TDOA-like method is a typical technique for joint positioning where both DL-PRS from multiple TRPs and SL-PRS from multiple anchor UEs are transmitted to target UE. Take Figure 10 as an example, target UE’s position cannot be calculated by either Uu positioning or SL positioning since DL-TDOA-like positioning requires at least 3 anchor nodes wherein one of them is a reference node. In such case, we have to merge both Uu measurements (i.e. measured between target UE and TRP 1/2) and SL measurements (i.e. measured between target UE and anchor UE 1/2).

Figure 10. DL-TDOA-like joint SL & Uu positioning

However, usually anchor UEs and TRPs are not accurately synchronized and reference nodes for SL and Uu positioning are selected separately. For Uu positioning, a reference TRP (e.g. TRP 1) is selected as reference node, Uu RSTD measurement is obtained by calculating relative timing difference between the anchor TRP and a reference TRP. For SL positioning, a reference UE (e.g. anchor UE 1) is selected as reference node, SL measurement RSTD is obtained by calculating relative timing difference between the anchor UE and the SL reference node. For UE-assisted positioning, DL measurement and SL measurement can be reported to LMF jointly. In order for the LMF to know the time difference or time synchronization error between Uu session and SL session, a special RSTD measurement can be introduced and reported from target UE to LMF. This special RSTD is used to specify the measurement of the relative timing difference between SL reference node and Uu reference node.

Proposal 14: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: Delta signaling (including all the need code and toaddmodlist/toreleaselist) is useful for forward compatibility.

Observation 2: A unified design of PC5-only UE and Uu+PC5 UE is desired in order to reuse the legacy approach (i.e., LPP) as much as possible.

- Proposal 1: Support to take relay UE (re)selection criteria as the baseline of anchor UE (re)selection criteria, i.e., the criteria that target UE can determine the UE can be an anchor UE is, whether anchor UE’s transmission RSRP is larger than a threshold. FFS on which RSRP, SL signaling RSRP or SL PRS RSRP.

- Proposal 2: Support to take anchor UE’s capability into consideration when target UE performs anchor UE (re)selection. I.e., the capability transfer can be broadcasted or groupcasted to assist anchor UE (re)selection before a SL positioning session is set up.

- Proposal 3: Support target UE to discover candidate anchor UEs according to NAS layer criteria, then select final anchor UEs according to AS layer criteria.

- Proposal 4: Support UE roles containing in mode A discovery announce message and mode B discovery request message. Take figure 1 and figure 2 for mode A and mode B discovery procedure as baseline.

Proposal 5: Capture at least the following 3 procedure figures in the stage-2 specification:

- Procedure 1: with LMF’s involvement, target UE is in coverage while anchor UEs in in coverage or out of coverage; (legacy IC/PC scenario)

- Procedure 2: with LMF’s involvement, target UE is out of coverage while one of the anchor UE is in coverage; (legacy PC scenario)

- Procedure 3:without LMF’s involvement, target UE is out of coverage while all the anchor UEs are out of coverage.(legacy OOC scenario)

Proposal 6: Support to take the figure 3: target UE in coverage when network assisted SL positioning(with LMF involvement) is performed, and figure 4:  target UE out of coverage while one of the anchor UE in coverage when network assisted SL positioning(with LMF involvement) is performed, for the LMF-involved procedure as stage-2 baseline..

Proposal 7: Support to take the figure 5 for the SL positioning procedure out of coverage without LMF’s involvement as stage-2 baseline.

Proposal 8: Not to introduce server UE in the stage-2 procedure figure. If the procedure involving server UE is necessary, it can be described in the step description or note.

- Proposal 9: For multiple target UEs in a same positioning session, support the service trigger(MO-LR, MT-LR, etc.) to include multiple target UE’s service request.

- Proposal 10: Support one SL-ProvideLocationInformation SLPP message to include multiple UE’s location measurements/estimates, each with corresponding UE identity.

Proposal 11: For SLPP specification design, support joint ASN.1 signaling of broadcast/groupcast and unicast, all the need code(need M, need R, need S, need N) can be applied, need M should be clarified that only applied to unicast mode. Further, toaddmodlist/toreleaselist should not be introduced in the joint signaling.

Proposal 12: Support to use the RSPP/SLPP+LPP between UE and the LMF, i.e., SLPP is designed as a container in the LPP.

Proposal 13: RAN2 to discuss and down-select the two structures of the SLPP container in LPP: add several SLPP containers per LPP message type, or add one SLPP container within one LPP Message.

Proposal 14: Support DL-TDOA-like joint SL & Uu positioning:

- For a target UE, Support reporting a RSTD for the timing difference between SL reference node and Uu reference node.

- References

- RP-223549, New WID on Expanded and Improved NR Positioning, 2022-12.

- Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece, 2023-03.

- TS 24.554, Proximity-services (ProSe) in 5G System (5GS) protocol aspects;Stage 3, 2023-01.

- TS 38.331, Radio Resource Control (RRC) protocol specification, 2023-03.

- TS23.586, Architectural Enhancements to support Ranging based services and Sidelink Positioning, 2023-02.

- Report of 3GPP TSG RAN WG2 meeting #121-bis-e, online, 2023-04.

- R2-2301938, LS on support of multiple Target UEs, SA2, 2023-03.

- Report of 3GPP TSG RAN WG2 meeting #119-bis-e, Online, 2022-10.

- Report of 3GPP TSG RAN WG2 meeting #119, Online, 2022-08.