3GPP TSG-RAN RAN2 #122						R2-2306093

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:		7.16.2.2

Source:			Huawei, HiSilicon

Title:			Discussion on data collection

Document for:		Discussion and Decision

1   Introduction

At RAN2#121b-e meeting, the following agreements on data collection were made:

Extend the previously endorsed table with 3 columns: Inference, Monitoring and Training, and explain in free text the applicability of the data collection method to the LCM purpose and the use case(s).

P1: RAN2 to understand/determine/capture requirements of data collection for the LCM functionalities and document the results. FFS on the exact presentation format. Expect RAN1 to provide some related information.

P2: RAN2 to capture the analysis (see P1 above) separately for the use-cases, i.e., CSI feedback enhancement, beam management and positioning enhancement.  FFS how we do the formatting/presentation of the results.

P3: Study the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes, i.e., inference, monitoring and (offline) training. FFS how we do the formatting/presentation of the results.

P4: With more progress on architectural discussion, consider the suitability of each identified data collection framework for the termination points and mapping with the location of LCM purposes/functions (inference, monitoring, (offline) training)

- - Model sidedness (UE side, NW side, two sided) FFS

- - Use case mapping FFS

P5: RAN2 to modify the previously endorsed table by adding 3 additional columns: inference; monitoring and (offline) training. Whether to, and how to further restructure the table is FFS.

In this paper, we mainly discuss the following aspects:

- Address the FFSes from the last RAN2 meeting

- Discuss EVEX

2   Discussion

2.1 Discussion on data collection requirements

Based on the latest RAN1 progress, we try to summarize the requirements in the following tables.

Table 1: Data collection requirements for CSI related use cases

|  |  | CSI related use cases |  |
| --- | --- | --- | --- |
|  |  | CSI compression (two-sided) | CSI prediction (UE-sided) |
| Training input: data output: model | Input data | Raw channel measurements Eigenvectors of the current CSI (there may be some new measurements) | Wait for RAN#100 decision on the use case |
| Inference input: data output: data | Input data | Encoder: input data is similar to training Decoder: input data is compressed bit stream |  |
|  | Output data | Encoder: output data is compressed bit stream Decoder: the expected output data is the input data for the Encoder (there may be some differences) |  |
| Monitoring input: data output: actions | Input data | Ground-truth measurements (may be similar to training) RAN1 is still discussing KPIs |  |

Table 2: Data collection requirements for BM related use cases

|  |  | BM related use cases |  |
| --- | --- | --- | --- |
|  |  | UE-sided | NW-sided |
| Training input: data output: model | Input data | L1-RSRP | Same as UE-sided |
| Inference input: data output: data | Input data | L1-RSRP | Same as UE-sided |
|  | Output data | top-k beam info, e.g. top-k beam IDs/RSRPs (UE may need to report it to gNB) | top-k beam info, e.g. top-k beam IDs/RSRPs |
| Monitoring input: data output: actions | Input data | RAN1 is still discussing alternatives, e.g. Beam prediction accuracy related KPIs, Link quality related KPIs. | Same as UE-sided |

Table 3: Data collection requirements for Positioning related use cases

|  |  | Positioning related use cases |  |  |
| --- | --- | --- | --- | --- |
|  |  | UE-sided | gNB-sided | LMF-sided |
| Training input: data output: model | Input data | PRS measurement Labels Others：potential new measurements: CIR, CFR, PDP etc. | SRS measurement Labels：PRUs/UEs may report the ground truth LOS/NLOS state or TOA to the gNB | PRS measurement: from PRU/UE to LMF, case 2b SRS measurement: from gNB to LMF, case 3b, corresponding labels (LMF uses the known PRU location) |
| Inference input: data output: data | Input data | PRS measurement | SRS measurement | PRS measurement for case 2b; SRS measurement for case 3b; |
|  | Output data | UE's location coordinates for direct positioning; intermediate metrics (e.g. LOS/NLOS states, TOA etc.) for assisted positioning; | intermediate metrics (e.g. LOS/NLOS states, TOA etc.) for assisted positioning | UE's location coordinates for direct positioning for both cases 2b and 3b |
| Monitoring input: data output: actions | Input data | Inference output and the corresponding ground-truth labels | Inference output and the corresponding ground-truth labels | Inference output and the corresponding ground-truth labels |

For Table 1, 2, 3, we understand that RAN1 is still discussing details and RAN1 will finalize the analysis by Aug RAN1 meeting (the last meeting for this SI).

Proposal 1: RAN2 to consider using Table 1/2/3 as starting point of data collection requirement per use case per LCM, and follow RAN1 progress.

For the above tables, it is also important to understand the following requirements:

- For each measurement, which entity generates it (e.g. UE, gNB)

- For each measurement, if node A generates it, whether node A needs to send the measurement to node B. If needed, the data collection requirement is valid. For example, if a measurement is generated at UE side, and gNB needs the measurement due to some reasons, then the UE needs to report the measuremnt to gNB, which is a valid data collection requirement and thus should be discused in the standard

From our point of view, the above requirements seem to be discussed and confirmed by RAN1. If so, RAN2 can follow and check possible RAN2 impacts.

Proposal 2: For the input/output data, RAN2 can follow the RAN1 discussion on the requirements, such as:

- For each measurement, which entity generates it

- For each measurement, whether it is needed to send it from node A to node B

For RAN2, we understand that different LCM purposes (e.g. training, inference, monitoring) may correspond to different data collection mechanisms. The following table shows a summary.

Table 4: Data collection analysis per LCM aspect

|  | Data collection for Model training | Data collection for Model inference | Data collection for Model monitoring |
| --- | --- | --- | --- |
| Latency for the reporting | Can be high latency | Low latency | Low latency |
| Need to be (temporarily) stored at the UE | May be needed | No need | No need |
| Data size | May be large | Small or medium | Small or medium |

Proposal 3: For data collection requirements, it is proposed RAN2 to also discuss the following aspects for model training, inference and monitoring:

- Latency for the reporting

- Need to be (temporarily) stored at the UE

- Data size

2.2 Discussion on the applicability (and limitations) of each identified data collection framework for each of the identified LCM purposes

The latest table for data collection mechanisms is listed as below.

|  | Involved Network entity | RRC state to generate data | Max payload size per reporting* | Contents to be collected | End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Logged MDT | TCE/OAM (It can be utilized by gNB) | RRC_IDLE/RRRC_INACTIVE | <9kbyte | L3 cell/beam measurements, location info, sensor info, timing info | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message, Privacy via user consent |
| Immediate MDT | TCE/OAM (It can be utilized by gNB) | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements, location info, sensor info | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Event triggered report, Periodic reporting | AS security via RRC message, Privacy via user consent |
| L3 measurements | gNB | RRC_CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | Event triggered report, Periodic reporting | AS security via RRC message. |
| L1 measurement (CSI reporting) | gNB | RRC_CONNECTED | <1706bit in PUCCH, <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic report and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | Aperiodic report, Semi-persistent report, Periodic report | No AS security |
| UAI | gNB | RRC_CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Early measurements | gNB | RRC_IDLE/RRC_INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| LPP | LMF | RRC_CONNECTED | <9kbyte | Location info | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive NW request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | UE-triggered, NW-triggered | AS security via RRC message |

Based on our analysis in section 2.1, we think immediate MDT is a possible candidate solution. The immediate MDT can cover the CSI/BM use cases.

For positioning use cases, RAN2 can discuss whether to use LPP or use immediate MDT for data collection purpose. In our understanding, if the data collection requirement is between UE and gNB, immediate MDT mechanism can be considered; and if the data collection requirement is between UE and LMF, LPP mechanism can be considered.

Observation 1: For L1 measurement (CSI reporting), if all the data to be collected can be supported by existing reporting scheme, L1 measurement (CSI reporting) can be reused.

Proposal 4: For CSI/BM use cases, immediate MDT can be considered to do the data collection. How to enhance immediate MDT can be further discussed.

Proposal 5: For Positioning use case, if data collection requirement is between UE and gNB, immediate MDT mechanism can be considered to do the data collection; and if data collection requirement is between UE and LMF, LPP mechanism can be considered to do the data collection. How to enhance immediate MDT/LPP can be further discussed.

2.3 Discussion on the data overhead and possible enhancements

The table in [3] has captured most of companies’ opinions, and thus it is a good starting point. For now, we have no idea much data will be collected for AIML purposes (e.g. for model training), so it is worth to consider any problems caused by large data.

Among all mechanisms, logged MDT is to collect data when the UE is in idle/inactive states, and then try to send the reports to the NW when the UE moves to connected state. According to the current TS 38.306, the UE supporting logged MDT shall support 64 KB for the logging. When the UE goes into connected state, the UE can include logMeasAvailable-r16 in some UL RRC messages, and then the NW can use UEInformation procedure to request the UE to send the logged MDT reports. If the original logged MDT report is large (e.g. 10 KB), it will need two UEInformation procedures (9KB+1KB) for transferring the whole report.

loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB.

Observation 2: For logged MDT, the UE is required to support 64KB for the buffer, and the logMeasAvailable-r16 and UEInformation procedure are used for transferring the whole report. There may be multiple procedures for the transferring.

For other data collection mechanisms, if the RRC state to generate data and to transfer data to the network is connected state, the maximum payload per reporting is 9KB. For now, it seems to be no problems as the collected data may not be large. In addition, there is a Note in TS 38.331, and the data collection mechanisms can not use the segmentation of UL RRC message.

NOTE:	The segmentation of UL DCCH message is only applicable to UECapabilityInformation and MeasurementReportAppLayer in this release.

Observation 3: The segmentation of UL RRC message is only applicable to UECapabilityInformation and MeasurementReportAppLayer, and it is not applicable to other UL RRC messages (related to data collection mechanisms).

With observation 2, for the data for AIML purpose, if the size is more than 9KB, the current mechanisms can not work, and it may need some enhancements. The following option can be considered:

- To apply the segmentation of UL RRC message here. For example, for MeasurementReport message for immediate MDT, if the segmentation of UL RRC message is applicable, the UE can transfer the data with more than 9KB.

Proposal 6: For data collection mechanisms (i.e. the UE generates and transfers data only in RRC connected state), if the data size is more than 9KB, the current mechanisms can not work, and some enhancements can be considered, e.g. the segmentation of UL RRC message.

2.4 Discussion on EVEX

At the last RAN2 meeting, EVEX was discussed, and no conclusions were made.

For EVEX, the relevant specifications are:

- TS 26.531 [5], Stage-2 for EVEX

- TS 26.532 [6], Stage-3 for EVEX

According to [5] and [6], our understanding is that EVEX is about the data collection for UE application layer, and UE can transfer the data to Data Collection AF. So far, the data is not involving AS related data.

Observation 4: For EVEX, the data is only from UE application layer.

For different LCM purpose, the network entities may be different, so the motivation should be clarified. In addition, the current EVEX has not included AS related data, because there is no use case for the Data Collection AF to handle the data. If AS related data is considered for EVEX, we observe the following issues:

- How the user privacy is guaranteed for the relevant AS data. For AIML-based features, the relevant data is still under RAN1 discussions, and our understanding is that most of the data are about radio information. The data is privacy sensitive, and some of RAN2 identified data collection mechanisms have provided user privacy protection (e.g. MDT). If the configuration and collection of the data is performed by the Data Collection AF, we have concerns on the user privacy part. For example, TS 26.531 has defined “NOTE:	The Data Collection AF may be deployed outside the trusted domain, in which case the services it exposes to API invokers are mediated by the NEF.”, so we wonder how the user privacy is protected if an entity outside the trusted domain is involved

- How the data is exchanged between UE AS layer and UE Application layer, and how the configuration works

- UE capability considerations

Proposal 7: For EVEX for the purpose of AI for air interface, the following aspects should be discussed:

- Motivation

- How the user privacy is guaranteed for the relevant AS data

- How the data is exchanged between UE AS layer and UE Application layer, and how the configuration works

- UE capability considerations

3   Conclusion

In conclusion, we have the following observations and proposals:

On the requirements

Proposal 1: RAN2 to consider using Table 1/2/3 as starting point of data collection requirement per use case per LCM, and follow RAN1 progress.

Proposal 2: For the input/output data, RAN2 can follow the RAN1 discussion on the requirements, such as:

- For each measurement, which entity generates it

- For each measurement, whether it is needed to send it from node A to node B

Proposal 3: For data collection requirements, it is proposed RAN2 to also discuss the following aspects for model training, inference and monitoring:

- Latency for the reporting

- Need to be (temporarily) stored at the UE

- Data size

On the applicability

Observation 1: For L1 measurement (CSI reporting), if all the data to be collected can be supported by existing reporting scheme, L1 measurement (CSI reporting) can be reused.

Proposal 4: For CSI/BM use cases, immediate MDT can be considered to do the data collection. How to enhance immediate MDT can be further discussed.

Proposal 5: For Positioning use case, if data collection requirement is between UE and gNB, immediate MDT mechanism can be considered to do the data collection; and if data collection requirement is between UE and LMF, LPP mechanism can be considered to do the data collection. How to enhance immediate MDT/LPP can be further discussed.

On the data overhead and possible enhancements

Observation 2: For logged MDT, the UE is required to support 64KB for the buffer, and the logMeasAvailable-r16 and UEInformation procedure are used for transferring the whole report. There may be multiple procedures for the transferring.

Observation 3: The segmentation of UL RRC message is only applicable to UECapabilityInformation and MeasurementReportAppLayer, and it is not applicable to other UL RRC messages (related to data collection mechanisms).

Proposal 6: For data collection mechanisms (i.e. the UE generates and transfers data only in RRC connected state), if the data size is more than 9KB, the current mechanisms can not work, and some enhancements can be considered, e.g. the segmentation of UL RRC message.

On EVEX

Observation 4: For EVEX, the data is only from UE application layer.

Proposal 7: For EVEX for the purpose of AI for air interface, the following aspects should be discussed:

- Motivation

- How the user privacy is guaranteed for the relevant AS data

- How the data is exchanged between UE AS layer and UE Application layer, and how the configuration works

- UE capability considerations

4   Reference

[1]  	Draft_R2_121_meeting_report_v1

[2]	R2-2300708	Open issues on AI/ML model delivery and data collection in post-meeting email discussion	Apple

[3]	R2-2302286 	Summary of [AT121][025]: Progress table of analyzing data collection framework (Apple)	Apple

[4]	R2-2303893, Discussion on model ID and mapping of functionality to entities, Huawei, HiSilicon

[5]	TS 26.531, Technical Specification Group Services and System Aspects; Data Collection and Reporting; General Description and Architecture (Stage-2 for EVEX)

[6]	TS 26.532, Technical Specification Group Services and System Aspects; Data Collection and Reporting; Protocols and Formats (Stage-3 for EVEX)