3GPP TSG-RAN WG2 Meeting #122	R2-2306118

Incheon, Korea, 22nd May – 26th May 2023					 Revision of R2-2303925

Agenda Item:	6.7.2

Source: 	ASUSTeK, vivo, ZTE Corporation, Sanechips

Title:  	Discussion on deriving timer length for DRX timers

Document for:	Discussion and Decision

- Introduction

In RAN2#121 and RAN2#121bis-e meeting, there were discussions regarding determination of symbol length for drx-HARQ-RTT-TimerSL and the slot length for drx-RetransmissionTimerSL. During email discussion [AT121bis-e][505] [1], no conclusion was made and the change was postponed:

R2-2304223 Summary on [AT121bis-e][505][V2XSL] DRX timer numerology ASUSTek … R2-2304224 Corrections on deriving timer length for DRX timers ZTE Corporation, Sanechips, ASUSTeK, vivo CR Rel-17 38.331 17.4.0 4032 1 F NR_SL_enh-Core => Postponed

In this contribution, we further clarify the scenario, summarizes companies’ view on this topic and provides options for solving this issue.

- Discussion

Further clarification on problem regarding SL grant without PDCCH

In the current specification, the value of the length of drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL is in number of symbols or slots of the DL BWP where “the PDCCH” was transmitted:

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL Value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL Value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

For dynamic sidelink grant, the intended PDCCH should be the PDCCH scheduling the sidelink grant. For configured sidelink grant (i.e. sidelink configured grant Type 1), however, it is unclear which PDCCH the description is referring to, and therefore it is unclear which DL BWP should be used for the UE to derive the numerology of the timers.

Observation: 	For sidelink configured grant Type 1, it is unclear which DL BWP numerology the UE should use as a reference to derive the symbol length of drx-HARQ-RTT-TimerSL and slot length of drx-RetransmissionTimerSL.

Reference DL BWP for sidelink configured grant Type 2

During the online session in the previous meeting, it is raised by some companies whether the same approach used for SL CG Type 2 can be used for SL CG Type 1, but there were some different understanding on what is the current behaviour for deriving timer length for Type 2 as well. That is, whether the reference DL BWP for Type 2 is the DL BWP on which the PDCCH scheduling the CG configuration is received or the PDCCH activating the CG. Based on the feedbacks in email discussion [1], it seems most companies’ understanding is currently the activation command is used as the reference PDCCH for Type 2, and spec changes should be applied for  SL CG Type 1 only. It would be better to clarify this behaviour for Type 2 before we discuss reference DL BWP for sidelink configured grant Type 1.

Proposal 1: 	For sidelink configured grant Type 2, the reference PDCCH, to derive the symbol length of drx-HARQ-RTT-TimerSL and slot length of drx-RetransmissionTimerSL, is the PDCCH activating the sidelink configured grant Type 2.

Reference DL BWP for sidelink configured grant Type 1

As for SL CG Type 1, in the previous discussion, there are several solutions proposed by companies. Combining further suggestions and comments provided by other companies during and after online discussion, options for Type-1 SL CG timer derivation are listed as below:

- Option 1: referring to active DL BWP.

  - Option 1a: referring to active DL BWP of the PCell.

  - Option 1b: referring to active DL BWP where DCI format 3_0 was monitored.

- Option 2: referring to the DL BWP on which the PDCCH transmission indicating the PDSCH carrying the RRCReconfiguration containing rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted.

- Option 3: leave it to UE implementation.

Option 1: referring to active DL BWP.

For Option 1a, it was the majority companies’ preference in email discussion [505] that the UE refers the active DL BWP of the PCell when deriving timer length for SL CG Type 1. This would be a unified solution for all UEs. On the other hand, there are some concerns that the selection of the DL BWP should not be restricted on PCell only, and should be based on active DL BWP of the Cell where DCI for activating/scheduling SL grant was monitored. Based on 38.213, the UE does not expect to receive DCI format 3_0 on more than one Cells, so it would be fixed to be the PCell or one of the SCells, so the UE can select the Cell for deriving the reference DL BWP without ambiguity and without restriction to PCell only:

(38.213) # 16 UE procedures for sidelink … A UE does not expect to be provided search space sets associated with CORESETs on more than one cell to monitor PDCCH for detection of DCI format 3_0 or DCI format 3_1.

Possible text proposal for option 1a is as below:

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL For sidelink configured grant Type 1, value in number of symbols of the activated DL BWP of PCell. For other cases, value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL For sidelink configured grant Type 1, value in number of slot lengths of the activated DL BWP of PCell. For other cases, value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

Option 2: referring to the DL BWP on which the PDCCH transmission indicating the PDSCH carrying the RRCReconfiguration containing rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted.

For Option 2, one advantage would be that the numerology selected for deriving the timers will be aligned with the numerology when configuring the Type 1 SL CG. On the other hand, this would lead to the UE having to store the information of which DL BWP was used to schedule MAC PDUs and the associations between the MAC PDUs, disassembled RLC PDUs, disassembled PDCP PDUs, and disassembled RRC messages. UE implementation complexity is increased for protocol layers interactions.

In addition, different type-1 CG configurations can be provided in different RRC messages, and each RRC message containing the type-1 CG configuration can be segmented into multiple transmissions. If the transmissions encompasses multiple DL BWP and/or numerologies, it is still unclear for the UE which one to refer to among all the DL BWPs. While some companies questioned whether this is a valid scenario, the current specification doesn’t seem to avoid this behaviour for message transmission. If segments of a RRC reconfiguration message transmission will always be received via a same DL BWP, this option would be more viable.

Possible text proposal for option 2 is as below:

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL Value in number of symbols of the BWP where the PDCCH scheduling or activating the corresponding SL grant or the PDCCH indicating the PDSCH carrying the RRCReconfiguration containing the rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL Value in number of slot lengths of the BWP where the PDCCH scheduling or activating the corresponding SL grant or the PDCCH indicating the PDSCH carrying the RRCReconfiguration containing the rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

Option 3: referring to SL BWP where the transport block is transmitted

In Uu, the timer length of the corresponding DRX timers for UL, drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL, are derived based on BWP where the transport block was transmitted:

drx-RetransmissionTimerUL Value in number of slot lengths of the BWP where the transport block was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

Reusing what is specified for drx-HARQ-RTT-TimerUL and drx-RetransmissionTimerUL is also a viable way to derive timer length for corresponding timers for Type 1 SL CG. For example:

DRX-ConfigSL field descriptions

drx-HARQ-RTT-TimerSL For sidelink configured grant Type 1, value in number of symbols of the SL BWP where the transport block was transmitted. For other cases, value in number of symbols of the BWP where the PDCCH was transmitted. Value 0 is used in case sl-PUCCH-Config is not configured and the corresponding resource pool is not configured with PSFCH.

drx-RetransmissionTimerSL For sidelink configured grant Type 1, value in number of slot lengths of the SL BWP where the transport block was transmitted. For other cases, value in number of slot lengths of the BWP where the PDCCH was transmitted. sl0 corresponds to 0 slots, sl1 corresponds to 1 slot, sl2 corresponds to 2 slots, and so on.

Option 4: leave to UE implementation.

While it can be left to UE implementation to select any of the DL BWPs (either the ones carrying the RRC messages or arbitrary DL BWP), it may cause UE DRX active time state to be unsynchronized between the UE and the network, which could cause data loss. While the network implementation may ensure data can be received by the UE for all possible combination/DL BWP selected, it would be an unnecessary restriction for NW scheduling. If UE implementation is selected for SL CG Type 1, at least a NOTE would be needed to clarify that it is up to UE implementation for numerology selection for SL CG Type 1.

Proposal 2:	Spec change is needed for SL UE to derive symbol length for drx-HARQ-RTT-TimerSL and the slot length for drx-RetransmissionTimerSL corresponding to SL configured grant Type 1.

In conclusion, we would propose RAN2 to select one of the 3 options to handle derivation for timer length for drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL.

Proposal 3:	RAN2 to selects from one of the following Options for derivation of timer length for drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL for sidelink configured grant Type 1:

- Option 1: referring to active DL BWP.

        - Option 1a: referring to active DL BWP of the PCell.

        - Option 1b: referring to active DL BWP where DCI format 3_0 was monitored.

- Option 2: referring to the DL BWP on which the PDCCH transmission indicating the PDSCH carrying the RRCReconfiguration containing rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted.

- Option 3: referring to the SL BWP where the transport block is transmitted.

- Option 4: leave it to UE implementation.

- Conclusion

We have the following observation and proposals for SL DRX timer derivation:

Observation: 	For sidelink configured grant Type 1, it is unclear which DL BWP numerology the UE should use as a reference to derive the symbol length of drx-HARQ-RTT-TimerSL and slot length of drx-RetransmissionTimerSL.

Proposal 1: 	For sidelink configured grant Type 2, the reference PDCCH, to derive the symbol length of drx-HARQ-RTT-TimerSL and slot length of drx-RetransmissionTimerSL, is the PDCCH activating the sidelink configured grant Type 2.

Proposal 2:	Spec change is needed for SL UE to derive symbol length for drx-HARQ-RTT-TimerSL and the slot length for drx-RetransmissionTimerSL corresponding to SL configured grant Type 1.

Proposal 3:	RAN2 to selects from one of the following Options for derivation of timer length for drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL for sidelink configured grant Type 1:

- Option 1: referring to active DL BWP.

        - Option 1a: referring to active DL BWP of the PCell.

        - Option 1b: referring to active DL BWP where DCI format 3_0 was monitored.

- Option 2: referring to the DL BWP on which the PDCCH transmission indicating the PDSCH carrying the RRCReconfiguration containing rrc-ConfiguredSidelinkGrant for the corresponding SL grant was transmitted.

- Option 3: referring to the SL BWP where the transport block is transmitted.

- Option 4: leave it to UE implementation.

- Reference

[1] R2-2304223	Summary on [AT121bis-e][505][V2XSL] DRX timer numerology	ASUSTek