3GPP TSG-RAN WG2 Meeting #122	R2-2306120

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:	7.4.2.3

Source: 	ASUSTeK

Title:  	Discussion on fallback RACH for L1L2-triggered mobility

Document for:	Discussion and Decision

- Introduction

In RAN2#121bis-e meeting, some agreements and assumptions were made regarding model for L1L2-triggered mobility (LTM):

R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data. Following behaviors of LTM supervisor timer are agreed: - 1: The UE starts the LTM supervisor timer, upon reception of the LTM cell switch MAC CE; - 2: The UE stops the LTM supervisor timer, upon successful completion of LTM cell switch; - 3: If the LTM supervisor timer for MCG expires, as baseline, the UE considers LTM failure and initiates RRC re-establishment. (SCG switch case FFS) LTM supervisor timer is RRC layer timer. At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB. […]

In this contribution, we discuss further details on possible procedures for dynamic switching in L1 L2 mobility procedure based on previous discussions and R1/R2 status.

- Discussion

Fallback mechanism for RACH-less LTM

In the previous meeting, a supervisor timer has been agreed for failure handling of LTM. For RACH-less LTM, in addition to previously acquired TA (e.g. provided by NW or via early TA acquisition), the UE needs to determine beam(s) to be used to connect to target Cell in the LTM procedure, e.g. to monitor PDCCH or to transmit the LTM complete message. In RAN1#112 meeting [1], agreements were made that beam indication for the target Cell(s) is conveyed in MAC CE:

(RAN1#112 agreement) Agreement - RAN1 shares the same understanding as RAN2 on agreement: - The LTM mobility trigger information is conveyed in a MAC CE - The same MAC CE is used for the LTM triggering. Agreement The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

If the beam misalignment occurs when the indicated beam is not qualified for performing RACH-less LTM (e.g. quality of the UL TCI state is lower than a threshold), the RACH-less LTM may not be successful. It is not effective to keep using the unqualified beam for the RACH-less LTM, and it would induce unnecessary latency to wait for timer expiry and perform re-establishment. Before the timer expiry, a fallback mechanism can be leveraged for the UE to perform RACH-based LTM instead and can reselect a beam for attempting to connect to the target Cell.

Proposal:	For RACH-less LTM, the UE falls back to RACH-based LTM in the target Cell when quality of the beam, indicated by the MAC CE triggering LTM, is lower than a threshold.

- Conclusion

We have the following proposal for solutions for LTM:

Proposal:	For RACH-less LTM, the UE falls back to RACH-based LTM in the target Cell when quality of the beam, indicated by the MAC CE triggering LTM, is lower than a threshold.

- Reference

[1] Report of 3GPP TSG RAN WG1 #112