3GPP TSG-RAN WG2 Meeting #122	R2-2306122

Incheon, Korea, 22nd May – 26th May 2023

Agenda Item:		7.7.4.2

Source:				ASUSTeK

Title:		Discussion on handover enhancement with common signalling

Document for: 	Discussion and Decision

- Introduction

In previous RAN2 meetings, it has been agreed to consider common configuration for handover in release 18 NTN. In the last meeting, the signalling gain for reducing common configuration in dedicated handover command has been discussed. In this contribution, we further discuss the common configuration for handover enhancements in NTN.

- Discussion

Based on TR 38.821 [1], the average hand-out rate for a LEO cell of diameter 50 km could be 9912 UEs/s. And as discussed in the offline discussion [AT121bis-e][108] in the last meeting, each UE could save 12.5% of the RRCReconfiguration message if ServingCellConfigCommon is delivered via a common signalling. In the summary [2] of this offline discussion, the rapporteur proposes a way forward that providing the common target cell configuration of the handover command (e.g. ServingCellConfigCommon) via broadcast is supported in NR NTN release 18.

Observation 1: The benefit of reducing common target cell configuration in dedicated handover command has been discussed in RAN2 #121bis meeting.

The network could determine whether to provide the common configurations (e.g. servingCellConfigCommon, t304 and smtc) in a system information. The network could broadcast the common configurations before the stop serving time of the serving cell or when an amount of UE is going to lose coverage. With the flexibility, the network doesn’t need to broadcast the common configurations for a long time. Then the network could transmit a dedicated handover command with reduced configurations to each UE that needs handover to the same target cell.

Proposal 1: The network can broadcast the common configurations in a system information, and then transmit a RRCReconfiguration message to trigger handover for the UE.

The UE could receive the system information including common configurations before handover, and then receive a RRCReconfiguration without the common configurations. When the UE receives the RRCReconfiguration, the UE could use the common configuration from the system information to perform handover.

Proposal 2: The UE uses the common configurations for target cell from the system information, if the common configurations are not provided in the dedicated handover command (i.e. RRCReconfiguration message).

Within the configuration of ReconfigurationWithSync, most configurations doesn’t need to be included in a dedicated handover command. Since ServingCellConfigCommon is the common configuration for a target cell, this configuration could be provided in a system information to multiple UEs. Other configurations such as t304 and smtc could also be provided in system information if a common value is to be used for the target cell. The configuration of ReconfigurationWithSync form TS 38.331 [3] is quoted as below. Among the configuration of ReconfigurationWithSync, t304 is the timer value of timer T304 to handle reconfiguration with sync failure. And smtc indicates the SSB periodicity/offset/duration configuration of the target cell. ServingCellConfigCommon, t304 and smtc could be configured with the same value in the target cell. The UEs handover to the same target cell could use the same values for these configurations. DAPS and sidelink are not supported in NTN, and thus daps-UplinkPowerConfig and sl-PathSwitchConfig doesn’t need to be included in the corresponding handover command. For the case that dedicated RACH resource is not used for handover, the network could only provide C-RNTI in ReconfigurationWithSync.

ReconfigurationWithSync ::= SEQUENCE {

spCellConfigCommon       ServingCellConfigCommon                           OPTIONAL,   -- Need M

newUE-Identity           RNTI-Value,

t304                     ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated     CHOICE {

uplink                  RACH-ConfigDedicated,

supplementaryUplink     RACH-ConfigDedicated

}                                                                          OPTIONAL,   -- Need N

...,

[[

smtc                     SSB-MTC                                           OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16 DAPS-UplinkPowerConfig-r16                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17    SL-PathSwitchConfig-r17                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

Proposal 3: Within ReconfigurationWithSync, configurations other than C-RNTI and optionally dedicated RACH configuration for handover could be provided in system information and be omitted in dedicated handover command.

Moreover, as discussed in the offline discussion [2], the number of target cells for which common configuration is provided in the system information could be limited to 2. Since the trajectory of serving cell is predictable, the network doesn’t need to provide common configurations for many neighbor cells. And it could be simple to add the common configurations in SIB19. A text proposal is provided as below.

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                    NTN-Config-r17                       OPTIONAL,       -- Need R

t-Service-r17                     INTEGER (0..549755813887)            OPTIONAL,       -- Need R

referenceLocation-r17             ReferenceLocation-r17                OPTIONAL,       -- Need R

distanceThresh-r17                INTEGER(0..65525)                    OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17       NTN-NeighCellConfigList-r17          OPTIONAL,       -- Need R

lateNonCriticalExtension          OCTET STRING                         OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720  NTN-NeighCellConfigList-r17          OPTIONAL        -- Need R

]]

[[

ntn-TargetCellList-r18            NTN-TargetCellList-r18               OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=   SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=       SEQUENCE {

ntn-Config-r17                    NTN-Config-r17                       OPTIONAL,       -- Need R

carrierFreq-r17                   ARFCN-ValueNR                        OPTIONAL,       -- Need R

physCellId-r17                    PhysCellId                           OPTIONAL        -- Need R

}

NTN-TargetCellList-r18 ::=   SEQUENCE (SIZE(1..2))  OF NTN-TargetCellConfig-r18

NTN-TargetCellConfig-r18 ::=       SEQUENCE {

spCellConfigCommon       ServingCellConfigCommon,

t304                     ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},   -- Need R

smtc                     SSB-MTC                                        OPTIONAL,    -- Need R

...,

}

Proposal 4: The network provides up to 2 target cells with the common configurations in SIB19.

As for the dedicated handover command, the network could configure another ReconfigurationWithSync to overwrite the legacy one. servingCellConfigCommon could be omitted in the new ReconfigurationWithSync. And t304 and smtc could be optionally present based on network configuration. In addition, a PCI should be needed to indicate the target cell. Then the UE could acquire corresponding common configuration form the system information. A text proposal is provided as below.

SpCellConfig ::=                        SEQUENCE {

servCellIndex                       ServCellIndex                                               OPTIONAL,   -- Cond SCG

reconfigurationWithSync             ReconfigurationWithSync                                     OPTIONAL,   -- Cond ReconfWithSync

rlf-TimersAndConstants              SetupRelease { RLF-TimersAndConstants }                     OPTIONAL,   -- Need M

rlmInSyncOutOfSyncThreshold         ENUMERATED {n1}                                             OPTIONAL,   -- Need S

spCellConfigDedicated               ServingCellConfig                                           OPTIONAL,   -- Need M

...,

[[

lowMobilityEvaluationConnected-r17  SEQUENCE {

s-SearchDeltaP-Connected-r17        ENUMERATED {dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1},

t-SearchDeltaP-Connected-r17        ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5,

spare4, spare3, spare2, spare1}

}                                                                                               OPTIONAL,   -- Need R

goodServingCellEvaluationRLM-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

goodServingCellEvaluationBFD-r17    GoodServingCellEvaluation-r17                               OPTIONAL,   -- Need R

deactivatedSCG-Config-r17           SetupRelease { DeactivatedSCG-Config-r17 }                  OPTIONAL    -- Cond SCG-Opt

]]

[[

reconfigurationWithSync-r18         ReconfigurationWithSync-r18                                     OPTIONAL,   -- Need R

]]

}

ReconfigurationWithSync-r18 ::=         SEQUENCE {

physCellId                                                  PhysCellId,

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000}                                                               OPTIONAL,   -- Need R

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                          OPTIONAL,   -- Need R

smtc                                SSB-MTC                                OPTIONAL    -- Need R

}

| Conditional Presence | Explanation |
| --- | --- |
| ReconfWithSync | The field is mandatory present in the RRCReconfiguration message: - in each configured CellGroupConfig for which the SpCell changes, - in the masterCellGroup: - at change of AS security key derived from KgNB, - in an RRCReconfiguration message contained in a DLInformationTransferMRDC message, - path switch of L2 U2N remote UE to the target PCell, - path switch of L2 U2N remote UE to the target L2 U2N Relay UE, - in the secondaryCellGroup at: - PSCell addition, - SCG resume with NR-DC or (NG)EN-DC, - update of required SI for PSCell, - change of AS security key derived from S-KgNB in NR-DC while the UE is configured with at least one radio bearer with keyToUse set to secondary and that is not released by this RRCReconfiguration message, - MN handover in (NG)EN-DC. Otherwise, it is optionally present, need M. The field is absent in the masterCellGroup in RRCResume and RRCSetup messages and is absent in the masterCellGroup in RRCReconfiguration messages if source configuration is not released during DAPS handover. If ReconfigurationWithSync-r18 is present, the field is ignored. |

Proposal 5: The network configures a new ReconfigurationWithSync in RRCReconfiguration message to omit the common configurations.

- Conclusion

In this contribution, we discuss the handover enhancement with common signalling in NTN and have following observation and proposals:

Observation 1: The benefit of reducing common target cell configuration in dedicated handover command has been discussed in RAN2 #121bis meeting.

Proposal 1: The network can broadcast the common configurations in a system information, and then transmit a RRCReconfiguration message to trigger handover for the UE.

Proposal 2: The UE uses the common configurations for target cell from the system information, if the common configurations are not provided in the dedicated handover command (i.e. RRCReconfiguration message).

Proposal 3: Within ReconfigurationWithSync, configurations other than C-RNTI and optionally dedicated RACH configuration for handover could be provided in system information and be omitted in dedicated handover command.

Proposal 4: The network provides up to 2 target cells with the common configurations in SIB19.

Proposal 5: The network configures a new ReconfigurationWithSync in RRCReconfiguration message to omit the common configurations.

- Reference

- TR 38.821 V16.1.0 Solutions for NR to support NTN.

- R2-2304248, “Report of [AT121bis-e][108][NR NTN Enh] Common (C)HO configuration (Ericsson).”

- TS 38.331 V17.4.0 3GPP TSG RAN NR RRC protocol specification.