3GPP TSG-RAN WG2 Meeting #122	R2-2306125

Incheon, Korea, 22th – 26th May 2023

Agenda Item:	7.9.2

Source:	ASUSTeK

Title:	Discussion on aspects of AS layer configuration for L2 U2U Relay

Document for:	Discussion and Decision

1	Introduction

In RAN2#121 meeting, the following agreements were made:

RAN2 confirms the user plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-1 and control plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-2 of TR 38.836 [2].

RAN2 confirms Remote UE E2E Radio Bearer ID should be included in the adaptation layer in first and second PC5 hop.

RAN2 confirms Remote UE determines the egress RLC channel based on the mapping from the E2E bearer ID to egress RLC channel, for a particular target Remote UE.

FFS if multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the destination remote UE is needed in the first hop (Tx remote UE to relay), at least in case multiplexing of different destinations in the same RLC channel is supported.

An ID mappable to the source remote UE is needed in the second hop (relay to Rx remote UE).

FFS if the IDs are different (e.g., source and destination UE IDs) or common (e.g., a local ID for the pair).

FFS whether both UE IDs are included in the header or the relay UE does a mapping.

RAN2#121bis-e meeting further made the following agreements:

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

Multiplexing of different destinations in the same RLC channel of the first hop is supported.

RAN2 confirms that multiplexing of the different bearers from the different source remote UEs into the same RLC channel in the second hop is supported.

Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID and egress RLC Channel for a particular pair between source remote UE and target remote UE.

WA: E2E bearer ID (i.e., configuration index in the list of SLRB configurations) is used as input for the L2 U2U relay ciphering and deciphering at PDCP.

LS to SA3 to confirm the feasibility of using the configuration index.

In this contribution, we first discuss support of E2E PC5-S messages and then discuss how to configure end-to-end (E2E) SLRBs and PC5 RLC channels for supporting L2 UE-to-UE (U2U) Relay.

2	Discussion

2.1 	E2E PC5-S messages for supporting L2 U2U Relay

It was agreed in RAN2#121bis-e that end-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.

According to  [1], the signalling of the E2E PC5 link establishment performed between two ProSe end UEs (i.e. Source UE and Target UE) are forwarded by U2U Relay UE after the unicast links between each remote UE and the relay UE on both hops are established. The figure in clause 6.7.2 of [1] for describing the overall of connection establishment for L2 U2U relay communication is quoted below.

### 6.7.2 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay This procedure applies to 5G ProSe Layer-2 UE-to-UE Relay. Figure 6.7.2-1: 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay … 4. The source 5G ProSe End UE establishes an end-to-end connection for unicast mode communication with the target 5G ProSe End UE as described in clause 6.4.3.7. The data is transferred between the source 5G ProSe End UE and the target 5G ProSe End UE via the 5G ProSe Layer-2 UE-to-UE Relay. The 5G ProSe Layer-2 UE-to-UE Relay forwards all the data messages between the source 5G ProSe End UE and the target 5G ProSe End UE, as specified in TS 38.300 [12].

Since RAN2 confirms the user plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-1 and control plane protocol stack for L2 UE-to-UE Relay in Figure 5.5.1-2 of TR 38.836 [2], the adaptation layer is supported for transfer of E2E user plane traffic and PC5-RRC message. However, whether the adaptation layer is also supported for transfer of E2E PC5-S message has not been discussed yet.

As described in Figure 6.7.2-1 and clause 6.4.3.7 in [1], the E2E PC5-S messages may include at least those PC5-S messages (e.g. DCR message, Security messages, and DCA message) exchanged between Source UE and Target UE during the E2E PC5 link establishment. Thus, forwarding E2E PC5-S messages between the two ProSe end UEs via U2U Relay UE is required for supporting L2 UE-to-UE Relay. Basically, E2E PC5-S messages also belong to C-plane signalling. Therefore, the adaptation layer is also needed for E2E PC5-S message forwarding.

- The adaptation layer is needed for E2E PC5-S message forwarding.

Further taking existing R17 PC5-S messages into consideration, there will be two categories of PC5-S message for supporting L2 UE-to-UE relaying: (1) per-hop PC5-S messages transmitted between ProSe end UE and U2U Relay UE and (2) E2E PC5-S messages transmitted between two ProSe end UEs via U2U Relay UE.

- According to TS23.304-i10 [1], there will be two categories of PC5-S message for supporting L2 UE-to-UE relay: (1) per-hop PC5-S messages transmitted between ProSe end UE and U2U Relay UE and (2) E2E PC5-S messages transmitted between two ProSe end UEs via U2U Relay UE.

Considering that E2E PC5-S messages are transmitted between Source Remote UE and Target Remote UE via L2 U2U Relay UE with an adaptation layer while per-hop PC5-S messages are transmitted between ProSe end UE (e.g. Source/Target Remote UE) and L2 U2U Relay UE without adaptation layer, we think these two categories of PC5-S message should be transmitted in separate sets of PC5 RLC channels. Thus, we propose the following:

- In L2 UE-to-UE Relay, separate PC5 RLC channels are used for transmitting (1) per-hop PC5-S messages between ProSe end UE and U2U Relay UE and (2) E2E PC5-S messages between ProSe end UEs via U2U Relay UE.

- A control plane protocol stack for E2E PC5-S messages is added in R18 TS 38.300 for supporting L2 UE-to-UE Relay.

2.2 	AS layer configuration aspect

In legacy sidelink communication, when a new PC5 QoS flow is created, the UE would add a new SL DRB with the peer UE. Besides, the UE also configures a mapping of the new PC5 QoS flow to the new SL DRB with the peer UE. If the UE is in RRC_CONNECTED, gNB provides the UE with the SL-RadioBearerConfig (containing SL-SDAP-Config and SL-PDCP-Config) and the SL-RLC-BearerConfig (containing SL-RLC-Config and SL-LogicalChannelConfig) for establishing the PC5 QoS flow-to-SL DRB mapping, the new SL DRB and the corresponding SL logical channel based on reported information. We think L2 U2U relay communication can follow the same concepts in legacy sidelink communication with following modifications.

For a new PC5 QoS flow, Source remote UE in RRC_CONNECTED reports the QoS profile of the new PC5 QoS flow associated with the first hop to its serving gNB for providing a SDAP configuration and a PDCP configuration for Source remote UE to establish a new E2E SL DRB. gNB also provides Source remote UE with a 1st hop PC5 RLC channel configuration to establish a first hop PC5 RLC channel mapped to the new E2E SL DRB. Since Source remote UE may connect with multiple Target remote UEs via the U2U Relay UE, there is a need for Source remote UE to include an ID of the Target remote UE in the SidelinkUEInformation message to distinguish PC5 QoS flows with the same QoS flow ID (associated with different Target remote UEs), in addition to the destination ID of the U2U Relay UE.

- In addition to destination ID of the U2U Relay UE, Source remote UE in RRC_CONNECTED includes an ID of Target remote UE in a SidelinkUEInformation message sent to gNB for providing SDAP configuration, PDCP configuration and 1st hop PC5 RLC channel configuration for establishing a new E2E SL DRB for a new PC5 QoS flow. The Target remote UE ID is FFS.

To establish the new E2E SL DRB with Target remote UE, Source remote UE needs to send an E2E RRCReconfigurationSidelink message to Target remote UE to provide the SDAP configuration and the PDCP configuration for establishing the new E2E SL DRB.

- Source remote UE transmits an E2E RRCReconfigurationSidelink message to Target remote UE to provide the SDAP configuration and the PDCP configuration for establishing the new E2E SL DRB for a new PC5 QoS flow.

In addition, Source remote UE also needs to send a RRCReconfigurationSidelink message to U2U Relay UE to provide the 1st hop PC5 RLC channel configuration for establishing the first hop PC5 RLC channel for the new PC5 QoS flow. Similarly, since Source remote UE may connect with multiple Target remote UEs via the U2U Relay UE, there is a need for Source remote UE to include an ID of the Target remote UE in the RRCReconfigurationSidelink message to distinguish PC5 QoS flows from different Target remote UEs so that the U2U Relay UE may then send a SidelinkUEInformation message to the gNB to request sidelink transmission resource for establishing the 2nd hop PC5 RLC channel for the new PC5 QoS flow. Similarly, since multiple Source remote UEs may connect with Target remote UE via the U2U Relay UE, there is a need for the U2U Relay UE to include an ID of the Source remote UE in the SidelinkUEInformation message to distinguish PC5 QoS flows from different Source remote UEs.

- Source remote UE includes an ID of Target remote UE in a RRCReconfigurationSidelink message sent to U2U Relay UE to provide the 1st hop PC5 RLC channel configuration for establishing 1st hop PC5 RLC channel(s) in L2 U2U Relay for a new PC5 QoS flow. The Target remote UE ID is FFS.

- U2U Relay UE in RRC_CONNECTED includes an ID of Source remote UE in a SidelinkUEInformation message so that gNB can provide the 2nd hop PC5 RLC channel configuration for a new PC5 QoS flow. The Source remote UE ID is FFS.

It was agreed in RAN2#121bis-e that Relay UE determines the egress RLC Channel based on the mapping of E2E bearer ID (i.e. configuration index of the SLRB configuration) and egress RLC Channel for a particular pair between source remote UE and target remote UE. In order for gNB to provide U2U Relay UE with the E2E RB-to-egress PC5 RLC channel mapping, we think the U2U Relay UE needs to report the E2E bearer ID associated with the PC5 QoS flow to its serving gNB. In this situation, there is a need for Source remote UE to provide the E2E bearer ID associated with the PC5 QoS flow to the U2U Relay UE.

- Source remote UE includes E2E bearer ID(s) (i.e. configuration index of the SLRB configuration) for the mapped PC5 QoS flow(s) in a RRCReconfigurationSidelink message sent to U2U Relay UE.

- U2U Relay UE in RRC_CONNECTED includes E2E bearer ID associated with a PC5 QoS flow in a SidelinkUEInformation message sent to gNB so that gNB can provide the 2nd hop PC5 RLC channel configuration and E2E RB-to-egress PC5 RLC channel mapping for the L2 U2U Relay.

3 	Conclusion

In this contribution, we observe and propose:

- The adaptation layer is needed for E2E PC5-S message forwarding.

- According to TS23.304-i10 [1], there will be two categories of PC5-S message for supporting L2 UE-to-UE relay: (1) per-hop PC5-S messages transmitted between ProSe end UE and U2U Relay UE and (2) E2E PC5-S messages transmitted between two ProSe end UEs via U2U Relay UE.

E2E PC5-S message related proposals:

- In L2 UE-to-UE Relay, separate PC5 RLC channels are used for transmitting (1) per-hop PC5-S messages between ProSe end UE and U2U Relay UE and (2) E2E PC5-S messages between ProSe end UEs via U2U Relay UE.

- A control plane protocol stack for E2E PC5-S messages is added in R18 TS 38.300 for supporting L2 UE-to-UE Relay.

AS configuration for U2U Relay related proposals:

- In addition to destination ID of the U2U Relay UE, Source remote UE in RRC_CONNECTED includes an ID of Target remote UE in a SidelinkUEInformation message sent to gNB for providing SDAP configuration, PDCP configuration and 1st hop PC5 RLC channel configuration for establishing a new E2E SL DRB for a new PC5 QoS flow. The Target remote UE ID is FFS.

- Source remote UE transmits an E2E RRCReconfigurationSidelink message to Target remote UE to provide the SDAP configuration and the PDCP configuration for establishing the new E2E SL DRB for a new PC5 QoS flow.

- Source remote UE includes an ID of Target remote UE in a RRCReconfigurationSidelink message sent to U2U Relay UE to provide the 1st hop PC5 RLC channel configuration for establishing 1st hop PC5 RLC channel(s) in L2 U2U Relay for a new PC5 QoS flow. The Target remote UE ID is FFS.

- U2U Relay UE in RRC_CONNECTED includes an ID of Source remote UE in a SidelinkUEInformation message so that gNB can provide the 2nd hop PC5 RLC channel configuration for a new PC5 QoS flow. The Source remote UE ID is FFS.

- Source remote UE includes E2E bearer ID(s) (i.e. configuration index of the SLRB configuration) for the mapped PC5 QoS flow(s) in a RRCReconfigurationSidelink message sent to U2U Relay UE.

- U2U Relay UE in RRC_CONNECTED includes E2E bearer ID associated with a PC5 QoS flow in a SidelinkUEInformation message sent to gNB so that gNB can provide the 2nd hop PC5 RLC channel configuration and E2E RB-to-egress PC5 RLC channel mapping for the L2 U2U Relay.

4 	References

[1] TR 23.304 v18.1.0, Proximity based Services (ProSe) in the 5G System (5GS) (Release 18)

[2] TR 38.836 v.17.0.0, Study on NR sidelink relay (Release 17)