3GPP TSG-RAN WG2 Meeting #122      	                                  R2-2306135

Incheon, Korea, May 22-26, 2023

Source: 	Xiaomi

Title:  	Discussion on measurement reporting for NR UAV

Agenda Item:	7.8.2	Measurement reporting for mobility and interference control

Document for:	Discussion and Decision

Introduction

A revised WID on NR support for UAV (Uncrewed Aerial Vehicles) [1] has been approved. In the WID, one objective is to enhance measurement reports as described below.

1. Specify the following enhancements on measurement reports [RAN2]: - UE-triggered measurement report based on configured height thresholds - Reporting of height, location and speed in measurement report - Flight path reporting - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In this contribution, we will provide our understandings on measurement reporting for NR UAV.

Discussion

  - The content of the measurement report

In previous meetings, RAN2 agreed to use the LTE principle as a baseline for measurement reporting of NR UAV and event H1 and event H2 have been introduced. And when event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results) [2]. For the content of the measurement report triggered by event H1 or H2, there are still some issues:

1) Whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting

In LTE UAV, event H1 and event H2 can be used for the height reporting, and aerial UE triggers a height report when event H1 or event H2 is fulfilled. If the triggerType is set to event and eventId is set to eventH1 or eventH2, UE shall set the heightUE to include the altitude of the UE in measurement report. And the height information is always included in the measurement report triggered by event H1 or event H2 [3]. In LTE, the heightUE indicates the height of the UE in meters relative to the sea level [3].

For NR UAV, the LTE principle and procedure can be reused. When event H1 or event H2 is fulfilled, the height of aerial UE should be included in the measurement report for NR UAV, i.e. UAV UE’s height is mandatorily included in the measurement report. As in LTE, the separate height reporting should be supported for NR UAV, and a field like heightUE should be included in measurement results for height reporting to indicate the height of the UE in meters relative to the sea level.

Proposal 1: As in LTE, when event H1 or event H2 triggers, UAV UE’s height is mandatorily included in the measurement report.

Proposal 2: For NR UAV, a field like heightUE in TS 36.331 can be used for height reporting to indicate the height of the UE in meters relative to the sea level.

2) Which parameter/IE is used for location and speed reporting

For location reporting of NR UAV, the existing location reporting mechanism can be reused for the measurement report triggered by event H1 or event H2. UE location information, i.e. CommonLocationInfo shall be included in the measurement report if the includeCommonLocationInfo is configured. Horizontal and vertical speed can be included in the location information.

Proposal 3: For measurement reporting triggered by event H1 and event H2, the existing location reporting mechanism can be reused. If the includeCommonLocationInfo is configured by network, UE location information, i.e. CommonLocationInfo shall be included in the measurement report.

Proposal 4: Horizontal and vertical speed can be included in the location information.

3) Which measurement results can be included in the measurement report

In the existing RRC procedure of measurement reporting triggered by event Ax, measurement results (RSRP/RSRQ/SINR) of serving cells and neighbour cells can be included the content of the measurement report. The RS type and quantities of measurement results are configured by network in reportConfig. And which and how many neighbour cells can be included in the measurement report are also configured by network. For measurement reporting triggered by event H1 or event H2, the existing procedures can be reused and measurement results of serving cells and neighbour cells can be included in the content of the measurement report. UE reports the RSRP/RSRQ/SINR measurement results of serving cells and neighbour cells based on the network configuration.

Proposal 5: For measurement reporting triggered by event H1 or event H2, measurement results of serving cells and neighbour cells can be included in the content of the measurement report based on the network configuration.

  - Height-depending combinations

In previous meetings, RAN2 agreed to support the joint use of height-dependent condition and RSRP/RSRQ/SINR-based condition for measurement report triggering in NR Rel-18 UAV and the combination of existing events will be used [2]. However, how to combine Event H1/H2 and Event Ax need to be discussed.

For existing CHO, the combination of multiple events has been supported to trigger the conditional handover. In CHO, network can indicate two measIds to UE to combine two events. The similar mechanism can reused for the combination of measurement events for NR UAV. A new IE can be introduced to indicate two combined measIds. One measId is associated with event H1/H2 and the other measId is associated with event Ax.

Proposal 6: To support the combination of Event H1/H2 and Event Ax, a new IE can be introduced to indicate two combined measIds, and one measId for event H1/H2, the other measId for event Ax.

For mobility management, at least event A3, A4 and A5 can be considered for the combination of Event H1/H2 and Event Ax.

Proposal 7: At least event A3, A4 and A5 can be considered for the combination of Event H1/H2 and Event Ax.

For the combination of multiple events, how to trigger measurement reporting is still FFS. In CHO. When all events are fulfilled, CHO will be triggered. Similar to CHO, only when all combined events are fulfilled, measurement reporting will be triggered. If one event becomes non-fulfilled, the measurement reporting will be stopped.

Proposal 8: When all combined events are fulfilled, measurement reporting will be triggered. If one event becomes non-fulfilled, the measurement reporting will be stopped.

  - Height-depending more-than-one configurations

In previous meetings [2][4], RAN2 reached the following agreements for height-dependent more-than-one configurations:

Agreements in RAN2#121:

- Support configuring height-dependent more-than-one configurations targeting measurement and measurement reporting enhancement. UE applies corresponding configuration based on the UE height. The proposed solutions should aim at avoiding RAN4 impacts.  FFS how this would be configured (i.e. different MO configurations or different parameters FFS Exact parameters and details.

Agreements in RAN2#121bis-e:

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

According to above agreements, RAN2 reached a consensus on which configuration(s)/parameter(s) need ability to be configured with different height-dependent configurations/values. However, how to specify multiple configurations for different height or height range is FFS.

Because height-dependent more-than-one configurations is supported on different fields/values within the same MO, for SSB-ToMeasure in the MO configuration, multiple SSB-ToMeasure for different height or height range can be configured in one MO configurations. Then, we should specify the maximum number of parameters/values for SSB-ToMeasure that can be configured in one MO configuration.

Proposal 9: Multiple SSB-ToMeasure for different height or height range can be configured in one MO configuration to achieve height-depending more-than-one SSB-ToMeasure. And RAN2 can discuss the maximum number of parameters/values for SSB-ToMeasure that can be configured in one MO configuration.

For MR configurations, how to specify height-dependent configuration of MR configuration parameters has been discussed in [5]. More than half of companies think is can be achieve by the combination of event H1/H2 and event Ax. We share the same view with majority and support to use the combination of events H1/H2 with event Ax to achieve height-dependent configuration of MR configuration parameters/values (e.g. Event A4 threshold and/or numberoftriggeringcells). Each MR configuration parameter/value can be considered as a configuration for event Ax, and the certain height or height range can be configured by event H1/H2. Hence, the combination of event H1/H2 and event Ax can be used to associate each MR configuration and a certain height or height range. And RAN2 has support the combination of event H1/H2 and event Ax. Using combination of events H1/H2 with other events to support height-dependent configuration of MR configuration is a simple method without extra work.

Proposal 10: Height-dependent configuration of MR configuration parameters is supported using combination of events H1/H2 with other events (i.e event Ax).

Conclusions

According to the analysis given above, we have the following proposals:

The content of the measurement report

Proposal 1: As in LTE, when event H1 or event H2 triggers, UAV UE’s height is mandatorily included in the measurement report.

Proposal 2: For NR UAV, a field like heightUE in TS 36.331 can be used for height reporting to indicate the height of the UE in meters relative to the sea level.

Proposal 3: For measurement reporting triggered by event H1 and event H2, the existing location reporting mechanism can be reused. If the includeCommonLocationInfo is configured by network, UE location information, i.e. CommonLocationInfo shall be included in the measurement report.

Proposal 4: Horizontal and vertical speed can be included in the location information.

Proposal 5: For measurement reporting triggered by event H1 or event H2, measurement results of serving cells and neighbour cells can be included in the content of the measurement report based on the network configuration.

Height-depending combinations

Proposal 6: To support the combination of Event H1/H2 and Event Ax, a new IE can be introduced to indicate two combined measIds, and one measId for event H1/H2, the other measId for event Ax.

Proposal 7: At least event A3, A4 and A5 can be considered for the combination of Event H1/H2 and Event Ax.

Proposal 8: When all combined events are fulfilled, measurement reporting will be triggered. If one event becomes non-fulfilled, the measurement reporting will be stopped.

Height-depending more-than-one configurations

Proposal 9: Multiple SSB-ToMeasure for different height or height range can be configured in one MO configuration to achieve height-depending more-than-one SSB-ToMeasure. And RAN2 can discuss the maximum number of parameters/values for SSB-ToMeasure that can be configured in one MO configuration.

Proposal 10: Height-dependent configuration of MR configuration parameters is supported using combination of events H1/H2 with other events (i.e event Ax).

Reference

[1] RP-230782, Nokia, “Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)”

[2] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[3] TS 36.331 V17.3.0

[4] R2-xxxxxxx, “Report of 3GPP TSG RAN WG2 meeting #121bis”

[5] R2-2304491, “Report of [AT121bis-e][306][UAV] Measurement Reporting (Qualcomm)”