3GPP TSG-RAN WG2 Meeting #122      	                                  R2-2306136

Incheon, Korea, May 22-26, 2023

Source: 	Xiaomi

Title:  	Discussion on flight path reporting for NR UAV

Agenda Item:	7.8.3 Flight path reporting

Document for:	Discussion and Decision

Introduction

A revised WID on NR support for UAV (Uncrewed Aerial Vehicles) [1] has been approved. In the WID, one objective is to enhance measurement reports, including flight path reporting.

In previous meetings [2-6], the following agreements have been reached for flight path reporting for NR UAV:

RAN2#119-e: 3 As in LTE, flight path plan reporting will be introduced. Location list of waypoints (3D location information) and timestamp is adopted as the basic content of flight path report. FFS if timestamp is mandatory or optional for NR. FFS if further enhancements are needed RAN2#119bis-e: 1. The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). 2. Allow the flight path to be updated. FFS on the details. 3. FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting RAN2#120: 1. A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. 2. A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity 3. No requirements are placed on spatial distribution of waypoints 4. A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. 5. UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. 6. UAI message can also be used to indicate the UE has flight path availability. 7. FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS. RAN2#121: 1. The granularity of flightpath timestamp is 1s. 2. Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE RAN2#121bis-e: 1. Flightpath update indication in UAI is configurable by the network 2. Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE 3. Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307] 4. As a baseline, we can consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s) 5. As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication. FFS if further differentiation is needed if we decide to have delta signaling

In this contribution, we will provide our understandings on the flight path reporting for NR UAV.

Discussion

  - The content of flight path information

The meanings of waypoint and timestamp has reached consensus in previous meetings. However, which waypoints should be included in the flight path information is FFS. Some options have been provided in previous meetings, e.g. the flight path information follows the granularity of the waypoints (i.e. the distance or interval between consecutive waypoints) configured by network and/or provides the information on the destination of the flight. However, it is uncertain whether UE has full flight path information or not e.g. whether the flight path information stored in UE includes the destination of flight and can satisfy the granularity configured by network. And the size and speed of UAV UE may influence over the granularity of waypoint. Hence, whether the requirement of flight path information configured by network can be fulfilled or not depend on the flight path information in UE. It may be difficult to configured appropriate requirements to UE to limit the content of flight path information. And, in LTE UAV, which waypoints are included in flight path information is up to UE implementation. Therefore, no optimization for the content of flight path information is needed for NR UAV.

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

  - Flight path reporting

RAN2 has agreed that flight path availability can be reported within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message and UE can indicate the UE has a new flight path availability (whether it is initial or update).

In legacy LTE procedure, the initial flight path availability can be reported within above-mentioned RRC complete messages. However, if UE doesn’t have flight path information available upon reception of the corresponding RRC message, UE will not report the indication of flight path availability, i.e. flightPathInfoAvailable, and then if UE has flight path information available after the transmission of RRC complete message, UE will not report flightPathInfoAvailable unless UE receives a new RRC message e.g. RRCReconfiguration, RRCReestablishment, RRCResume, or RRCSetup, which will cause that network cannot obtain flight path information timely.

In RAN2#120 meeting, RAN2 has agreed that UAI message can be used to indicate the UE has flight path availability. UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication via UAI if configured.

Proposal 2：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication via UAI if configured.

UAI message also can be used for the update of flight path and whether and what triggering conditions are specified for flight update is still FFS. For flight update, when UE has a new flight path available, the UE should inform network and network can required the UE to report the new flight path information. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report the flight path update indication via UAI if configured.

Proposal 3: UAI information can be used for flight path update. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report the flight path update indication via UAI if configured.

To support flight path update, RAN2 has discuss the trigger conditions for flight path reporting(i.e. how to confirm the flight path information has been changed) in previous meetings and agreed to consider a simple network control mechanisms (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. However, some details of the network control mechanism is FFS. For example, the kind of threshold(s) for the mechanism need to be discussed, and the time/distance threshold(s) and the number of waypoints are considered in [7]:

Based on the time/location threshold(s), UE can detects the change between the available waypoint and the reported waypoint. When the change of time/distance between the currently available waypoint and the reported waypoint is larger than or equal to the time/location threshold(s), UE detects the waypoint reported in last time is unavailable. According to the configured number of waypoints, if the number of unavailable waypoints reported in last time is larger than or equal to the configured number, UE should trigger the flight path update indication.

Proposal 4: 	Network can configure one or more time/distance threshold(s). When the change of time/distance is larger than or equal to the threshold(s), UE can trigger the flight path update indication in UAI.

Proposal 5: If the number of the last reported waypoints which is not included in the currently available flight path information is larger than or equal to the configured number of waypoints, UE can trigger the flight path update indication in UAI.

Based on proposal 4 and 5, the time/location threshold(s) and the configured number of waypoints can be configured together, if UE deviates from the waypoints reported in last time by more than the time or location threshold(s) and the number of deviate waypoints is larger than or equal to the configured number of waypoints, UE can trigger the flight path update indication.

To reduce signaling overhead, delta flight path information can be supported for flight path update and the new flight path information is the delta of the flight path information reported in last time. The timestamp of waypoints can be used as the index. If timestamps is not included in the previous report. Full flight path information can be reported.

Proposal 6: Delta flight path information can be supported for flight update. The timestamp of waypoints can be used as the index for delta configuration.

Conclusions

According to the analysis given above, we have the following proposals:

The content of flight path information

Proposal 1: It is up to UE implementation which waypoints are included in flight path information.

Flight path reporting

Proposal 2：UAI message can be used to report the indication of initial flight path availability. If UE has flight path information available and the UE has not reported the indication in the latest complete message, the UE should report the initial flight path available indication via UAI if configured.

Proposal 3: UAI information can be used for flight path update. If UE has flight path information available and the flight path information in the UE is changed compared to last time the UE reported this information, the UE should report the flight path update indication via UAI if configured.

Proposal 4: 	Network can configure one or more time/distance threshold(s). When the change of time/distance is larger than or equal to the threshold(s), UE can trigger the flight path update indication in UAI.

Proposal 5: If the number of the last reported waypoints which is not included in the currently available flight path information is larger than or equal to the configured number of waypoints, UE can trigger the flight path update indication in UAI.

Proposal 6: Delta flight path information can be supported for flight update. The timestamp of waypoints can be used as the index for delta configuration.

Reference

[1] RP-230782, Nokia, “Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)”

[2] R2-2209301, “Report of 3GPP TSG RAN WG2 meeting #119-e”

[3] R2-2211101, “Report of 3GPP TSG RAN WG2 meeting #119bis-e”

[4] R2-2300002, “Report of 3GPP TSG RAN WG2 meeting #120”

[5] R2-2302401, “Report of 3GPP TSG RAN WG2 meeting #121”

[6] R2-xxxxxx, “Report of 3GPP TSG RAN WG2 meeting #121bis-e”

[7] R2-2302867, “Report from [Post121][314][UAV] Flight path reporting”