3GPP TSG-RAN WG2 Meeting #122	R2-2306137

Incheon, Korea, May 22nd-26th, 2023

Agenda item:	7.5.4.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on PDU set discarding for XR traffic

Document for:	Discussion and Decision

Introduction

XR WID has approved to specify the discard operation of PDU Sets for DL and UL. For UL, RAN2#121 bis [1] has agreed that the PDU set discarding contains the timer-based discarding at PDCP.

PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

In addition to that, according to the latest running CR [2], the transmitter (UE in UL and RAN in DL) can discard the remaining PDUs of a PDU set if one PDU of that PDU set is known to be lost.

When the PSIHI is set for a QoS flow, as soon as one PDU of a PDU set is known to be lost, the remaining PDUs of that PDU Set can be considered as no longer needed by the application and may be subject to discard operation at the transmitter to free up radio resources.

Moreover, in case of congestion, UE can perform the PDU set discarding based on PSI.

In case of congestion, the PSI may be used for PDU set discarding and in uplink, a PDU set discard mechanism taking the PSI into account will be introduced.

In summary, there are 3 kinds of PDU set discarding:

- Timer-based;

- PDU loss triggered; and

- PSI based in case of congestion.

In the remainder of this document, we will discuss these three kinds of PDU set discarding one by one.

Discussion

SA2 defined a new QoS parameter “PDU Set Integrated Handling Indication” (PSIHI) to indicate if all PDUs of the PDU set are needed for the application layer to be able to use it, see the following text quoted from TS 23.501 [3].

The PDU Set Integrated Handling Indication (PSIHI) indicates that whether all PDUs are needed for the usage of the PDU Set by the application layer in the receiver side.

If the PDU Set Integrated Handling Indication indicates that not all PDUs are needed, then even if one or more PDUs have been lost, the remaining PDUs of the same PDU set still need to be transmitted. Hence, PDU set level discarding should be configurable by the network based on the received PSIHI information. The network would normally configure it only for the DRBs containing QoS flows configured with the “PDU Set Integrated Handling Indication”, and the UE will apply PDU set level discarding only to such DRBs while the legacy PDU discarding will be applied otherwise.

- PDU set level discarding should be configurable per DRB.

- Another issue that was raised in the past is whether any enhancements to layers below PDCP are needed for XR PDU set discarding. If an XR RLC PDU/segment has already been delivered to MAC layer, since the XR RLC PDU/segment may be multiplexed into a TB together with other non-XR data, the MAC layer should not discard the TB even if the associated XR PDCP SDU/PDU has been discarded. For RLC layer, today the RLC entity discards the indicated RLC SDU only if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. In fact, for RLC UM, nothing would really prevent those segments which have not been submitted to the lower layers to be discarded at RLC layer. However, there is no such mechanism currently because the gain is limited while keeping UM and AM mode operation aligned simplifies the UE implementation. For this reason, we prefer to keep the existing principle.

- No additional triggers are introduced for PDU/SDU discarding at RLC and MAC layers due to PDCP PDU set discarding mechanism.

- 2.1 Timer-based PDU set discarding

A PDU set will not be valid forever and its validity is related to PSDB. Moreover, as RAN2#121 bis has agreed [1], the timer based PDU set discarding should reuse the existing PDCP discard timer.

PDU set discard is modelled using the existing PDCP discard timer for the uplink. The timer is in network control.

Then there are 2 possible solutions:

Option 1: The length of PDCP discard timer varies for different PDUs.

For example, suppose the PSDB is 10 ms, the 1st PDU arrives at time 0 and the associated PDCP discard timer is set to be 10 ms, 3ms later the 2nd PDU arrives, then its associated PDCP discard timer will be 7ms (i.e., the current value of the previous PDU’s PDCP discard timer), another 2ms later the 3rd PDU arrives and its associated PDCP discard timer is set to be 5ms, and so on. By doing so, all of the PDCP discard timers associated to the PDUs of the same PDU set will expire at the same time.

Option 2: Expiry of the first PDCP discard timer triggers the discard of the whole PDU set.

More specifically, once the PDCP discard timer of the 1st PDU expires, all the PDUs within the same PDU set shall be discarded. In other words, once the 1st PDU’s discard timer expires, the discard timers of the other PDUs belonging to the same PDU set are regarded as expired.

These 2 options can achieve the same goal, but option 2 reuses the current PDCP procedure to the largest extent and is much simpler from both specification and implementation point of view.

- The expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

- 2.2 PDU loss triggered PDU set discarding

In addition to the timer based PDU set discarding discussed above, PDU loss can also trigger the PDU set discarding, see the following text quoted from the running CR [2].

When the PSIHI is set for a QoS flow, as soon as one PDU of a PDU set is known to be lost, the remaining PDUs of that PDU Set can be considered as no longer needed by the application and may be subject to discard operation at the transmitter to free up radio resources.

From the description, it can be seen that as soon as the sender knows a PDU set has no chance of being decoded because a PDU was lost already, the sender can discard the remaining PDUs of the PDU set to save the radio resources. The question that needs to be answered is how the sender can know a PDU has been lost. Considering RAN2 has agreed that the PDU set discarding should be performed at the PDCP layer, the PDCP status report can be used with some enhancements to inform the transmitter that one or more PDUs of the PDU set have been lost. Currently, the receiving PDCP entity only triggers the PDCP status report in some rare scenarios like the uplink data switching, PDCP entity re-establishment or data recovery (AM DRB). Therefore, a new trigger for PDCP status report is needed for the receiver to be able to inform the transmitter about the PDU loss. At the PDCP layer, a PDCP SDU can be regarded as lost if it is not successfully received when the t-Reordering expires and this event can be used as a PDCP SR trigger.

- The gNB can configure the UE to trigger a PDCP status report when the t-Reordering expires.

- Based on such status report the gNB may discard the remaining PDUs of the PDU set. Similarly, for UL direction, the gNB should be able to configure the UE to discard the remaining PDUs of the PDU set when one of the PDUs is lost based on the PDCP SR sent from the gNB to the UE.

- If PDU set level discarding is configured for the DRB, the transmitting PDCP entity at the UE shall discard the remaining PDUs of a PDU set if one PDU of that PDU set is known to be lost according to the PDCP status report received from the gNB.

- 2.3 PSI based PDU set discarding in case of congestion

The baseline idea behind the PSI based PDU set discarding is not complicated. When the congestion happens, the PDU set with lower importance can be discarded while the PDU set with higher importance still get transmitted. For DL, this is very straightforward as the gNB may just decide to do this any time where the load becomes too high. The more complicated part is that UEs are unable to detect the congestion by themselves. Each UE can only know its own buffer status and data rate information, but that does not represent network load status properly, since there may be a lot of UEs connected to the network at the same time.

- UE is unable to detect the network congestion by itself, hence the PSI based PDU set discarding can only be controlled by gNB.

There are at least the following two options:

Option 1: The gNB temporarily adjusts the PDCP dicardTimer length of low PSI PDU sets, for example set the PDCP discardTimer to 0. In consequence, low PSI PDU sets will be discarded when the congestion happens.

Option 2: The gNB indicates the UE to discard the PDU sets with low PSI.

When the value of PDCP discard timer is set to 0 for the first option, the outcome of both these solutions is the same, but the first option allows to reuse the existing specifications and procedures to a large extent. On the other hand, option 2 introduces a new indictor and UE behaviour to achieve the same thing. Furthermore, in the case of option 1, gNB has more flexibility as it does not have to set the discard timer to 0, but can use another value, shorter than the previous one. This way, it is possible to have at least part of low PSI packets to get transmitted, if there is a scheduling occasion that can be used for this. Hence, option 1 is preferred due to its simplicity and flexibility.

- Introduce PSI-specific PDCP discard timer with additional PDCP discard timer values, including (but not limited to) 0ms.

- Another issue that needs to be discussed is whether the reconfiguration of the discard timers should be done using RRC protocol or rather in lower layers, e.g. using a MAC CE. The first option allows to reuse the current reconfiguration mechanism and has least impacts to specifications. On the other hand, it may be impossible for the gNB to timely adjust the setting of the discard timer in line with the dynamically changing congestion situation.

- RAN2 should discuss which protocol layer should be responsible for adjusting the PSI-specific PDCP discard timers during congestion, e.g. RRC and/or MAC.

Conclusion

In this contribution, we have discussed the PDU set discarding for XR traffic and made the following observations and proposals:

- UE is unable to detect the network congestion by itself, hence the PSI based PDU set discarding can only be controlled by gNB.

- PDU set level discarding should be configurable per DRB.

- No additional triggers are introduced for PDU/SDU discarding at RLC and MAC layers due to PDCP PDU set discarding mechanism.

- The expiry of the 1st PDU’s discard timer triggers discard of the whole PDU set.

- The gNB can configure the UE to trigger a PDCP status report when the t-Reordering expires.

- If PDU set level discarding is configured for the DRB, the transmitting PDCP entity at the UE shall discard the remaining PDUs of a PDU set if one PDU of that PDU set is known to be lost according to the PDCP status report received from the gNB.

- Introduce PSI-specific PDCP discard timer with additional PDCP discard timer values, including (but not limited to) 0ms.

- RAN2 should discuss which protocol layer should be responsible for adjusting the PSI-specific PDCP discard timers during congestion, e.g. RRC and/or MAC.

Reference

- Chairs note of RAN2#121 bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_121bis-e/Inbox/Chairs_Notes/RAN2-121bis-e%20LTE%20MUSIM%20QoE%20XR%20(Tero)_2023-04-26-EOM.docx

- R2-2304393, Stage 2 Overview of XR Enhancements, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_121bis-e/Docs/R2-2304393.zip.

- 3GPP TS 23.501 V18.1.0, System architecture for the 5G System (5GS).