3GPP TSG-RAN2#122	R2-2306142

Incheon, 22th May – 26th May, 2023

Agenda item:		7.18.3 (NR_MT_SDT-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on MT-SDT procedure

Document for:	Discussion and Decision

1.	Introduction

In RAN2#121, RAN2 made agreements for MT-SDT as follows:

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

In this contribution, we present our view on the details of MT-SDT based on the above agreements.

2.	Discussion

In RAN2#121, RAN2 made some agreement for MT-SDT. According to the agreements and the current SDT procedure, Rel-18 MT-SDT procedure can be depicted as follows. Based on Figure 1, we present our understanding on MT-SDT procedure and have further discussion for each step of MT-SDT procedure.

Figure 1. Overall procedure of MT-SDT

In Step 1, the network provides the SDT configuration via SIB or RRCRelease message as in Rel-17 SDT. Regarding signalling for SDT configuration, we think there may be no objection that this will be same as Rel-18 MT-SDT configuration.

In Step 2, if data belonging to RBs configured for MT-SDT is available in the network side, the network triggers MT-SDT procedure. For the configuration for SDT RBs, RAN2 agreed that RBs configured for SDT are common for MO-SDT and MT-SDT in RAN2#121.

In Step 3, upon triggering MT-SDT procedure in the network side, the network sends the paging message to a UE. Regarding the paging message, RAN2 agreed to include one-bit indication for MT-SDT indication in a paging message for MT-SDT.

In Step 4, upon the reception of the paging message, the UE checks MT-SDT criteria. For checking MT-SDT criteria, RAN2 made agreements as follows.

Agreements in RAN2#120

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

Agreements in RAN2#121

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

- If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

Based on the agreements, MT-SDT criteria would be as follows.

- If SDT RSRP threshold is met,

  - If SUL threshold is met, SUL is chosen

  - Otherwise, NUL is chosen

  - If CG-SDT criteria is met, CG-SDT resource is chosen.

  - Else if RA-SDT criteria is met, RA-SDT resource is chosen.

  - Otherwise, non-SDT RA resource is chosen.

Meanwhile, there was a concern that if the CG periodicity is long, the paging response can be delayed and there was an argument that CG periodicity needs to be considered for CG-SDT criteria. Whether to optimize for case when CG periodicity is too long is remained as FFS.

Currently, CG-SDT validity is determined only considering RSRP change threshold and CG-SDT-TAT, and the time criterion between CG resource and RA resource is not considered.

Considering that dense configuration of CG resource brings waste of reserved radio resource, the network may configure CG resource with long periodicity and there may be a case where the next CG resource is farther than RA resource. In this case, if CG-SDT is selected, the paging response may be delayed and this may lead to a significant latency on DL data reception.

Therefore, we think additional criterion is needed, taking into account the remaining time to the RA resource or CG resource from initiating MT-SDT procedure, i.e., time based criterion. However, if the only remaining time is considered for time based criterion, the UE does not prioritize CG-SDT but always selects the earliest resource. Thus, RAN2 needs to further discuss time based criterion.

Proposal 1. Time based criterion is considered for the CG-SDT validity check.

In Step 5, if MT-SDT criteria is satisfied in Step 4, the UE transmits RRCResumeRequest message in response to the paging message and RRCResume cause set to new RRCResume cause, i.e. mt-SDT.

However, when MT-SDT criteria is not satisfied, how to set RRCResume cause is remained as FFS. Although MT-SDT criteria is not satisfied, the network has DL data and the UE needs to receive the DL data. Thus, we think RRCResume casue can be set to mt-Access.

Proposal 2. When the paging message for MT-SDT is received but MT-SDT criteria is not satisfied, RRCResume cause is set to mt-Access.

Before transmitting RRCResumeRequest message, the UE receives the paging message, decodes it, initiates SDT procedure, and selects UL resource and generates the uplink response. Depending on UE processing capability, the UE may transmit the uplink response a long time after receiving the paging message.

We think there are two problems due to the long time.

First problem is that the uplink response may not be transmitted within the network’s paging response monitoring window. Processing time is UE’s internal behaviour, so it is difficult to predict time between the paging message and the uplink response, in the network side. If the uplink response is transmitted after the network’s paging response monitoring window, the network may not receive the uplink response. Then, the network has to transmit the paging message to larger area, or, in the extreme case, the network cannot receive the uplink response at all. Then, MT-SDT data transmission would fail.

Figure 3. Example of failure of uplink response

Thus, it would be beneficial to transmit a positive HARQ feedback to the network upon receiving the paging message. After transmitting the positive HARQ feedback, the UE initiates SDT procedure to transmit the uplink response, selects UL resource and generate the uplink response message, and after receiving the positive HARQ feedback, the network waits the uplink response message.

Proposal 3. UE transmits a positive HARQ feedback to the network upon receiving the paging message for MT-SDT.

Second problem is that CG-SDT-TAT may be expired unexpectedly. If CG-SDT-TAT is expired while SDT procedure is ongoing, the UE considers that the ongoing CG-SDT procedure is terminated, and goes to RRC_IDLE. Once the UE moves to RRC_IDLE, the UE cannot use SDT procedure anymore until the UE goes to RRC_CONNECTED. Thus, CG-SDT-TAT should be managed during MT-SDT procedure.

Currently, TAT or CG-SDT-TAT is restarted upon receiving DL signalling, i.e., TAC MAC CE, RAR or RRCRelease, so it would be desirable to restart CG-SDT-TAT based on DL signalling in MT-SDT procedure as well, and the DL signalling can be the paging message. Upon receiving the paging message, the UE restarts CG-SDT-TAT and considers CG-SDT resource ass valid if CG-SDT-TAT is running.

Proposal 4. CG-SDT-TAT is restarted upon receiving the paging message for MT-SDT.

In Step 6, if RRCResumeRequest is successfully transmitted, the network transmits MT-SDT data to the UE. If there is MO-SDT data, the UE transmits it using CG-SDT or dynamic UL grant. If there is no UL grant, the UE initiates RACH procedure using non-SDT RACH resource as agreed in RAN2#121.

In Step 7, when the network determines there is no data to be transmitted in both the UE and the network, RRCRelease is transmitted for terminating SDT procedure.

3.	Conclusion

In this document, we present our understanding on overall MT-SDT procedure, and make proposals as follows.

Proposal 1. Time based criterion is considered for the CG-SDT validity check.

Proposal 2. When the paging message for MT-SDT is received but MT-SDT criteria is not satisfied, RRCResume cause is set to mt-Access.

Proposal 3. UE transmits a positive HARQ feedback to the network upon receiving the paging message for MT-SDT.

Proposal 4. CG-SDT-TAT is restarted upon receiving the paging message for MT-SDT.

4.	Reference

- RP-213583	New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR	ZTE Corporation