3GPP TSG RAN WG2 #122	R2-2306165

Incheon, Korea, May 22-26, 2023

# Title:	DRAFT LS reply on FR2 unkonwn SCell activation enhancement

# Response to:	R2-2304636 (R4-2306321)

# Release:	Release 18

Work Item:	     NR_RRM_enh3

Source:	Apple (to be RAN2)

To:	RAN4

Cc:	RAN1

Contact Person:

Name:	Fangli XU

E-mail Address:	fangli_xu@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 would like to thank RAN4 for the LS on on FR2 unkown SCell activation enhancement. RAN2 would like to provide answers to the questions as follows.

Question 1: How to configure the report of L3 measurement result for unknown FR2 SCell activation enhancement

- Technical enhancement goal:

  - network configures UE to report L3 measurement result when triggering conditions in (2) are met.

  - It was RAN4 understanding that a one-time report may be configured. The type of report configuration may be RAN2 decision.

- Action request to RAN2:

  - RAN4 requests RAN2 to design corresponding signalling for the technical enhancement goal above

[RAN2 Answer to Q1]

- The fast SCell measurement report is configured for a newly added SCell in RRCReconfiguration message.

- The fast SCell measurement report is configured via the measurement configuration including the SCell’s servingMO and a new report event config.

Question 2: How to trigger the report of L3 measurement result for unknown FR2 SCell activation enhancement

- Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting,

- Agreements (GTW, Monday Apr 17, 2023) - The L3 measurement report for unknown FR2 Scell activation after Scell activation command is triggered by Scell activation command. - FFS whether to have additional events for report triggering

- Action request to RAN2:

  - RAN4 requests RAN2 to decide:

    - How the measurement report gets triggered by SCell activation command.

    - RAN4 request RAN2 to design the corresponding signaling and decide whether a new event is needed to achieve above technical enhancement goal.

[RAN2 Answer to Q2]

- RAN2 understands the fast SCell measurement report triggering procedure is as follows:

  - Step 1: When UE MAC receives SCell activation command MAC CE, if the activated SCell is configured with the fast SCell measurement report, UE MAC will inform UE RRC;

  - Step 2: Based on UE MAC indication, UE RRC evaluates whether the trigger condition for fast SCell measurement report is met; RAN2 assumes the trigger condition will be discussed in RAN4;

  - Step 3: If the trigger condition for fast SCell measurement report is met, UE RRC initiates the L3 measurement report transmission procedure.

Question 3: Contents of the report of L3 measurement result for unknown FR2 SCell activation enhancement and how to send the report

- Technical enhancement goal: RAN4 had following agreements in RAN4 #106-bis-e meeting,

- Agreements (GTW, Monday Apr 17, 2023) - UE needs to report the L3 measurement result after SCell activation command - Agreement (in 2nd round issue 1-1-5 of R4-2306315) - UE reports L3 measurement result with SSB index for unknown FR2 SCell activation enhancement

- Action request to RAN2:

  - RAN4 requests RAN2 to design corresponding signaling for “report of L3 measurement result” after SCell activation command for unknown FR2 SCell activation enhancement.

  - It’s up to RAN2’s decision on which layer’s signaling shall be used to send the report to NW

[RAN2 Answer to Q3]

- Legacy RRC measurement report message is used for the fast SCell measurement report;

- In RRC mearuement report message, it includes the SCell cell level quality and the best beam list, and for each beam info, it should include the SSB index and optional the SSB specific quality.

2. Actions:

To RAN4 group

ACTION:	RAN2 respectfully asks RAN4 to take the above information into account for their future works.

3. Dates of next RAN2 meetings:

TSG-RAN WG2#123                      2023-08-21 to 2023-08-25		Toulouse, FR

TSG-RAN WG2#124                      2023-10-09 to 2023-10-13		Xiamen, CN