3GPP TSG-RAN WG2 Meeting #122                                                                        R2-2306175

Incheon, Korea, May 22-26, 2023

Source:	CATT

Title:	Discussion on non-simultaneous UL and DL from different two bands during UL CA

Agenda Item:	7.25.1

Document for:	Discussion and Decision

- 1. Introduction

An RAN4 LS [1] about the non-simultaneous UL and DL from different two bands during UL CA was sent to RAN2, this contribution will discuss the question raised in the LS.

- 2. Discussions

In RAN4 LS [1], RAN2 is asked “Whether or not it is feasible for dual UL CA of CA_n5-n8 to take a state of (UL, DL) = (n5+n8, n8) with the existing RAN2 specifications”.

From RAN2 signaling point of view, when SCell addition is configured, the DL configuration is mandatory:

ServingCellConfigCommon ::= SEQUENCE { physCellId PhysCellId OPTIONAL, -- Cond HOAndServCellAdd, downlinkConfigCommon DownlinkConfigCommon OPTIONAL, -- Cond HOAndServCellAdd uplinkConfigCommon UplinkConfigCommon OPTIONAL, -- Need M … //omit } HOAndServCellAdd; This field is mandatory present upon SpCell change (including path switch between a serving cell and a L2 U2N Relay UE) and upon serving cell (PSCell/SCell) addition. Otherwise, the field is absent..

However, the configuration of the each channel is optional present. Furthermore, whether shceduling on each DL/UL is up to NW implementation. So for the case of CA_n5-n8, the NW can config CA of 2DL+2UL to UE, and schedule data on 2DL+1UL or 1DL+2UL at different time as shown in the RAN4 LS[1]. So we propose:

Proposal 1: It is feasible for dual UL CA of CA_n5-n8 to take a state of (UL, DL) = (n5+n8, n8) with the existing RAN2 specifications.

In addition, A new UE capability of supporting non-simultaneous n5 DL and n8 UL operation for UL CA of CA_n5-n8 is needed to be introduced. So the NW can make decision on the configuration and scheduling for the case of CA_n5-n8.

Proposal 2: A new UE capability is needed for indicating support of non-simultaneous n5 DL and n8 UL operation for UL CA of CA_n5-n8.

Based on the above analysis, we propose to give the answer to RAN4 as following:

From RAN2 point of view, it is feasible for dual UL CA of CA_n5-n8 to take a state of (UL, DL) = (n5+n8, n8) with the existing RAN2 specifications, and a new UE capability is needed for indicating support of non-simultaneous n5 DL and n8 UL operation for UL CA of CA_n5-n8, so the network can make decision on the configuration and scheduling for the case of CA_n5-n8.

- 3. Conclusion

This contribution gives some initial considerations on the non-simultaneous UL and DL from different two bands, and proposes:

Proposal 1: It is feasible for dual UL CA of CA_n5-n8 to take a state of (UL, DL) = (n5+n8, n8) with the existing RAN2 specifications.

Proposal 2: A new UE capability is needed for indicating support of non-simultaneous n5 DL and n8 UL operation for UL CA of CA_n5-n8.

- 4. Reference

[1] R4-2304166 LS on non-simultaneous UL and DL from different two bands during UL CA