3GPP TSG-RAN WG2 Meeting #122	R2-2306176

Incheon, Korea, 22nd - 26th May, 2023

Agenda Item:	7.5.4.1

Source: 	MediaTek Inc.

Title:  	BSR enhancements for XR

Document for:	Discussion, decision

- Introduction

In last RAN2 meeting (RAN2 #121bis-e), we have some agreements related to BSR enhancement with FFS points as below:

- Support of new BSR table(s) is based on NW configuration and UE capability. FFS whether the UE capability can apply to non-XR UEs.

- As a working assumption, at most one BS index or BS value is reported by an LCG. This assumption can be revisited if new BSR table design cannot achieve a target level of quantization error. FFS what this target level should be.

- Design/configuration for new BSR table(s) should include support for narrower ranges (i.e. finer granularity) than the legacy. Details can be discussed after an agreement on how UE obtains new BSR table(s) (e.g. pre-definition vs RRC configuration) is made.

- At least linear distribution is used for generating code points in new BSR table(s). FFS whether exponential distribution can be considered too. FFS if piecewise linear distribution is supported.

- New BSR table(s) can be used by any UEs that support such a capability. However, design of the new BSR table(s) should be based on XR-specific use cases and requirements.

- Network can configure which BSR table(s) an LCG is eligible to use. UE determines which BSR table (i.e. legacy or something else) the LCG should use. FFS details of this determination (e.g. based on buffer size) and how network knows which BSR table each LCG uses.

- As working assumption (depending on how we create the new BSR table(s) and the MAC CE format), If more than one new BSR table are introduced, all of them have the same size BS field. FFS on the exact size.

- Deprioritize Option 2c (static + dynamic BSR tables) and Option 2d (reference table + scaling factor).

- Have more discussions on Option 2a (static BSR tables) vs Option 2b (RRC configured BSR tables). In next meeting, companies should explain how BSR table(s) are created and how many tables would be needed, and how the MAC CE structure will look like. Should also explain what is the expected quantization error.

This paper will continue discuss FFS points from last meeting and show our view about XR capacity enhancement from BSR perspective.

# 2	Discussion

Three enhanced directions for BSR are identified during study item phase, 1) Enhanced BSR table specific for XR packet, 2) Enhanced BSR format with delay information 3) New trigger condition for BSR associated to XR packet. We will discuss them one by one.

## 2.1	Enhanced BSR Table

We first provide our understanding about XR traffic model from TR 38.838 [2], then based on the XR traffic model, we share our views about enhanced BSR table in this section.

## 2.1.1	XR Traffic model and profile

In Rel-17 Study Item “Study on XR Evaluations for NR”, many of the XR and CG use cases are studied and characterised by quasi-periodic traffic (with possible jitter) with high data rate in DL (i.e., video stream) combined with the frequent UL (i.e., pose/control update) and/or UL video stream. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB), the results are captured in TR 38.838 [2] and a list of statistical parameters for generalized XR traffic model is shown in Table 1.

Table 1: Statistical parameters for packet size following truncated Gaussian distribution

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M |
| Max | byte | 150% of M | 109% of M |
| Min | byte | 50% of M | 91% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |

Observation 1: In XR Traffic model, two factors 1) data rate (R) and 2) frames per second (FPS) determines the XR traffic characteristic, including mean, maximum and minimum packet sizes.

Among these two factors, FPS is much important than data rate. In order to maintain user experience under congest situation, XR applicaition usually use bitrate adaptation mechanism, i.e., keep the same FPS but reduce the data rate, to address this issue. That means when bitrate adaptation mechanism is enabled, the mean, maximum and minimum packet size will be affected as well.

Observation 2: XR bitrate adaptation mechanism leads to different mean, maximum and minimum packet sizes.

## 2.1.2	BSR Table

Current solutions on the table, option 2a (static BSR tables) and option 2b (RRC configured BSR tables) both can achieve narrower ranges (i.e., finer granularity) than legacy if designed properly. However, regarding target for quantisation error. We think any error is bad, since NW might overallocates accordingly. Our goal is to achieve minimum error rate for any cases under the same size BS field agreed in last meeting. We analyze option 2a (static BSR tables) and option 2b (RRC configured BSR tables) as below:

- For option 2a (static BSR tables), the spec impact is to add new fixed BSR table(s) in the MAC spec, but the problem is its scalability, if there are new types of XR services with different or higher rates or the XR services enable bitrate adaptation mechanism, then, we need to add other new fixed BSR(s).

- For option 2b (RRC configured BSR tables), the spec impact will be new BSR formula in the MAC spec, and new RRC IEs to configure related parameters (e.g., minimum buffer size and step size of configured BSR tables), it can perfectly handle new types of XR services and XR bitrate adaptation mechanism. The RRC configured BSR table should avoid quantisation errors of any form, i.e., RRC configured BSR tables can be as accurate as needed. Some additional advantages such as only a single table generation mechanism needs to be defined for UE and NW implementation can take XR traffic evolution into account (no future spec updates required), etc.

Observation 3: The RRC configured BSR table can avoid quantisation errors of any form and can achieve better target level of quantization error than static BSR tables.

Some arguments that fixed BSR table has low implementation complexity might not be correct, the implementation complexity is highly depends on how UE implement it. We already had agreement that linear distribution is used for generating code points in new BSR table(s), the complexity can be kept even lower than static BSR table(s). Both options are workable, and as a UE vendor, we don't have an issue with option 2b (RRC configured BSR tables).

Proposal 1: RAN2 to adopt RRC configured BSR tables mechanism for new BSR table(s)

|  | Option 2a (static BSR tables) | Option 2b (RRC configured BSR table) |
| --- | --- | --- |
| Spec impact | Most in MAC spec, e.g., - New fixed BSR table - New MAC CE. | MAC and RRC spec, e.g., - RRC parameters to configure new BSR table - New formula to construct BSR table. |
| Quantization error | Poor and fixed when static BSR table(s) are harded in the MAC spec. | Better, with configurable RRC parameters to construct suitable BSR table to minimize quantization error. |
| Scalability and future proof | Poor | Better |
| Complexity | Low | Low (for linear distribution) |
| Overall procedures | Same as legacy BSR. | 1. NW uses RRC message to configure UE to construct new BSR table. 2. UE constructs new BSR table and use the new BSR table for reporting. |

The new BSR table can be configured by gNB for XR-specfic traffic, which belongs to certain logical channel group (LCG). In order to differentiate the new BSR table and legacy BSR table, the configuration of minimum packet size could be optional. If it is configured, that means new BSR table is used. If not, that means legacy BSR table is used.

Proposal 2: gNB to control which BSR table is used by UE via presence of RRC BSR IE(s).

Proposal 3: BSR MAC CE format can be the same as legacy BSR MAC CE.

Some details about the configurable RRC parameters are analyzed here. The granularity of new RRC parameters can be same as reporting unit of BSR, i.e., per LCG. The new formula to generate new BSR table(s) can be as follow:

Bk = Bmin + (k-1) * (step size),

Two potential approaches to generate a linear table:

- Approach 1:

  - NW configures Bmin and Bmax

  - BSR entry k corresponds to: Bmin + (k-1) * (Bmax - Bmin)/N, where N = table size

- Approach 2:

  - NW configures Bmin and step-size

  - BSR entry k corresponds to: Bmin + (k-1) * step size

Approach 1 includes a division which is computationally expensive, it also result in quantisation errors depending on whether fixed point or floating point division is used. Approach 2 does not have the same issues as Approach 1, while providing the same BSR table result. Therefore, we prefer approach 2.

Proposal 4: BSR entry is generated using the formula Bmin + (k - 1) * step-size, where Bmin and step size and configured by the NW.

- If Bmin and step size are not configured, use legacy BSR table.

- If configured, use new linear BSR table.

## 2.2	Delay reporting

The packet delay budget is associated with buffer data, however, the buffer data might have different remaining packet delay budget, we suggest to choose the minimum value as the reported information. The reported information is different among different LCGs. We also suggest that the reported information should associated with BSR. The meaning of each reporting index could follow similar mechanism as BSR, i.e., the delay information could be a range with exponential step size. As we approach the PDB expiry, network needs to know more accurately how much delay is left. If PDB value range is still large, the range could be less precise (larger set of values). One example is shown in Table 2.

Table 2: PDB table

| PDB Index | PDB value (ms) |
| --- | --- |
| 0 | 0 |
| 1 | ≤ 1 |
| 2 | ≤ 2 |
| … | … |
| 8 | ≤ 80 |
| … | … |
| 2N-2 | > … |
| 2N-1 | Reserved |

Proposal 5: UE chooses minimum delay value of buffered data as reported information.

Proposal 6: UE reports delay information with BSR.

Proposal 7: The reported delay information could be a range.

## 2.2.3	BSR trigger condition

A BSR can be triggered by different events, it can be triggered when new higher priority data and expiry of retxBSR-Timer, which is called “Regular BSR”. It can also be triggered by expiry of periodicBSR-Timer, which is called Periodic BSR. The last one is called “Padding BSR”, which can be piggyback when UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader.

When XR packet comes to buffer, we assume it has higher priority than other traffic, then a “Regular BSR” can be be triggered. However, as discussed in 2.2.1, for XR packet with tight latency requirement, it might not have too much chance to send another BSR. To increase the chance to send BSR, new trigger condtion for BSR is needed. We assume the XR packet will be segmented into different TBs for transmission. The new trigger condition could allow that for each TB transmission with XR packet, BSR is always included. The concept is more like padding BSR, which is piggyback at each TB transmission with XR packet.

Proposal 8: RAN2 to study new trigger condition for BSR for XR services.

# 3	Conclusion

We have the following observations and proposals:

Observation 1: In XR Traffic model, two factors 1) data rate (R) and 2) frames per second (FPS) determines the XR traffic characteristic, including mean, maximum and minimum packet sizes.

Observation 2: XR bitrate adaptation mechanism leads to different mean, maximum and minimum packet sizes.

Observation 3: The RRC configured BSR table can avoid quantisation errors of any form and can achieve better target level of quantization error than static BSR tables.

Proposal 1: RAN2 to adopt RRC configured BSR tables mechanism for new BSR table(s)

Proposal 2: gNB to control which BSR table is used by UE via presence of RRC BSR IE(s).

Proposal 3: BSR MAC CE format can be the same as legacy BSR MAC CE.

Proposal 4: BSR entry is generated using the formula Bmin + (k - 1) * step-size, where Bmin and step size and configured by the NW.

- If Bmin and step size are not configured, use legacy BSR table.

- If configured, use new linear BSR table.

Proposal 5: UE chooses minimum delay value of buffered data as reported information.

Proposal 6: UE reports delay information with BSR.

Proposal 7: The reported delay information could be a range.

Proposal 8: RAN2 to study new trigger condition for BSR for XR services.

# 4	References

[1]	RP-230786, Updated WID on XR Enhancements for NR, Nokia, Qualcomm.

[2]	TR 38.838