3GPP TSG-RAN WG2 Meeting #122		R2-2306181

Incheon, Korea, 22nd – 26th May 2023

Source:	vivo

Title:	On MAC CE for Joint TCI State Indication

Agenda Item:	7.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1#112bis [1], the agreements regarding BH link beam indication were updated as the following：

For the backhaul link beam indication · The TCI state for the DL beam indication of backhaul link is selected from the TCI state list in active DL BWP of C-link. · The TCI state or SRI for the UL beam indication of backhaul link is selected from the UL TCI state list or SRI list in active UL BWP of C-link. o NOTE: For the case of joint TCI state, how to indicate/determine TCI state for the UL beam of backhaul link is subject to RAN2. · The SRI for the UL beam indication of backhaul link is associated with active UL BWP of C-link.

According the agreements, the MAC CE for joint TCI state indication is up to RAN2. In the previous RAN2 meetings, the MAC CEs respectively for DL and UL TCI state/SRI indication was already agreed in RAN2. This paper discusses whether a new MAC CE for joint TCI state indication should be introduced for BH link beam indication.

# Discussion

In the endorsed MAC CE in RAN2-121bise, the MAC CEs for separate DL and UL beam indication were defined as the following [2]:

Figure 6.1.3.y-1: NCR Downlink Backhaul Beam Indication MAC CE

Figure 6.1.3.z-1: NCR Uplink Backhaul Beam Indication MAC CE

As a reference, in Rel-17, the defined Unified TCI States Activation/Deactivation MAC CE can indicate up to 8 candidate TCI states per cell. When more than 1 candidate TCI state is configured via Unified TCI States Activation/Deactivation MAC CE, an DCI is transmitted to UE to indicate which of the candidate TCI states to be applied for the UE. In this format, Pi can indicate if the i-th code point has multiple TCI states or single TCI states. If Pi is set to 1, it means the i-th codeword includes the DL TCI state and the UL TCI state; if Pi is set to 0, it means the i-th codepoint includes only the DL/Joint TCI state or the UL TCI state. D/U bit in each octet indicates the following TCI state is joint/DL TCI state or UL TCI state. When single candidate TCI state is indicated in the Unified TCI States Activation/Deactivation MAC CE, the candidate TCI state is to be applied without subsequent DCI indication.

Figure 6.1.3.47-1: Unified TCI state activation/deactivation MAC CE

As there is already no time in RAN1 and only limited time in RAN2, we can just consider single candidate TCI state indication for joint TCI state indication. Meanwhile, considering that there are already separate MAC CEs for DL and UL beam indication, there is no need to also support multiple TCI states indication in the same MAC CE for joint TCI state indication, if the new MAC CE for joint TCI state indication is introduced.  Furthermore, according the current assumption, there is only one serving cell for the control link and the active UL BWP and active DL BWP for the NCR-MT are already aware between the NCR-MT and the gNB, there is no need to indicate the serving cell ID or UL/DL BWP as well. In such sense, if the majority desire to introduce a new MAC CE format for joint TCI state indication, the new MAC CE format as illustrated in Figure 1 can be considered.

Figure 1 Example MAC CE for joint TCI state indication.

- If new MAC CE for joint TCI state indication is introduced, it should be only used for the joint TCI state indication (see example format in Figure 1).

- If new MAC CE for joint TCI state indication is introduced, RAN2 to adopt the MAC CE format as illustrated in Figure 1.

If it is not acceptable to introduce a third MAC CE for join TCI state indication in addition to the existing two MAC CEs, one can consider to improve one of the existing MAC CEs (i.e. NCR Downlink Backhaul Beam Indication MAC CE or NCR Uplink Backhaul Beam Indication MAC CE in [2]) to be reused for joint TCI state indication. The R bit in NCR Downlink Backhaul Beam Indication MAC CE (or NCR Uplink Backhaul Beam Indication MAC CE) can be redefined to indicate if joint TCI state indication or DL TCI state (or UL TCI state) indication is carried (see Figure 2). If J/R = 0, the carried TCI state is for DL (or UL) TCI state indication, otherwise, the carried TCI state is for the joint TCI state indication.  For an NCR-MT which does not support joint TCI state indication, the J/R bit is always zero and the J/R bit is treated as an R bit.

Figure 2 Shared MAC CE for joint TCI state indication and DL (or UL) TCI state indication.

- RAN2 consider to improve NCR Downlink Backhaul Beam Indication MAC CE (or NCR Uplink Backhaul Beam Indication MAC CE) to be reused for joint TCI state indication by redefining the R bit to indicate if joint TCI state or DL (or UL) TCI state is carried.

# Conclusion

In this contribution, we discuss the upper layer procedures. Based on the discussion, we have the following observations:

Observation 1	The MAC CE for separate DL/UL BH link beam indication takes 2 bytes.

Observation 2	When DL/UL beam is indicated via separate MAC CEs, 4 bytes are used but one eLCID can be saved.

Based on the discussions and observations, we have the following proposals:

Proposal 1	If new MAC CE for joint TCI state indication is introduced, it should be only used for the joint TCI state indication (see example format in Figure 1).

Proposal 2	If new MAC CE for joint TCI state indication is introduced, RAN2 to adopt the MAC CE format as illustrated in Figure 1.

Proposal 3	RAN2 consider to improve NCR Downlink Backhaul Beam Indication MAC CE (or NCR Uplink Backhaul Beam Indication MAC CE) to be reused for joint TCI state indication by redefining the R bit to indicate if joint TCI state or DL (or UL) TCI state is carried.

# References

- R1-xxxx, 3GPP RAN1#112bise

- R2-2304415	Introducing support for Network Controlled Repeaters to 38.321	Samsung