3GPP TSG-RAN WG2 Meeting #122	R2-2306190

Incheon, Korea, 22 - 26 May, 2023

Agenda item:	7.25.1

Source:	Huawei, HiSilicon

Title:	Discussion on RAN4 LS on FR2 unknown SCell activation enhancement

Document for:	Discussion

- Introduction

RAN4 is working on reducing the FR2 unknown SCell activation delay, and the related RAN4 agreements are sent to RAN2 in LS R4-2306321 with the request for RAN2 signaling design.

In this contribution, to enable the RAN4 agreed solution, we discuss the potential options of RAN2 signaling design.

- Discussion

The agreed RAN4 solution to reduce the FR2 unknown SCell activation delay is:

When SCell activation command is received and UE has valid L3 measurement results available but not reported , RAN4 agreed that SCell activation delay can be shortened if the available L3 measurement results are reported to NW.

The RAN4 requirement is that RAN2 to design solution for following aspects:

1. How to configure the report of L3 measurement result 2. How to trigger the report of L3 measurement result 3. Contents of the report of L3 measurement result and how to send the report.

Thus the key points from RAN2 perspective is that the L3 measurement results to be reported are obtained according to the existing L3 measurement framework, i.e. the UE performs L3 measurement according the existing measurement configuration, including measurement objects, reporting configurations, measurement identities, and quantity configurations, which means only minimum specification changes are needed.

Observation 1: The L3 measurement results to be reported are obtained according to the existing L3 measurement framework, and only minimum specification changes are needed to enable RAN4 solution.

According to the current L3 measurement behavior, the UE performs measurement for each serving cell on which servingCellMO is configured, and stores the measurement results. When there is a reporting condition fulfilled, the UE will report the measurement results. The scenario on which RAN4 is considering is the UE periodically perform measurement on the configured SCell as serving cell, but the reporting condition has not been fulfilled yet when the SCell is to be activated. Then the SCell activation MAC CE needs to be considered as a new reporting condition. Since the SCell activation MAC CE is handled by MAC layer, it would be more symmetrical to deliver the measurement results via MAC layer reporting. This means a new MAC CE is to be specified, which should include SCell index, SSB index and corresponding RSRP. This MAC CE based reporting should be enabled/disabled by network. For instance, the UE will report the UE capability to the network, and when the network supports and wants to use this feature, the network enables this feature by explicit RRC configuration.

Observation 2: The following MAC CE based reporting can meet the RAN4 requirement with limited specification impact on MAC and RRC.

- The network indicates a UE to enable the MAC CE based reporting via an explicit RRC configuration.

- Upon reception of a MAC CE for SCell activation, the UE includes the L3 measurement results (if available) for the to-be-activated SCell in a new MAC CE, and sends the MAC CE to network.

In RAN4 discussion, companies also considered RRC based reporting, which is likely to reuse the existing measurement result message. Then the discussion point is how to specify the reporting condition in RRC layer. One possible way is to introduce a new measurement event which is receiving MAC CE for SCell activation.

Observation 3: The following RRC based reporting seems to be feasible as well.

- A new measurement event that a SCell is activated by MAC CE is to be specified.

- Upon reception of a MAC CE for SCell activation, the MAC layer indicates RRC layer, and RRC layer considers the new measurement event is fulfilled and then reports the existing L3 measurement results.

Although the RRC based solution can work, the existing events all based on L3 evaluation, now the SCell activation is from MAC layer, which seems a bit strange. Thus we prefer to adopt the MAC CE based reporting, which is clean and also have some advantage of processing delay.

Proposal: RAN2 to adopt the following MAC CE based reporting:

- The network indicates a UE to enable the MAC CE based reporting via an explicit RRC configuration.

- Upon reception of a MAC CE for SCell activation, the UE includes the L3 measurement results (if available) for the to-be-activated SCell in a new MAC CE, and sends the MAC CE to network.

- Conclusion

Based on the above discussion, the proposals from our side are:

Observation 1: The L3 measurement results to be reported are obtained according to the existing L3 measurement framework, and only minimum specification changes are needed to enable RAN4 solution.

Observation 2: The following MAC CE based reporting can meet the RAN4 requirement with limited specification impact on MAC and RRC.

- The network indicates a UE to enable the MAC CE based reporting via an explicit RRC configuration.

- Upon reception of a MAC CE for SCell activation, the UE includes the L3 measurement results (if available) for the to-be-activated SCell in a new MAC CE, and sends the MAC CE to network.

Observation 3: The following RRC based reporting seems to be feasible as well.

- A new measurement event that a SCell is activated by MAC CE is to be specified.

- Upon reception of a MAC CE for SCell activation, the MAC layer indicates RRC layer, and RRC layer considers the new measurement event is fulfilled and then reports the existing L3 measurement results.

Proposal: RAN2 to adopt the following MAC CE based reporting:

- The network indicates a UE to enable the MAC CE based reporting via an explicit RRC configuration.

- Upon reception of a MAC CE for SCell activation, the UE includes the L3 measurement results (if available) for the to-be-activated SCell in a new MAC CE, and sends the MAC CE to network.