3GPP TSG-RAN WG2 Meeting #122			                                          R2-2306203

Incheon, Korea, May 22-26, 2023

Agenda Item:    	7.5.3

Source: 		III

Title:			Discussion on various frame rates supported for XR-specific power saving

Document for:		Discussion

1 Introduction

In previous RAN2#121bis-e meeting, the following agreements were reached for XR-specific power saving [1]:

RAN2#121bis-e: 1:RAN2 will not consider solution 3, i.e. multiple active DRX configurations as a solution to the non-integer periodicity for XR traffic, i.e. UE would have only one DRX configuration. - 2: To address SFN wrap around, it is proposed to adopt option with a counter in DRX formula that increments at every SFN wrap around and an DRX reference SFN signalled by network. FFS if this is based on H-SFN, E-SFN or a generic counter.

This contribution discusses the various frame rates supported for DRX cycle mismatch in XR specific power saving. Furthermore, unnecessary power and unnecessary delay should be considered in the design of C-DRX enhancement for XR.

2 Discussion

The mismatch between non-integer periods of XR traffic and legacy DRX periods was discussed in previous meeting [2]. Three mechanisms were supported by most companies.

Option 1.1: non-integer DRX cycle.

Option 2.1: non-uniform integer DRX cycles pattern. As shown in Figure 1, there are two DRX cycles: 17ms and 16ms. The two DRX cycles are applied in a sequence of [16, 17, 17].

Figure 1.  DRX cycle pattern (e.g. 16, 17, 17)

Option 2.3: Adjustment DRX cycle length/start offset every number of cycles with integer DRX cycle. As shown in Figure 2, the alignment will be adjust every three DRX cycles.

Figure 2. drx-startoffset changed periodical to keep aligned

2.1 Various frame rates supported for the DRX cycle mismatch

In TR38.835, it mentioned XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) correspond to periodicities which are not an integer (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively) [3]. The DRX cycle pattern should support various frame rates.

The mechanism adjustment/modification for the above 3 options is as following:

For option 1-1, Modification of IE DRX-Config by adding non-integer DRX cycle value (66.66, 33.33, 22.22, 16.66, 13.88, 11.11 and 8.33ms respectively). How to modify the DRX cycle value for recurring decimal of non-integer DRX cycles, it may need more bits to approach recurring decimal by floating point (e.g. 16.67ms) or rational number (two integer values to express recurring decimal, e.g. 50/3ms).

Observation 1: The spec impact need to further clarify.

For option 2-1, 2 DRX cycle values is configured (e.g. 16ms and 17ms in IE DRX-Config: drx-LongCycleStartOffset value). Legacy long DRX cycle only configure with one value (in ms), how to configure two DRX cycle values need to further discussed, when XR traffic is 60fps (50/3ms), DRX cycle pattern needs 3-cycle period. In TR38.835, there mentioned many XR frame rate, design different suitable DRX cycle pattern is needed. For XR frame rates (15, 30, 60 and 120fps), 3-cycle period sequence is suitable. For XR frame rates (45, 72 and 90fps), 9-cycle period sequence is suitable. In general, design 9-cycle period pattern is sufficient for whole XR frame rates (15, 30, 45, 60, 72, 90 and 120fps) in TR 38.385.

Observation 2: 9-cycle period sequence of 2 DRX cycle values can be considered.

For option 2-3, insert drx-startoffset(s) in each cycle period for aligning the non-integer traffic interval. Due to minimize the average delay time of DRX cycle starting, the drx-startoffset inserting can’t be done after each cycle period. Insert drx-startoffset(s) of 1 uniformly over the cycle period will be a feasible solution. The uniformly adjustment of drx-startoffset in each 9-cycle period can be considered.

Observation 3: The adjustment of drx-startoffset uniformly in each 9-cycle period can be considered.

2.2 Unnecessary power and unnecessary delay consideration for DRX cycle patterns

Table 1 unnecessary power and delay for 
XR traffic of 60fps.

| 60fps | Power | Delay |
| --- | --- | --- |
| [17,17,16] | 0 | 2% |
| [17,16,17] | 0.68% | 0.66% |
| [16,17,17] | 2% | 0 |

Table 1 unnecessary power and delay for 
XR traffic of 60fps.

In XR power saving, different DRX patterns will cause unnecessary power and unnecessary delay. For each possible DRX pattern of 60fps [(17, 17, 16), (16, 17, 17), (17, 16, 17)], different unnecessary delay and unnecessary power was show as table 1.

| 60fps | Unnecessary power | Unnecessary delay |
| --- | --- | --- |
| [17,17,16] | 0 | 2% |
| [17,16,17] | 0.68% | 0.66% |
| [16,17,17] | 2% | 0 |

For example, the calculation of unnecessary power for DRX pattern (16, 17, 17) is as below (unnecessary power consumption was calculated proportionally to early wake-up time in average):

“(0.67ms+0.33ms+0ms) / 50m] *100% = 2%”. As show in figure 3.

- 0.67ms was caused by early wake-up for first cycle (16ms vs 16.67ms).

- 0.33ms was caused by early wake-up for second cycle (33ms vs 33.33ms).

- 0 ms was caused by early wake-up for third cycle (50ms vs 50ms).

The other two patterns ([17, 17, 16], [17, 16, 17]) were also calculated.

Figure 3. Unnecessary power for mismatch of DRX cycle (16, 17, 17) non-integer (50/3) XR traffic.

For example, the calculation of unnecessary delay for DRX pattern (17, 17, 16) is as below (unnecessary delay was calculated proportionally to early packet arrival time in average):

(0.33ms+0.66ms) / 50m] *100% = 2%. As show in figure 4.

- 0.33ms was caused by early packet arrival time for first cycle (17ms vs 16.67ms).

- 0.66ms was caused by early packet arrival time for second cycle (34ms vs 33.34ms).

- 0 ms was caused by early packet arrival time for third cycle (50ms vs 50ms).

The other two patterns ([16, 17, 17], [17, 16, 17]) were also calculated.

Figure 4. Unnecessary delay for mismatch of DRX cycle (17, 17, 16) and non-integer (50/3) XR traffic.

In previous meetings, the mismatch between legacy DRX cycle length of 16ms and non-integer XR traffic cycle of 16.67ms (60fps) was discussed but no further clarify the additional consumptions (e.g. unnecessary power, unnecessary delay).

The above proposals produce fixed patterns, either produce maximum unnecessary delay (e.g. [17, 17, 16], as show in Figure 4.) or maximum unnecessary power (e.g. [16, 17, 17] as show in Figure 3). Therefore, a flexible mechanism producing different patterns is necessary.

Proposal 1: More DRX Cycle patterns are needed for each DRX cycle to achieve flexible trade-off between unnecessary power and delay.

When DRX cycle of periodicity is represented in rational numbers (e.g. 60fps-50/3ms, 90fps-100/9ms) for both short and long DRX cycles. Then” floor” is added to the formula for determining the start time of DRX on durations by major companies.

For option 1.1, a new K value can be added to the formula for producing more patterns, the updated formula for short DRX cycle is as follows:

- floor{ [(SFN × 10) + subframe number + K] modulo (drx-ShortCycle) }=

floor{ (drx-StartOffset) modulo (drx-ShortCycle)}

When K satisfy below equation, each K value is mapping to one DRX cycle pattern.

- K modulo [celling (drx-shortcycle) - drx-shortcycle] = 0 , K belong to [0,1)

For option 2.1, a new K index was introduced for different DRX pattern.

For option 2.3, a new K index was introduced for different drx-startoffset pattern.

For 60fps, the corresponding K and DRX patterns are shown as Table 2, Table 3 and Table 4.

  - Option 1.1: K values (0, 1/3, 2/3), non-integer DRX cycle of unnecessary power and delay for XR, as show in Table 2.

  - Option 2.1: K index (0, 1, 2), non-uniform integer DRX cycles pattern -unnecessary power and delay for XR traffic, as show in Table 3.

  - Option 2.3: drx-startoffset pattern([1,1,0], [1,0,1], [0,1,1]) of unnecessary power and delay for XR traffic, as show in Table 4.

Table 2 Ration number pattern of unnecessary power and delay for XR traffic (60fps).

| 60fps | Unnecessary power | Unnecessary delay | K(value) |
| --- | --- | --- | --- |
| [17,17,16] | 0 | 2% | 0 |
| [17,16,17] | 0.67% | 0.67% | 1/3 |
| [16,17,17] | 2% | 0 | 2/3 |

| 60fps | Unnecessary power | Unnecessary delay | K(index) |
| --- | --- | --- | --- |
| [17,17,16] | 0 | 2% | 0 |
| [17,16,17] | 0.67% | 0.67% | 1 |
| [16,17,17] | 2% | 0 | 2 |

Table 3 Non-uniform integer DRX cycles pattern -unnecessary power and delay for XR traffic (60fps).

| 60fps | Unnecessary power | Unnecessary delay | K(index) | Drx-startoffset pattern |
| --- | --- | --- | --- | --- |
| [17,17,16] | 0 | 2% | 0 | [1,1,0] |
| [17,16,17] | 0.67% | 0.67% | 1 | [1,0,1] |
| [16,17,17] | 2% | 0 | 2 | [0,1,1] |

Table 4 drx-startoffset pattern of unnecessary power and delay for XR traffic (60fps).

- For 90fps, the corresponding K and DRX patterns are shown as Table 5, Table 6 and Table 7.

  - Option 1.1: K values (0, 1/9, 2/9, 3/9, 4/9, 5/9, 6/9, 7/9, 8/9), non-integer DRX cycle of unnecessary power and delay for XR, as show in Table 5.

  - Option 2.1: K index (0, 1, 2, 3, 4, 5, 6, 7, 8), non-uniform integer DRX cycles pattern -unnecessary power and delay for XR traffic, as show in Table 6.

  - Option 2.3: drx-startoffset pattern([1,1,0], [1,0,1], [0,1,1]) of unnecessary power and delay for XR traffic, as show in Table 7.

Table 5 unnecessary power and delay for XR traffic of 90fps

Table 2 unnecessary power and delay for XR traffic of 90fps

| 90fps | Unnecessary power | Unnecessary delay | K |
| --- | --- | --- | --- |
| [12,11,11,11,11,11,11,11,11] | 0 | 4% | 0 |
| [11,12,11,11,11,11,11,11,11] | 0.11% | 3.15% | 1/9 |
| [11,11,12,11,11,11,11,11,11] | 0.33% | 2.37% | 2/9 |
| [11,11,11,12,11,11,11,11,11] | 0.66% | 1.7% | 3/9 |
| [11,11,11,11,12,11,11,11,11] | 1.1% | 1.14% | 4/9 |
| [11,11,11,11,11,12,11,11,11] | 1.65% | 0.69% | 5/9 |
| [11,11,11,11,11,11,12,11,11] | 2.31% | 0.35% | 6/9 |
| [11,11,11,11,11,11,11,12,11] | 3.08% | 0.12% | 7/9 |
| [11,11,11,11,11,11,11,11,12] | 4% | 0 | 8/9 |

Table 6 Non-uniform integer DRX cycles pattern -unnecessary power and delay for XR traffic of 90fps.

| 90fps | Unnecessary power | Unnecessary delay | K |
| --- | --- | --- | --- |
| [12,11,11,11,11,11,11,11,11] | 0 | 4% | 0 |
| [11,12,11,11,11,11,11,11,11] | 0.11% | 3.15% | 1 |
| [11,11,12,11,11,11,11,11,11] | 0.33% | 2.37% | 2 |
| [11,11,11,12,11,11,11,11,11] | 0.66% | 1.7% | 3 |
| [11,11,11,11,12,11,11,11,11] | 1.1% | 1.14% | 4 |
| [11,11,11,11,11,12,11,11,11] | 1.65% | 0.69% | 5 |
| [11,11,11,11,11,11,12,11,11] | 2.31% | 0.35% | 6 |
| [11,11,11,11,11,11,11,12,11] | 3.08% | 0.12% | 7 |
| [11,11,11,11,11,11,11,11,12] | 4% | 0 | 8 |

Table 7 drx-startoffset pattern of unnecessary power and delay for XR traffic of 90fps.

| 90fps | Unnecessary power | Unnecessary delay | K | drx-startoffset pattern |
| --- | --- | --- | --- | --- |
| [12,11,11,11,11,11,11,11,11] | 0 | 4% | 0 | [1,0,0,0,0,0,0,0,0] |
| [11,12,11,11,11,11,11,11,11] | 0.11% | 3.15% | 1/9 | [0,1,0,0,0,0,0,0,0] |
| [11,11,12,11,11,11,11,11,11] | 0.33% | 2.37% | 2/9 | [0,0,1,0,0,0,0,0,0] |
| [11,11,11,12,11,11,11,11,11] | 0.66% | 1.7% | 3/9 | [0,0,0,1,0,0,0,0,0] |
| [11,11,11,11,12,11,11,11,11] | 1.1% | 1.14% | 4/9 | [0,0,0,0,1,0,0,0,0] |
| [11,11,11,11,11,12,11,11,11] | 1.65% | 0.69% | 5/9 | [0,0,0,0,0,1,0,0,0] |
| [11,11,11,11,11,11,12,11,11] | 2.31% | 0.35% | 6/9 | [0,0,0,0,0,0,1,0,0] |
| [11,11,11,11,11,11,11,12,11] | 3.08% | 0.12% | 7/9 | [0,0,0,0,0,0,0,1,0] |
| [11,11,11,11,11,11,11,11,12] | 4% | 0 | 8/9 | [0,0,0,0,0,0,0,0,1] |

Proposal 2: Add K value/index of different DRX cycle patterns to achieve flexibility.

- Conclusions

The following observations were made:

Observation 1: The spec impact need to further clarify.

Observation 2: 9-cycle period sequence of 2 DRX cycle values can be considered.

Observation 3: The adjustment of drx-startoffset uniformly in each 9-cycle period can be considered.

Proposal 1: More DRX Cycle patterns are needed for each DRX cycle to achieve flexible trade-off between unnecessary power and delay.

Proposal 2: Add K value/index of different DRX cycle patterns to achieve flexibility.

4 References

- R2-2304202, Report on LTE legacy, XR, QoE and MUSIM, Nokia, February-March, 2023

- R2-2303861, DRX cycle alignment for XR, Nokia, Nokia Shanghai Bell, CMCC, China Unicom, DENSO CORPORATION, Ericsson, Intel, Google Inc., Huawei, HiSilicon, Samsung, Xiaomi, 17th – 26th April 2023.

- TR 38.835 Study on XR enhancements for NR, March 2023.