3GPP TSG-RAN WG2 Meeting #122	R2-2306215

Incheon, Korea, 22 May – 26 May, 2023                    (revision of R2- 2303808)

Agenda item:	7.8.2

Source: 	Huawei, HiSilicon

Title: 	Measurement report enhancement for UAV

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #121-bis, the following agreements have been reached regarding measurement report enhancement  for NR UAVs [1]:

Agreements

- Height-dependent more-than-one configurations is supported on parameter/field level (i.e. different fields/values within the same MO) where different values (or value ranges) of the parameter/field applies to different height or height range.

- For MO configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: SSB-ToMeasure. Details on how to specify is FFS.    FFS on UE behavior on L1 and L3 measurement.

- For MR configuration parameters: at least the following will have ability to be configured with height-dependent more-than-one configurations/values, each for a specific height region: Event A4 threshold and numberoftriggeringcells.  Details on how to specify is FFS (i.e. maybe it can be achieved by combination of events).

- When height-dependent more-than-one configurations are provided, UE applies the new value once it moves to new height (or height range) similar to the case of RRC reconfiguration. Need Codes, field descriptions, etc. as in legacy specifications apply

- If a height-specific value is not explicitly configured for certain height, whether to keep using the value that was used or consider the parameter as released (i.e. parameter/value not applicable at this height) should be looked into case by case, and can be clarified by need code, field description, or procedural text as needed.   FFS details

As we can see, there are some FFS that remain to be analysed. This contribution a revision of R2-2303808. Compared to the previous version we analysed a few more technical aspects due to be resolved in RAN2 (i.e. FFSs).

- Discussion

As the UAV ascends, the radio environment will change dramatically when it crosses the roof level. This is because the NLoS path is dominant when the UAV is under the roof level, and the LoS path will be dominant gradually as the UAV is over the roof level. Besides, even with the UAV over the roof level, the radio environment may also vary in different height ranges. This is because the signal received by the UAV are the side lobes of the gNBs, and it varies in different height ranges. Thus, the height-dependent configurations/values need to consider multiple height ranges.

Proposal 1: The height-dependent configurations/values need to consider multiple height ranges.

RAN2 agreed to use height-dependent SSB-ToMeasure in the last meeting. As we know, SSB-ToMeasure is used to indicate the set of SS blocks to be measured within the SMTC measurement duration. For the height-dependent SSB-ToMeasure, it should be configured by the network, i.e., the network configures the correspondence between the SSB-ToMeasure and different height ranges for the UAV in advance.

Proposal 2: Height-dependent SSB-ToMeasure can be indicated by the correspondence between the SSB-ToMeasure and different height ranges and is configured by the gNB.

Regarding the L1 and L3 measurements when the height-dependent SSB-ToMeasure is applied, one issue needs to be considered, i.e., what should the UE do if it is still measuring the beams but the UAV reaches the next height range? We think the UAV should keep the measurement result and continue to measure in order to minimize the spec impact. This is mainly because if the UAV removes the measurement results and re-measures them, there may be RAN1 impact, and we do not have enough time to discuss it.

Proposal 3: For height-dependent SSB-ToMeasure, the UAV should keep the measurement results and continue to measure the beams when it reaches the next height range.

RAN2 also agreed on the height-dependent event A4 threshold in the last meeting. Some companies said it could be achieved through event combination. However, if we use the combination of event A4 and H1/H2 as the solution for the height-dependent event A4 threshold, it will be redundant, especially for different height ranges. For instance, assume that the network wants to configure the correspondence between the height and event A4 threshold, e.g., event A4 threshold A with height range (100m, 200m). Obviously, it can be simply configured in one ReportConfig. But, if we want to describe it by event combinations, the network needs to configure at least two ReportConfig, i.e., event A4 with event H1 and event A4 with event H2, if we adopt one measID to link two ReportConfig. Compared to configuring the correspondence in one ReportConfig, the event combination is more complex. Thus, we think the height-dependent event A4 threshold should be configured by the network, i.e., the NW configures the correspondence between the event A4 threshold and different height ranges for the UAV in advance.

Proposal 4: Height-dependent event A4 threshold can be indicated by the correspondence between the event A4 threshold and different height ranges and is configured by the gNB.

Regarding the height-dependent numberOfTriggeringCells, as the height-dependent event A4 threshold, we think the height-dependent numberOfTriggeringCells should be configured by the network, i.e., the NW configures the correspondence between the numberOfTriggeringCells and different height ranges for the UAV in advance.

Proposal 5: Height-dependent numberOfTriggeringCells can be indicated by the correspondence between the numberOfTriggeringCells and different height ranges and is configured by the gNB.

If one cell fulfills the triggering condition, it will be added to the cellTriggeredList. Then, the UE sends the MR to the NW when the number of cells in the cellTriggeredList is greater than numberOfTriggeringCells. This is the measurement report mechanism in LTE if the numberOfTriggeringCells is configured. According to 38.331, the UE will remove the cell in the cellsTriggeredList if the NumberOfTriggeringCells is re-configured because the measurement reporting entry needs to be removed when the ReportConfig is re-configured. The report delay issue will be heavier if the UE removes the cell in the cellsTriggeredList when the NumberOfTriggeringCells is re-configured. Thus, if height-dependent numberOfTriggeringCells is applied, the UE applies a new numberOfTriggeringCells when it reaches a new height range. At this moment, the UE should keep the cells in the cellTriggeredList because it can alleviate the measurement report delay that is introduced by numberOfTriggeringCells.

Proposal 6: The UE should maintain the measured cells in cellTriggeredList when a new numberOfTriggeringCells is applied due to the UE reaching a new height range.

The ping-pong issue is likely if height-dependent parameters are introduced. For example, if we assume the height threshold is 100m, if the UAV is flying around 100m, the MR will be triggered frequently. Thus, hysteresis should be considered for height-dependent parameters.

Proposal 7: In order to avoid the ping-pong issue after introducing height-dependent parameters, hysteresis should be considered.

In the present, we discuss the height dependent multiple configurations for RRC_connected UAV. However, the UAV may be in RRC_IDLE or RRC_Inactive state when it is flying. The radio environment in the air is different from the ground due to the side lobes and the scatter of the ground’s signal. For instance, because the coverage of a cell is different at different heights, the idle and inactive UAV needs to consider the height when it selects the camp cell in order to improve the efficiency of cell (re)selection. Thus, the height-dependent measurement configurations should be extended to RRC_IDLE or RRC_Inactive UAV. Furthermore, the NW can configure the allowed cell list to the UAV according to the height of UAV. Specifically, the NW broadcast the height threshold and the corresponding allowed cell list to measure. Then, the UAV measures the cells include in the corresponding list when it reaches the height threshold.

Proposal 8: The height-dependent multiple measurement configurations should be extended to idle/inactive UE.

- Conclusion

Proposal 1: The height-dependent configurations/values need to consider multiple height ranges.

Proposal 2: Height-dependent SSB-ToMeasure can be indicated by the correspondence between the SSB-ToMeasure and different height ranges and is configured by the gNB.

Proposal 3: For height-dependent SSB-ToMeasure, the UAV should keep the measurement results and continue to measure the beams when it reaches the next height range.

Proposal 4: Height-dependent event A4 threshold can be indicated by the correspondence between the event A4 threshold and different height ranges and is configured by the gNB.

Proposal 5: Height-dependent numberOfTriggeringCells can be indicated by the correspondence between the numberOfTriggeringCells and different height ranges and is configured by the gNB.

Proposal 6: The UE should maintain the measured cells in cellTriggeredList when a new numberOfTriggeringCells is applied due to the UE reaching a new height range.

Proposal 7: In order to avoid the ping-pong issue after introducing height-dependent parameters, hysteresis should be considered.

Proposal 8: The height-dependent multiple measurement configurations should be extended to idle/inactive UE.

- Reference

[1] Draft_R2_121bis_meeting_report_v1