3GPP TSG-RAN WG2 Meeting #122	R2-2306218

Incheon, Korea, 22 May – 26 May, 2023

Agenda item:	7.8.5

Source: 	Huawei, HiSilicon

Title: 	Discussion on UAV remote identification broadcast

Document for:	Discussion and Decision

- Introduction

In RAN2 meeting #121-bis, the following agreements have been reached regarding UAV ID broadcast for NR UAVs [1]:

Agreements:

- DAA can be supported using the same framework as used for BRID transmission over the LTE and NR PC5 interface, without any specific enhancements.

- LTE PC5 Mode-4 resource allocation is supported, and LTE PC5 Mode-3 is not supported

- NR PC5 mode-1 is not supported

- For LTE PC5, we will follow the NR PC5 framework agreements, unless explicitly identified e.g. a strong technical reason

- RAN2 assumes that BRID and DAA services will be delivered on a frequency designated by regulators

- As a baseline, we will use the existing V2X QoS framework.  FFS whether different resource pools are needed for UAV services

- No further enhancement on PC5 range for A2X broadcast services will be pursued in this release

- We will not investigate interference for BRID and DAA broadcast

- Send an LS to SA2 to:

- inform them as a result of RAN Plenary decision to re-use BRID RAN2 will only support PC5 broadcast for deconfliction in RAN in release 18.

- ask SA2 whether BRID and DAA broadcast over LTE and NR PC5 requires new QoS requirements and parameters not supported within the ranges supported for V2X

As we can see, there are some FFS that remain to be analysed. In this contribution, we analysed the FFS carefully and a proposal is provided.

- Discussion

In previous meeting, companies proposed to introduce separate resource pool for A2X communication to satisfy the QoS requirement. However, whether there is any specific QoS requirement for A2X communication is not clear. Therefore, in the last meeting, a liaison was sent out by RAN2 to check SA2’s view.

From the perspective of some companies, it seems that if there is new QoS requirement, there should be a separate resource pool for A2X communication. However, the logic is not valid from our point of view.

Firstly, even if SA2 confirms new QoS requirement for A2X communication, then how to prove the existing resource pool shared with V2X cannot satisfy the new QoS requirement? Normally, such discussion should be performed by RAN1 and simulation results should be provided to show that the shared resource pool is not enough and in addition, a separate resource pool without any additional enhancement comparing with legacy resource pool design is enough. However, we don't think such RAN1 standard work is possible at this stage and such work is not even in the scope of the WID/objective.

Secondly, if SA2 does not confirm the need for and specify any new QoS requirement for A2X communication, then the situation is quite simple, and the A2X communication can be treated in the same way as V2X communication from RAN perspective, i.e. no change is needed, unless some specific issue was identified.

In conclusion, in our understanding the introduction of separate resource pool for A2X communication can be considered only if there is a clear performance gap or new requirement which can be evaluated by RAN2 itself. But for now, we have not seen enough justification for this solution direction.

Proposal 1: No separate resource pool is needed for A2X communication.

- Conclusion

Proposal 1: No separate resource pool is needed for A2X communication.

- Reference

[1] R2_121bis-e_Notes_R18 UAV NES R15-17 UP_SDT (Diana)_04-25_15-30