3GPP TSG-RAN WG2 Meeting #122                                                       R2-2306220

Incheon, KR, May 22 – 26 2023

Source:              CATT

Title:	             On UAI RLM/BFD reports when “No DRX is used”

Agenda Item:	6.1.3.1

Document for:	Discussion and Decision

Introduction

This contribution discusses the issue raised during the offline [006][NR17] CP PowSav and DCCA Corrections (CATT) regarding the frequent UAI reports of RLM/BFD relaxation states when “No DRX is used”, referring to the RAN4 definition of the latter condition.

Discussion

  - Issue summary and RAN4 specification

During offline [006][NR17] CP PowSav and DCCA Corrections (CATT) discussed in RAN2#121bis-e [1][2], the CR in [3] observed that RAN4 specification TS38.133 disallows the UE to perform RLM/BFD relaxation while DRX timers are running (UE is in Active Time with traffic activity), and this results in the RLM/BFD relaxation state to toggle frequently. In such cases proponents suggest such state switches (due to DRX state switch) do not trigger a UAI report.

Looking closer at the RAN4 specification, the key issue reduces to how should be interpreted the condition “No DRX is used” in the below extracts of TS38.133:

#### 8.1.1.1 Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria … The UE is no longer allowed to relax RLM measurements and apply the relaxed radio link monitoring provided that at least one of the following conditions is met: - The UE sends out-of sync indications to the higher layers, - The timer T310 is running. - No DRX is used or DRX cycle is longer than 80ms … #### 8.5.1.1 Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria … The UE is no longer allowed to relax BFD measurements and apply the relaxed link recovery procedures provided that at least one of the following conditions is met: - The timer beamFailureDetectionTimer is running. - No DRX is used or DRX cycle is longer than 80ms

Since last e-meeting, we took the time to double-check internally with RAN4 colleagues and there seems to be no ambiguity that the following “No DRX is used” definition captured in Clause 3.6.1 applies to above clauses as well as other clauses specifying measurements in RRC Connected (e.g. it is also extensively referred to in chapter 9 of 38.133):

3.6.1 RRC connected state requirements in DRX For the requirements in RRC connected state specified in this version of the specification, the UE shall assume that no DRX is used provided the following conditions are met: - DRX parameters are not configured or - DRX parameters are configured and - drx-InactivityTimer is running or - drx-RetransmissionTimerDL is running or - drx-RetransmissionTimerUL is running or - ra-ContentionResolutionTimer is running or - a Scheduling Request sent on PUCCH is pending or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity Otherwise the UE shall assume that DRX is used

Our understanding of the RAN4 intention with the above definition of “No DRX is used” from clause 3.6.1 of TS38.133 is that the UE is not allowed to relax in Connected mode as soon as it is in C-DRX Active Time with traffic activity (data Tx/Rx or RACH or SR), which is why the drx-onDurationTimer is not included.

Now, whether this was indeed the RAN4 intention when adding this “No DRX is used” condition in above clauses 8.1.1.1 and 8.5.1.1 or not is neither clear nor captured in the cover page of the related RAN4 CR [4]. Moreover, during the offline discussion [006][NR17] CP PowSav and DCCA Corrections (CATT), several companies challenged this interpretation (including the company author of the RAN4 CR [4]) claiming it rather means “No DRX is configured”.

As a result, we expect that this “No DRX is used” definition from clause 3.6.1 and applicability to clauses 8.1.1.1 and 8.5.1.1 will be shortly re-discussed in RAN4. So, from RAN2 perspective, we suggest waiting for RAN4 to clarify the relevance of the issue before we decide a fix (if any) in RRC.

Proposal: RAN2 waits for RAN4 to converge on the applicability of the “No DRX is used” definition from TS38.133 clause 3.6.1 to the RLM/BFD relaxation conditions in TS38.133 clauses 8.1.1.1 and 8.5.1.1 before deciding a fix (if any) in RRC.

Conclusion

This contribution discussed the issue raised during of the frequent UAI reports of RLM/BFD relaxation states when “No DRX is used”, referring to the RAN4 definition of the latter condition, and concluded:

Proposal: RAN2 waits for RAN4 to converge on the applicability of the “No DRX is used” definition from TS38.133 clause 3.6.1 to the RLM/BFD relaxation conditions in TS38.133 clauses 8.1.1.1 and 8.5.1.1 before deciding a fix (if any) in RRC.

References

- R2-2304445, Report of [AT121bis-e][006][NR17] CP PowSav and DCCA Corrections (1st round), CATT

- R2-2304538, Report of [AT121bis-e][006][NR17] CP PowSav and DCCA Corrections – 2nd round, CATT

- R2-2302800, Correction on RLM/BFD relaxation state reporting	Nokia, Nokia Shanghai Bell

- R4-2211103, 38.133CR2386, CR on TS38.133 for applicability of RLM measurement relaxation, MediaTek Inc.