3GPP TSG-RAN WG2 Meeting #122	R2-2306224

Incheon South Korea, May 22nd - 26th 2023

Agenda Item:	7.19.1

Source:	Ericsson

Title:	Discussion on Msg4 PDSCH transmission to Rel-18 eRedCap UEs

Document for:	Discussion, Decision

# 1	Introduction

Related to the Rel-18 work item on “Enhanced support of reduced capability NR devices” [1], RAN2 has received an LS from RAN1 [2] with the following description:

RAN1 discussed Msg4 PDSCH transmission to Rel-18 eRedCap UEs and made the following agreement: Agreement Confirm the following working assumption by assuming that Msg3 indication is available Working Assumption - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.. RAN1 would like to inform RAN2 about the following case, to consider, if needed, the UE behavior in the RAN2 specifications, and ask RAN2 for feedback if any: - For UE BB complexity reduction, the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process The case was also discussed in RAN1 in Question 2.7-2b of summary R1-2303936.

The LS action from RAN1 to RAN2 is to take the above into account in future work and to provide feedback to RAN1 if any. In this contribution we provide our view on this topic.

# 2	Discussion

The LS from RAN1 asks about the case when a UE (supporting Rel-18 eRedCap UE BB complexity reduction) detects a DCI and discovers that the DCI schedules a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process.

RAN2 has already agreed that gNB receives an early indication in Msg3/MsgA PUSCH. Therefore, if there is no contention with some other UE(s), the gNB will be aware that the UE is a Rel-18 eRedCap UE when it schedules the Msg4 PDSCH transmission to the UE, and the gNB will schedule the Msg4 PDSCH transmission with a bandwidth that the UE can receive and process.

However, if two (or more) UEs happen to send the same Msg1 preamble, there will be contention, and then it might happen that a Rel-18 eRedCap UE detects a DCI for a Msg4 PDSCH transmission that has a larger bandwidth than the UE can receive or process, since the transmission was in fact intended for some other UE.

As mentioned in the LS, RAN1 has discussed this in Question 2-7-.2b in the RAN1 feature lead summary (FLS) in [3], where the following options are listed:

- Option 1: The UE considers the contention resolution as not successful.

- Option 2: The UE discards the DCI and continues monitoring the DCI until ra-ContentionResolutionTimer expires.

- Option 3: The UE behavior is up to the UE implementation.

In the FLS [3], most of the received responses to Question 2.7-2b indicate that they prefer Option 1 or Option 3 over Option 2, with an even split in preference between Option 1 and Option 3. The FLS [3] also contains a Proposal 2.7-2c corresponding to Option 1, with the received responses evenly split between ‘Yes’ and ‘No’. So, from the RAN1 discussion, Options 1 and 3 are the options with most (and equal) support.

In 38.321 the following procedure is captured for contention resolution:

### 5.1.5	Contention Resolution

Once Msg3 is transmitted the MAC entity shall:

1>	if the Msg3 transmission (i.e. initial transmission or HARQ retransmission) is scheduled with Type A PUSCH repetition:

<cut>

1>	else:

2>	start or restart the ra-ContentionResolutionTimer in the first symbol after the end of the Msg3 transmission.

1>	monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap;

1>	if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

2>	if the C-RNTI MAC CE was included in Msg3:

<cut>

2>	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI:

3>	if the MAC PDU is successfully decoded:

4>	stop ra-ContentionResolutionTimer;

4>	if the MAC PDU contains a UE Contention Resolution Identity MAC CE; and

4>	if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3:

5>	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

5>	if this Random Access procedure was initiated for SI request:

6>	indicate the reception of an acknowledgement for SI request to upper layers.

5>	else:

6>	set the C-RNTI to the value of the TEMPORARY_C-RNTI;

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Random Access procedure successfully completed.

4>	else:

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

1>	if ra-ContentionResolutionTimer expires:

2>	if Msg3 transmission was transmitted on a non-terrestrial network:

<cut>

2>	else:

3>	discard the TEMPORARY_C-RNTI;

3>	consider the Contention Resolution not successful.

1>	if the Contention Resolution is considered not successful:

<cut>

In legacy (as highlighted above), during initial random access if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI, contention resolution is considered not successful, and MAC PDU is discarded if it was successfully decoded.

Option 1 is more beneficial from UE power saving perspective, as the UE does not have to attempt receiving the corresponding PDSCH (which targets a UE without BB bandwidth reduction) or continue monitoring DCI. It can consider contention resolution not successful and continue with the procedure described in subclause 5.1.5. Option 2, on the other hand, is not beneficial from UE power saving perspective as it would require that the UE continues to monitor PDCCH until ra-ContentionResolutionTimer expires after discarding the DCI. Option 3 would lead to ambiguous UE behavior for UEs with UE BB bandwidth reduction as some may continue monitoring PDCCH until ra-ContentionResolutionTimer expires. Note that in the current MAC spec, the case where MAC PDU is not successfully decoded is not captured, so UE would need to wait until the ra-ContentionResolutionTimer expires which is not beneficial from UE power saving perspective.

For the case raised in the LS from RAN1 for UE BB complexity reduction, UE behavior is specified, i.e., it is not left up to the UE implementation.

We think that it would be beneficial if UE considers that contention resolution is not successful, similar to legacy, for the case raised in the LS from RAN1 for UE BB complexity reduction from UE power saving perspective.

For the case raised in the LS from RAN1 for UE BB complexity reduction, UE considers that contention resolution is not successful.

Since the case where MAC PDU is not successfully decoded is not captured in the MAC spec, we suggest the following update:

### 5.1.5	Contention Resolution

Once Msg3 is transmitted the MAC entity shall:

<cut>

2>	else if the CCCH SDU was included in Msg3 and the PDCCH transmission is addressed to its TEMPORARY_C-RNTI:

3>	if the MAC PDU is successfully decoded:

4>	stop ra-ContentionResolutionTimer;

4>	if the MAC PDU contains a UE Contention Resolution Identity MAC CE; and

4>	if the UE Contention Resolution Identity in the MAC CE matches the CCCH SDU transmitted in Msg3:

5>	consider this Contention Resolution successful and finish the disassembly and demultiplexing of the MAC PDU;

5>	if this Random Access procedure was initiated for SI request:

6>	indicate the reception of an acknowledgement for SI request to upper layers.

5>	else:

6>	set the C-RNTI to the value of the TEMPORARY_C-RNTI;

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Random Access procedure successfully completed.

4>	else:

5>	discard the TEMPORARY_C-RNTI;

5>	consider this Contention Resolution not successful and discard the successfully decoded MAC PDU.

3>	else

4>	stop ra-ContentionResolutionTimer;

4>	discard the TEMPORARY_C-RNTI;

4>	consider this Contention Resolution not successful.

1>	if ra-ContentionResolutionTimer expires:

<cut>

Update the MAC spec to capture that UE considers contention resolution as not successful for the case raised in the LS from RAN1 for UE BB complexity reduction.

Send an LS to RAN1 to inform that UE considers contention resolution as not successful for the case raised in the LS from RAN1 for UE BB complexity reduction and the related MAC spec update.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the case raised in the LS from RAN1 for UE BB complexity reduction, UE behavior is specified, i.e., it is not left up to the UE implementation.

Proposal 2	For the case raised in the LS from RAN1 for UE BB complexity reduction, UE considers that contention resolution is not successful.

Proposal 3	Update the MAC spec to capture that UE considers contention resolution as not successful for the case raised in the LS from RAN1 for UE BB complexity reduction.

Proposal 4	Send an LS to RAN1 to inform that UE considers contention resolution as not successful for the case raised in the LS from RAN1 for UE BB complexity reduction and the related MAC spec update.

# References

RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#98-e, December 2022.

R1-2304262, “LS on Msg4 PDSCH transmission to Rel-18 eRedCap UEs” (from RAN1 to RAN2)

R1-2303936, “FL summary #4 on Rel-18 RedCap UE complexity reduction”, Moderator (Ericsson)