3GPP TSG-RAN WG2 Meeting #122	R2-2306225

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	7.20.3

Source:	Samsung

Title:	Remaining issues on unified TCI extension to mTRP operation

Document for:	Discussion & Decision

Introduction

Based on the WID of Rel-18 MIMO (MIMO Evolution for Downlink and Uplink) [1], RAN1 has started to investigate the enhancements for uplink/downlink MIMO features and following objective has RAN2 impact.

2. Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

Regarding this objectives, RAN1 provided the intermediate results including all RAN2 relevant agreements which contains RRC and MAC impacts in LS [2][3]. Among all above issues, the study to enhance the multi-TRP operation using Rel-17 unified TCI state framework has defined in the objective and there are many progress in RAN1. See the section 5 (Appendix) including the RAN1 agreements.

In this contribution, we consider the remaining MAC issues on the unified TCI extension to mTRP operation.

Multi-TRP operation using unified TCI framework

In Rel-17, unified TCI framework was specified for multi-beam operation, where a common TCI state – separate DL or UL TCI state or joint DL and UL TCI state – can be indicated for UE-dedicated reception/ transmission on PDCCH/PDSCH/PUCCH/PUSCH.

Here, unified TCI state activate/deactivate MAC CE indicating the common TCI state signals maximum 16 TCI states (i.e. 8 TCI code points) for separate UL/DL TCI state case, if joint DL/UL TCI state is used at most 8 TCI states can be activated. For DCI (DCI format 1_1 or 1_2 with or without PDSCH assignment) indicates one code point used by UE. However, Rel-17 unified TCI framework only supports the single TRP (sTRP) operation, e.g. inter-cell beam management (ICBM).

In Rel-16/17, various mTRP operations were introduced using the legacy TCI framework.

- SDCI based PDSCH transmission schemes specified in Rel-16

- MDCI based PDSCH transmission scheme specified in Rel-16

- PDCCH/PUCCH/PUSCH repetition schemes specified in Rel-17

- HST-SFN (single frequency network) specified in Rel-17

Rel-18 unified TCI framework enhancements targets on the extending the Rel-17 unified TCI to the mTRP operation e.g. by indicating in a DCI multiple TCI states or pairs of TCI states for various MTRP schemes (sDCI based mTRP, mDCI based mTRP, etc.).

In below sub-clauses, we provides the view on MAC impacts for two mTRP operations (sDCI based mTRP and mDCI based mTRP) using the unified TCI framework based on RAN1 agreements (see Appendix).

Multi-PDCCH based multi-TRP operation

During the RAN2#121bis-e meeting, RAN2 made following working assumption based on the online discussion.

Working assumption:

Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

From our understanding, all RAN2 companies agreed with the contents of above working assumption but it would be better to wait further RAN1 agreements because those MAC CE design could be changed based on further RAN1 agreements. Note that the detail MAC CE design and explanations are provided in [4].

After checking the RAN1 agreements in RAN1#112bis meeting which are related with mDCI based mTRP operation, there were no agreements to change the above working assumption. Therefore, we think that the above RAN2 working assumption could be a new RAN2 agreement.

Proposal 1: RAN2 confirm the following working assumption as an agreement.

Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

Single-PDCCH based multi-TRP operation

In [5], the functional operations based on RAN1 agreements were well summarized for each sDCI based mTRP operations i.e. how the unified TCI states are indicated are different for the different physical channels (PDCCH, PDSCH, PUCCH, PUSCH). In addition RAN1 made following agreements for the sDCI based PUSCH transmission.

Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes

Following is the summary of the TCI state indication for different physical channels (PDCCH, PDSCH, PUCCH, PUSCH).

- sDCI based PDCCH repetition: RRC indication per CORESET

- sDCI based PDSCH transmission: DCI indication

- sDCI based PUCCH repetition: RRC indication per PUCCH resource/group

- sDCI based PUSCH repetition: DCI indication

- sDCI based type-1 CG-PUSCH transmission: RRC indication per CG-PUSCH transmission

- sDCI based DG and type-2 CG-PUSCH transmission: DCI indication

For sDCI based mTRP operation, two TCI states for mTRP could be indicated by MAC CE, and the required signaling could be different based on the signaling type of unified TCI state (i.e. either joint DL/UL TCI state or separate DL/UL TCI state).

- Joint DL/UL TCI state configuration

  - Since UL and DL uses the common TCI state fields (i.e. DL/joint TCI state), both TRP #1 and TRP #2 could use the same TCI state framework what it was configured.

- Separate DL/UL TCI state configuration

  - Since UL and DL uses the separate TCI state fields (i.e. DL/joint TCI state for DL, UL TCI state for UL), both TRP 1 and TRP2 could use the same TCI state framework what it was configured.

We wonder if the sDCI based mTRP enhancements using unified TCI framework considers the intra-cell case or both intra/inter cell cases because Rel-16 sDCI based mTRP operation only considers intra-cell case. It means, TCI state activation/deactivation MAC CE only includes the TCI states in a serving cell per TRP (i.e. do not support to indicate the different serving cell’s TCI states per TRP). However, we think that the unified TCI states are used for mTRP with mDCI/sDCI could support both intra-cell and inter-cell mTRP since the unified TCI states can refer to additional PCIs other than serving cell PCI. RAN1 made following agreements in RAN1#112bis-e meeting how mDCI based mTRP operation using unified TCI framework could be worked for intercell case.

Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL /UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . - Note: How to implement above in specification is up to spec editor

We think that sDCI based mTRP operation using unified TCI state framework could also support the inter-cell cases i.e. the activated unified TCI state(s) for the first TRP can be associated with the serving cell PCI and the activated unified TCI state(s) for the second TRP can be associated with a PCI other than the serving cell PCI .

Proposal 2: sDCI based mTRP operation using unified TCI state framework considers the intra-cell and inter-cell i.e. the activated unified TCI state(s) for the first TRP can be associated with the serving cell PCI and the activated unified TCI state(s) for the second TRP can be associated with a PCI other than the serving cell PCI.

For MAC CE design, there could be two options 1) revised the legacy unified TCI state activation/deactivation MAC CE, or 2) introduce the new MAC CE for sDCI based mTRP operation using unified TCI state framework. However, as RAN2 did in Rel-16 for sDCI based mTRP opration, introducing the new MAC CE for sDCI based mTRP operation using unified TCI state seems better because we can efficiently design the MAC CE.

RAN1 made following agreements during RAN1#112bis-e meeting which are helpful to design the detail MAC CE for TCI state activation/deactivation of sDCI based mTRP.

Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

Based on above RAN1 agreements, at least RAN2 needs to introduce the new field(s) to consider following aspects.

- A TCI codepoint includes TCI state(s) from one or two TRP(s)

- Whether TCI state(s) for TRP(s) uses the joint DL/UL TCI mode or the separate TCI mode

- If  the indicated TCI codepoint is consist of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

Proposal 3: For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE containing following TCI state information of mTRPs.

- If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRPs.

- Introduce the new field(s) indicating:

  - whether TCI state(s) for TRP(s) uses the joint DL/UL TCI mode or the separate TCI mode

  - if the unified TCI state of the second TRP is present or not,

  - if  the indicated TCI codepoint is consist of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

Conclusion

We propose following proposals:

Proposal 1: RAN2 confirm the following working assumption as an agreement.

Revise the legacy unified TCI state activation/deactivation MAC CE by adding a “CORESET Pool ID” field to support mDCI based mTRP operation.

Proposal 2: sDCI based mTRP operation using unified TCI state framework considers the intra-cell and inter-cell i.e. the activated unified TCI state(s) for the first TRP can be associated with the serving cell PCI and the activated unified TCI state(s) for the second TRP can be associated with a PCI other than the serving cell PCI.

Proposal 3: For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE containing following TCI state information of mTRPs.

- If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRPs.

- Introduce the new field(s) indicating:

  - whether TCI state(s) for TRP(s) uses the joint DL/UL TCI mode or the separate TCI mode

  - if the unified TCI state of the second TRP is present or not,

  - if  the indicated TCI codepoint is consist of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s)

References

RP-213598	WID: MIMO Evolution for Downlink and Uplink	Samsung.

R1-2302225	LS to RAN2/4 on Agreements for Rel-18 MIMO	Samsung.

RAN1 Chair’s Notes	RAN1#112bis-e.

R2-2303064	MAC impacts on the enhancements of the unified TCI state framework	Samsung

R2-2302880	Extension of unified TCI framework for mTRP		Huawei.

Appendix

RAN1 agreements (post-RAN1#112) on unified TCI for mTRP

with potential RAN2/RAN4 impact

RAN1#112bis

Agreement Agreement On unified TCI framework extension, the Rel-17 timeline for updating the indicated joint/DL/UL TCI state(s) is retained, i.e., the indicated joint/DL/UL TCI state(s) applied to the DL reception or UL transmission in each slot is updated based on the Rel-17 beam application time Agreement On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG) Agreement On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission - If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If both indicated joint/UL TCI states are applied: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB - FFS: SDM and SFN based PUSCH Tx schemes Agreement On unified TCI framework extension for S-DCI based MTRP, PDSCH-CJT Tx scheme is RRC-configured, and dynamic switching between PDSCH-CJT and other S-DCI based PDSCH Tx schemes is not supported Agreement If the UE is configured with SSB-MTC-AdditionalPCI and receives TCI state activation command (MAC-CE) that activates a set of joint/DL /UL TCI state(s) specific to each coresetPoolIndex value for M-DCI based MTRP in unified TCI framework extension, the activated joint/DL /UL TCI state(s) specific to one coresetPoolIndex value is associated with the serving cell PCI and the activated joint/DL /UL TCI state(s) specific to another coresetPoolIndex value can be associated with a PCI other than the serving cell PCI . - Note: How to implement above in specification is up to spec editor Agreement On unified TCI framework extension for M-DCI based MTRP , after NW response to TRP-specific BFR request to a BFD-RS set associated with a coresetPoolIndex value, QCL assumption/spatial Tx filter/PL-RS for channel(s)/signal(s) that applies the indicated joint/DL /UL TCI state specific to the coresetPoolIndex value are updated according to the new beam (q new ) corresponding to the BFD-RS set. Agreement On unified TCI framework extension for S-DCI based MTRP, the presence of the [TCI selection field] can be RRC-configured per DL BWP - FFS: Whether the presence of the [TCI selection field] can be configured individually for DCI format 1_1 and DCI format 1_2 in the same DL BWP Agreement On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint Agreement On unified TCI framework extension for M-DCI based MTRP, support at least Opt2 for PUCCH transmission, and Opt1 is not supported - Note: Opt3 and Opt4 are not precluded

RAN1#112

Agreement in RAN1 #112 On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported “RAN2 impact: RRC configuration/parameter per PUCCH resource/resource group” “RAN4 impact: none”

RAN1#111

Agreement On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception - The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception - FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field) - FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0. Above applies for the case where PDSCHs scheduled by the same DCI. “RAN2 impact: Presence of the new DCI field is configurable by RRC” “RAN4 impact: none” Agreement On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group - Note: Detail of the RRC configuration is left to RAN2 design “RAN2 impact: RRC configuration/parameter per PUCCH resource/resource group” “RAN4 impact: none”

RAN1#110b-e

Agreement On unified TCI framework extension for M-DCI based MTRP: - The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value - A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value “RAN2 impact: MAC-CE for TCI state activation” “RAN4 impact: none” Agreement On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following: - Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported) “RAN2 impact: RRC configuration/parameter per CORESET” “RAN4 impact: none” Agreement On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) - For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) “RAN2 impact: MAC-CE for TCI state activation” “RAN4 impact: none”

RAN1#110

RAN1#109e

Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 is endorsed in R1-2205639. “RAN2 impact: none” “RAN4 impact: UE Tx power”