3GPP TSG-RAN WG2 #122	R2-2306233

Incheon, Korea, May 2023

Agenda Item:	7.15.1

Source:	OPPO

Title:	Discussion on R1 LS on MCSt

Document for:	Discussion, Decision

Introduction

This paper is to further discuss resource (re)selection.

Discussion

R1 sent LS to R2 on the resource (re)selection impact due to MCSt.

Q1 as follows

Question 1 (for Approach 1 / Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2)  with the principle of “concatenating” across separate resource selection triggers (across TBs)

In Approach 1/2, the concatenation of resources for multi-TBs are in a best effort / opportunistic manner:

•	Step 3: Higher layer selects a candidate multi-slot resource either randomly (R16/17 behavior) or according to a consecutive-slots criterion (new behavior)  .

•	Step 4: Repeat Step 1-3 for different TB if required.

So basically, it is to say, e.g., the MAC layer firstly selects resources for TB-1, and later selects resources for TB-2, and when selecting resources for TB-2, the resources can be selected in a way to concatenate resources selected for TB-1 when possible.

For Q1 in R1-2304257, R2 replies that it is feasible to concatenate across separate resource selection triggers across TBs in a best-effort manner.

Q2 as follows

Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time

Obviously, it breaks the R2 specification framework, i.e., the SL process-specific resource selection procedure, so not feasible.

For Q2 in R1-2304257, R2 replies that it is infeasible to trigger the resource selection procedures for multiple SL processes at the same time.

Q3 as follows

Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

There could be different angles to this question.

One angle is MAC layer is in a better position to provide such parameters since the MAC layer knows better the traffic characteristic which may affect the PDU number in a burst, and also knows parameters like maximum HARQ retransmission number by reading the upper layer configured CBR table.

The other handle is PHY layer is in a better position to provide such parameters since PHY knows better the resource status, considering finally the number has to be dependent on resource availability.

For Q3 in R1-2304257, R2 further discusses the feasibility.

Conclusion

We have the following proposals:

Proposal 1	For Q1 in R1-2304257, R2 replies that it is feasible to concatenate across separate resource selection triggers across TBs in a best-effort manner.

Proposal 2	For Q2 in R1-2304257, R2 replies that it is infeasible to trigger the resource selection procedures for multiple SL processes at the same time.

Proposal 3	For Q3 in R1-2304257, R2 further discusses the feasibility.

Reference

[1] 3GPP RP-230077, WID revision: NR sidelink evolution, OPPO.

Annex: Draft LS Reply

3GPP TSG-RAN WG2 Meeting #122	R2-230xxxx

Incheon, Korea, May 22-26, 2023

Title:	[DRAFT] Response LS on MCSt resource (re)selection

Response to:	R1-2304257

Release:	Release 18

Work Item:	NR_SL_enh2-Core

Source:	OPPO [TSG RAN WG2]

To:	TSG RAN WG1

Cc:

Contact Person:

#### Name:	Qianxi Lu

E-mail Address:	qianxi.lu@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank RAN1 for their LS on MCSt resource (re)selection in R1-2304257.  RAN2 discussed the approaches described by RAN1 and had the following answers to the questions.

Question 1 (for Approach 1/ Approach 2): feasibility of selecting the resource for a single TB in MAC layer (single-slot under Approach 1, multi-slot under Approach 2) with the principle of “concatenating” across separate resource selection triggers (across TBs)

Response from RAN2:  R2 confirms it is feasible.

Question 2 (for Approach 3): feasibility of triggering the resource selection procedures for multiple SL processes at the same time

Response from RAN2:  R2 discussed this issues and concluded it is infeasible.

Question 3 (Approach 2/ Approach 3): feasibility of providing a new parameter “number of slots for MCSt” to L1 when triggering resource (re-)selection for MCSt

Response from RAN2:  [Pending R2 discussion conclusion]

2. Actions:

To RAN WG1

ACTION: 	RAN2 respectfully asks RAN1 to take the above feedback into account .

3. Date of Next TSG-RAN WG2 Meeting:

RAN2#123			from 2023-08-21 to 2023-08-25 		Toulouse, FR

RAN2#123-bis		from 2023-10-09 to 2023-10-13 		Xiamen, CN