3GPP TSG-RAN WG2 Meeting #122	R2-2306254

Incheon South Korea, May 22nd - 26th 2023	Revision of R2-2304065

Agenda Item:	7.6.3.1

Source:	Ericsson

Title:	Neighbour cell measurements before RLF

Document for:	Discussion, Decision

# 1	Introduction

In Release 18, a work item on IoT NTN was approved in [1] including the objective highlighted below on mobility enhancements:

4.1.2	Mobility enhancements

The following mobility enhancements objectives are listed.

-	Support of neighbour cell measurements and corresponding measurement triggering before RLF, using Rel-17 (TN) NB-IoT, eMTC as a baseline. [RAN2]

-	Re-use the solutions introduced in Rel-17 NR NTN for mobility enhancements for eMTC, with minimum necessary changes to adapt them to eMTC [RAN2]

- Define UE RRM core requirements for the above mobility enhancement features [RAN4].

The following agreements have been made on enhancements for neighbour cell measurements so far:

RAN2#119-e

- IoT NTN can use the mechanism for neighbor cell measurements in connected mode (specified in Rel-17 for NB-IoT). FFS if any enhancements are needed (e.g., triggers) for both NB-IoT and eMTC.

- RAN2 to continue working on a new time-based trigger for triggering intra and inter frequency measurements in connected mode, e.g., the serving cell is going to stop covering the current area, for both earth-moving and earth-fixed cell (FFS on distance-based trigger)

- Measurement results reporting is not supported in Rel-18 NB-IoT NTN.

RAN2#119bis-e

- None

RAN2#120

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation¨

RAN2#121

- Location-based connected mode measurement initiation is supported in quasi-Earth-fixed cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for quasi-Earth-fixed cell. FFS on whether the R17 IEs are reused or not. FFS if the same mechanism can also be used in idle (like in NR-NTN)

- Location-based connected mode measurement initiation is supported in earth-moving cell (UE is not required to update the GNSS location for this). A serving cell reference location and a distance threshold/radius for detecting when to trigger connected mode measurements will be broadcast for earth-moving cell. FFS on whether the R17 IEs are reused or not. FFS on whether additional information needs to be broadcast to inform the UE how the reference location moves over time or if this can be derived from other information (e.g., Epoch time and ephemeris). FFS if the same mechanism can also be used in idle (like in NR-NTN)

- We don’t introduce any new low mobility criterion for enhanced mobility in Rel-18

RAN2#121bis-e

- New SIBxx is introduced to broadcast the neighbor cell/satellite information.

  - Common TA parameters are broadcast as assistance information for neighbor cell measurements.

  - Kmac is broadcast as neighbor cell assistance information.

  - For moving cell, the UE can derive the trajectory of serving cell with rough accuracy based on serving satellite ephemeris and epochTime, with the assumption that the serving cell reference location broadcast by the network is the one at Epoch time (like in NR-NTN)

- Introduce satellite ID for the satellite in a list in new SIB-xx. FFS on the details of the new IE

- For eMTC NTN, for fixed cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN)

- For eMTC NTN, for moving cell, location-based measurement initiation can also be used in RRC_IDLE for cell re-selection purposes (like in NR-NTN). FFS whether to consider solution that does not require UE to update the GNSS for this same as in connected mode

- SIB3 is extended to include the reference location and distanceThreshL

In this contribution we continue the discussion on how neighbour cell measurements can be triggered before RLF so that time to re-establish connection can be shortened for NB-IoT and LTE-M devices in NTN.

# 2	Discussion

## 2.1	Neighbour cell measurements prior to RLF

### 2.1.1	Background

In NB-IoT, handover mechanism is not supported so mobility can only be triggered by RLF in connected mode. This is not the case for LTE-M, which is why this optional mechanism that provides indication from the network for a UE to trigger neighbour cell measurements in order to reduce the time it takes to re-establish RRC connection was specified only for NB-IoT in Rel-17. It is a feature specified for NB-IoTs in terrestrial networks that enhances the RLF/RRC connection re-establishment mechanism with the motivation mentioned above. The RLF/RRC connection re-establishment mechanism is supported by both NB-IoT and LTE-M UEs though.

In Rel-18, the objective is to introduce a mechanism for both NB-IoT and LTE-M UEs in NTN based on the mechanism specified for NB-IoT UEs in TN in Rel-17 with the same intention, i.e., to enhance the RLF/RRC connection re-establishment mechanism. Considering that RLF/RRC re-establishment procedures are supported by both NB-IoT and LTE-M devices, we see no reason why this particular enhancement in Rel-18 should not be specified for both NB-IoT and LTE-M devices provided that legacy measurement procedures are not affected.

Another aspect to consider is the highlighted statements below captured in TS 38.300 for NR NTN devices:

#### 6.14.3.3	Measurements

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities;

-	measurement gaps based on multiple SMTCs;

-	assistance information (e.g., ephemeris, Common TA parameters) provided in SIB19 for UE to perform measurement on neighbour cells in RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED.

NW-controlled adjustment of SMTCs can be based on UE assistance information reported in RRC_CONNECTED. A UE in RRC_IDLE/RRC_INACTIVE can adjust SMTCs based on its location and assistance information in SIB19.

UE assistance information consists of the service link propagation delay difference(s) between serving the cell and neighbour cell(s).

For a UE in Idle/Inactive mode it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell indicated in SIB4 but not included in SIB19.

For a UE in Connected mode, it's up to UE implementation whether to perform NTN neighbour cell measurements on a cell included in the measurement configuration but not included in SIB19.

In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurements on neighbour cells in RRC_IDLE/RRC_INACTIVE:

-	The timing and location information associated to the serving cell is provided in SIB19;

-	Timing information refers to the UTC time when the serving cell stops serving the current geographical area;

-	Location information refers to the reference location of the serving cell and a distance threshold to the reference location.

Measurement rules for cell re-selection based on timing information and location information are specified in clause 5.2.4.2 in TS 38.304 [10].

The text above states that it is up to UE implementation to measure NTN neighbour cells if satellite assistance information for the cell is not provided in SIB19. This applies to both RRC_CONNECTED and RRC_IDLE modes. Thus, if the related content is not available in SIB19, a UE may not perform measurements for the corresponding NTN neighbour cell which would restrict mobility.

A UE may not perform measurements for an NTN neighbour cell if satellite assistance information for the cell is not provided in SIB19.

RAN2 received an LS, i.e., R2-2211171, from RAN4 in RAN2#120. In the LS RAN4 informed RAN2 that the following agreement was reached:

“Similar as NR NTN, the mobility and measurement requirements for IoT NTN apply provided that valid information for the neighbour/target cell is made available to the UE.”

In response, RAN2 stated in an LS to RAN4 that “for neighbour cell measurements, it has been agreed not to introduce the satellite assistance information of neighbour cells in system information in Rel-17 IoT NTN, but this will be supported in Rel-18.” Note that even if such information is introduced satellite assistance information can only be provided for a few neighbour cells, i.e., at most 3 for LTE-M due to the limited TB size that can be supported in the DL.

For IoT NTN, satellite assistance information of neighbour cells is not provided in system information in Rel-17.

Even though satellite assistance information of neighbour cells is to be supported for IoT NTN in Rel-18, the information can be provided only for a limited number of cells, i.e., at most 3, due to TB size limitation.

One can observe that in a TN cell as the UE moves towards the cell border, there is a clear difference in RSRP compared to the cell centre. However, such observation does not seem to be valid in NTN cells [4], where the difference in signal strength is small between two beams in a region where there is overlap.

In NTN, difference in signal strength is small between the cell edge and the cell centre.

Smaller margins would impose tighter measurement thresholds that could lead to triggering neighbour cell measurements unnecessarily or require frequent configuration updates via RRC signalling.

This is yet another reason why this enhancement on RLF/RRC re-establishment mechanism is important for LTE-M devices as much as it does for NB-IoT devices. Therefore, we propose:

Introduce a mechanism to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

### 2.1.2	Triggering criteria for neighbour cell measurements in NTN

In RAN2#120, RAN2 made the following agreements for NB-IoT devices in NTN:

- For NB-IoT we support a trigger for neighbour cell measurements based on T-service (in the quasi-Earth fixed case) (this does not preclude anything for eMTC discussion)

- At least for NB-IoT NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before the t-Service if present. The exact time to start measurements in connected mode before t-Service can be left to UE implementation” (can revisit if we agree other proposal based on neighbour cell coverage)

- RAN2 will not specify the condition of stopping UE measurement before t-Service

- For earth-moving cell, the UE derives when loss of coverage of current cell happens (how to derive this information is FFS)

- For earth-moving cell, UE shall start intra/inter frequency measurements in RRC connected mode before losing coverage. The exact time to start measurements can be left to UE implementation¨

Considering the benefits of time and location-based criteria discussed so far for the quasi-earth-fixed and earth-moving cells, we propose the following:

Introduce time-based criteria, based on T-service, to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

For LTE-M in NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before T-service, if present. The exact time to start measurements in connected mode before T-service is left to UE implementation. FFS for earth-moving cells.

# 3	Conclusions

In this paper we discussed how neighbour cell measurements can be triggered before RLF so that time to re-establish connection can be shortened for NB-IoT and LTE-M devices in NTN. We made the following observations:

Observation 1	A UE may not perform measurements for an NTN neighbour cell if satellite assistance information for the cell is not provided in SIB19.

Observation 2	For IoT NTN, satellite assistance information of neighbour cells is not provided in system information in Rel-17.

Observation 3	Even though satellite assistance information of neighbour cells is to be supported for IoT NTN in Rel-18, the information can be provided only for a limited number of cells, i.e., at most 3, due to TB size limitation.

Observation 4	In NTN, difference in signal strength is small between the cell edge and the cell centre.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce a mechanism to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

Proposal 2	Introduce time-based criteria, based on T-service, to trigger neighbour cell measurements in connected mode for LTE-M in NTN.

Proposal 3	For LTE-M in NTN, for quasi-earth fixed cells, UE shall start intra/inter frequency measurement in connected mode before T-service, if present. The exact time to start measurements in connected mode before T-service is left to UE implementation. FFS for earth-moving cells.

# References

RP-221806, WID on IoT NTN enhancements, MediaTek, RAN#96, June 6-9, 2022

RP-220529, Additional enhancements for NB-IoT and LTE-MTC, Huawei, HiSilicon, RAN#95e, March 2022.

TS 36.331, Radio Resource Control (RRC) protocol specification, V17.1.0, June 2022.

TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.1.0, June 2021.