3GPP TSG-RAN WG2 Meeting #122							R2-2306267

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.12.2.1

Source:	AT&T

Title:	Enhancements for IAB-node mobility and onboard UEs

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Work Item on Mobile IAB for NR [1], 3GPP has agreed to specify enhancements for the mobility of IAB nodes including the following objectives:

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

This contribution discusses solutions for configuring and enabling group mobility for served UEs of a mobile IAB node.

# Mobile IAB and Group Mobility Enhancements

An example of a deployment of vehicle-mounted mobile-IAB-nodes providing 5G coverage/capacity to onboard and/or surrounding UEs is shown in Figure 1:

Figure 1. Mobile IAB with Group Mobility

One of the main advantages of group mobility is the ability to minimize the service interruption during a mobile IAB topology change since the UEs are moving in a correlated manner with the mobile IAB node and also will typically remain connected to the same serving cell of the mobile IAB node after the topology change and handover of the IAB-MT is complete. During RAN2#121bis the following agreements were made with respect to the feasibility and support of RACH-less HO for onboard UEs during an IAB migration procedure:

Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

- Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

- Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

The feasibility of RACH-less HO with the same TA for mobile IAB primarily depends on the beam handling of the target and source cells which are served by the dual DUs of the mobile IAB node during the migration event. Both Option 1 and Option 2 as discussed in RAN2#121bis are feasible. In case of Option 1, once the target DU becomes active, the UE can make measurements of the beams of the new cell and report it to the current serving cell of the source DU using existing beam-based RRM measurements and reports. In case of Option 2, the beamforming configuration can be coordinated internally within the mobile IAB node to ensure the beam indices remain identical before and after the RACH-less HO.

Proposal 1: RACH-less HO with same TA with security key change is supported for served UEs by using either Option 1 or Option 2 for beam handling during the mIAB DU migration.

Additionally, during RAN2#119bis the following agreement was made:

Regarding moving status/mode indication, R2 observes that legacy reporting of mobility state (e.g. mobilityState-r16) could be reused, and maybe also current location reporting from the UE. FFS whether any of this need to be enhanced or complemented, e.g. for the potential purpose of predictive mobility.

One additional enhancement discussed in RAN2 was the utilization of a Rel-18 NTN feature to trigger CHO based on a configured time value:

- Based on RAN2 agreement below, discuss whether and how time-based CHO execution defined for NTN can be easily adopted  for UEs during DU migration:

FFS: May support CHO with CondT1 if it is “for free”, i.e. if TS impact is just to slightly modify the description to make it also applicable to TN.

This feature is particularly attractive for Mobile IAB since it enables the dual DUs to efficiently manage the radio resources by migrating the onboard UEs “in waves,” without a large signaling storm. In addition, the value of CondT1 can be set based on the proportion of Rel-18 UEs which support this CHO feature compared to Rel-17 and earlier UEs in order to mitigate any potential user experience impact.

Proposal 2: RAN2 should support CHO with CondT1 for onboard Rel-18 UEs during a migration event between cells of a serving mobile IAB node.

# Conclusion

This contribution discussed solutions for configuring and enabling group mobility of an IAB node together with its served UEs. The following proposals were made:

Proposal 1: RACH-less HO with same TA with security key change is supported for served UEs by using either Option 1 or Option 2 for beam handling during the mIAB DU migration.

Proposal 2: RAN2 should support CHO with CondT1 for onboard Rel-18 UEs during a migration event between cells of a serving mobile IAB node.

# Reference

[1] RP-221815 New WID on Mobile IAB