3GPP TSG-RAN WG2 Meeting #122	R2-2306291

Incheon, Korea, 22nd May– 26th May, 2023

Agenda item:	7.13.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on MRO for inter-system handover for voice fallback

Document for:	Discussion and Decision

- 1. Introduction

In RAN2#121bis meeting, the following agreements and remaining issues were made with respect to MRO for inter-system handover for voice fallback:

Agreements:

1	RAN2 to support the scenario of “after RLF occurs shortly after successful HO from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell, which is agreed in RAN3”.

2	FFS: Introduce an indication for the scenario of RLF after successful voice fallback HO in the LTE RLF report regarding voice fallback.

3	UE logs the agreed indication regarding voice fallback in the NR RLF report.

4	FFS: RAN2 agree to differentiate an acceptable E-UTRA cell from a suitable E-UTRA cell in the RLF report in case of voiceFallback HOF. FFS explicit or implicit indications.

In this contribution, for the agreed scenarios and FFSs, we would like to provide further insights on potential issues and propose solutions accordingly.

- 2. Discussion

  - Enhancements to support shortly RLF after successful voice fallback

RAN3#117bis meeting has confirmed the following cases for MRO for voice fallback：

- Case 1: after failure (HOF/RLF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

-	Case 2: after failure (HOF) of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

From RAN2 perspective, we particularly analyse the specification impact for support of case 1. We believe case 1 should be divided into the following two sub-cases, since the two sub-cases have different requirements of RLF report format and forwarding:

Case 1a: handover failure from source cell1 to E-UTRA cell1 occurs, a suitable E-UTRA cell2 is selected;

Case 1b: handover from NG-RAN cell1 to E-UTRA cell 1 succeeds but shortly RLF in E-UTRA cell1 occurs, a suitable E-UTRA cell2 is selected.

At the time of connection failure, the last serving node in case 1a and case 1b is NR RAN node and E-UTRAN node, respectively. As a result, it should be noticed that UE records NR RLF report in case 1a and LTE RLF report in case 1b, respectively. That means, in order to support case 1b, the LTE RLF report should also include the explicit indication of voice fallback to enable the MRO analysis performed by the NR node.

Observation 1: To support case 1b (RLF in E-UTRA cell after successful voice fallback), there will be LTE specification impact on TS36.331.

Proposal 1: RAN2 consider that the UE logs the indication of voice fallback in LTE RLF report, which is similar to that in NR RLF report, in order to support case 1b (RLF in E-UTRA cell after successful voice fallback).

  - Voice fallback triggered by Emergency call

According to section 5.2.8 in TS 38.304, for emergency calls, the UE shall perform the cell selection/reselection to an acceptable cell if no suitable cell is found.

5.2.8 Camped on Any Cell state This state is only applicable for RRC_IDLE state. In this state, the UE shall perform the following tasks: [……] - if the UE supports voice services, the UE is not in SNPN access mode, and the current cell does not support IMS emergency calls as indicated by the field ims-EmergencySupport in SIB1 as specified in TS 38.331 [3], the UE shall perform cell selection/reselection to an acceptable cell that supports emergency calls in any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found. - if the UE supports voice services, the UE is in SNPN access mode, and the current cell does not support IMS emergency calls for any SNPN(s) as indicated by the field imsEmergencySupportForSNPN in SIB1 as specified in TS 38.331 [3], the UE shall perform cell selection/reselection to an acceptable cell of any available SNPN that supports emergency calls, if no suitable cell is found.

In combination with section 5.4.3.5 in TS 38.331, when the HOF occurs during the voice fallback procedure, the UE attempts to select a suitable E-UTRA cell first. If the voice fallback is triggered for the emergency call and no suitable E-UTRA cell is found, the UE will attempt to select an acceptable E-UTRA cell. If neither is found, UE reverts back to the configuration used in the source PCell.

In voice fallback case for the purpose of emergency call service, regardless that UE selects a suitable or an acceptable E-UTRA cell, the UE enters idle and performs setup procedure towards the selected suitable/acceptable E-UTRA cell. When the UE comes back to connected with the selected suitable/acceptable E-UTRA cell, the UE logs the selected suitable/acceptable E-UTRA cell in the eutraReconnectCellId of the IE reconnectCellId.

Observation 2: The UE can set the eutraReconnectCellId of the IE reconnectCellId to the suitable E-UTRA cell or the acceptable one for emergency voice fallback.

Fig. emergency voice fallback handover

As showed in the figure, HOF occurs when the UE is handed over to E-UTRA target cell 1 due to emergency voice service. The UE cannot find any suitable E-UTRA cell upon occurrence of HOF. Finally, the UE selects the acceptable E-UTRA cell 2 and successfully completes the connection setup procedure. The UE logs the E-UTRA cell 2 into the IE eutraReconnectCellId of the IE reconnectCellId in the RLF report. Technically speaking, except for emergency call, the E-UTRA cell 2 cannot be taken as the proper E-UTRA target cell for next EPS fallback triggered by normal IMS voice call.

Observation 3: The acceptable E-UTRA cell cannot be used as proper target cell for voice fallback triggered by normal IMS voice call.

However, based on the existing information provided in RLF report, the network cannot distinguish the suitable cell from acceptable one.  Consequently, the network may wrongly consider the acceptable E-UTRA cell as the potential appropriate target one and make wrong optimization. Thus, we think it is beneficial to indicate whether the cell in the IE eutraReconnectCellId is an acceptable E-UTRA cell or not.

Proposal 2: Introduce the indication of whether the reconnected E-UTRA cell is acceptable E-UTRA cell or not.

- 3. Conclusion

In this paper, we discuss miscellaneous enhancements, and we have the following observations and proposals:

Observation 1: To support case 1b (RLF in E-UTRA cell after successful voice fallback), there will be LTE specification impact on TS36.331.

Proposal 1: RAN2 consider that the UE logs the indication of voice fallback in LTE RLF report, which is similar to that in NR RLF report, in order to support case 1b (RLF in E-UTRA cell after successful voice fallback).

Observation 2: The UE can set the eutraReconnectCellId of the IE reconnectCellId to the suitable E-UTRA cell or the acceptable one for emergency voice fallback.

Observation 3: The acceptable E-UTRA cell cannot be used as proper target cell for voice fallback triggered by normal IMS voice call.

Proposal 1: If case 1b is confirmed by RAN2, the indication of voice fallback should be introduced to the LTE RLF report.

Proposal 2: Introduce the indication of whether the reconnected E-UTRA cell is acceptable E-UTRA cell or not.