3GPP TSG-RAN WG2 Meeting #122	R2-2306292

Incheon, Korea, 22nd May– 26th May, 2023

Title: 	SHR and SPCR

Source: 	Huawei, HiSilicon

Agenda item:	7.13.4

Document Type:	discussion

Introduction

In previous RAN2 meetings, there were some conclusions on both inter-RAT SHR and SPR. There are still some FFSs left. The agreements in RAN2 are listed below:

RAN2 agreements on inter-RAT SHR:

Agreements in RAN2#121 meeting:

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

5:  A new EUTRA target cell CGI is introduced in inter-RAT SHR.

6: For HO from NR to LTE, the T310 and T312 threshold is provided to the UE by source gNB in the otherConfig.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

RAN2 agreements on SPR:

Agreements in RAN2#119bis-e meeting

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

5c: Which node configures the triggering condition is FFS.

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

Agreements in RAN2#120 meeting:

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

There was a LS from RAN3 [1] for RAN2 to consider inter-RAT SHR for HO from LTE to NR.

In this paper, we will give our considerations on the above FFSs for intra-system inter-RAT SHR and NR-DC SPR.

Discussion

Intra-system inter-RAT SHR

In the LS [1], RAN3 has agreed that when the reception node fetches SHR from UE, it directly forwards it to the node which configured the SHR triggers that triggered the SHR.

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

That is, for inter-RAT HO from NR to LTE, the source NR node configures T310/312 triggers. If inter-SHR is generated and reported, the receiving node will forward it to the source NR node. Likely, for inter-RAT HO from LTE to NR, the target NR node configures T304 trigger. If inter-RAT SHR is reported, the receiving node forwards it to the target NR node. Therefore, there is no UE context retrieval between the target node and source node, which is different from the forwarding scheme of RLF report.

Observation 1: For inter-RAT SHR, the reception node directly forwards it to source NR node for inter-RAT HO from NR to LTE while to the target NR node for inter-RAT HO from LTE to NR, without UE context delivery between network nodes.

Intra-system inter-RAT SHR from NR to LTE

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

- C-RNTI (FFS target or source)

- FFS: Time between report generating and fetching

As mention in the above section, for R18 inter-RAT SHR from NR to LTE, the NR reception node will forward the NR SHR to the source NR node which generates the SHR T310/312 triggers that triggered the inter-RAT SHR. To assist the source NR node to identify the UE, an intuitive idea is to introduce the source NR C-RNT into the NR SHR.

Proposal 1: Introduce source C-RNTI in inter-RAT SHR from NR to LTE.

It is possible that the source NR C-RNTI may be reused in the source NR node later. For example, when the UE connects the network, it is allocated with C-RNTI a. If the UE leaves the network, the C-RNTI a is released and can be reassign to the other UE. Therefore, only the C-RNTI information is impossible for the network to know the UE information.

Observation 2: C-RNTI can be re-allocated if released.

Consequently, it is reasonable to include the time information to further help the source NR node to uniquely identify the UE. With the time information and the C-RNTI related that time point in the NR SHR, the source NR node is able to uniquely identify the UE and performs potential optimization for mobility parameters. For the time information, one option is to indicate the time elapsed since the HO command until SHR fetching. The alternative option is to indicate the time between report generating and fetching. The difference is about the starting point of the time information. For option 2, generation of the SHR depends on the completion of the successful RACH procedure with target cell. The duration of RACH procedure seems dynamic and correlates with the numbers of preamble transmission. When the UE1 performs the RACH procedure for inter-RAT HO from NR to LTE, the source NR node has released the related C-RNTI. It is possible that the C-RNTI is reused for a new UE2 and for some reason, e.g., voice fallback, the new UE2 is handed over to other cell when the previous UE1 with the same C-RNTI performs RACH with the target LTE cell. For this case, during the RACH procedure of the previous UE1, one C-RNTI may correlate two UEs. In this way, the network cannot uniquely identify the UE with the time information in option 2.

With the above analysis in mind, we believe option 1 is more reasonable where the C-RNTI corresponds to one UE.

Proposal 2: Introduce time between reception of HO Command and inter-RAT SHR fetching in inter-RAT SHR from NR to LTE.

Intra-system inter-RAT SHR from LTE to NR

There was a LS from RAN3 [1] for RAN2 to consider inter-RAT SHR for HO from LTE to NR. For this direction, only T304 trigger is considered. Based on our analysis in [2], it seems that some small LTE impacts are introduced. If RAN3 agreed to work on this topic, we think that RAN2 can further discuss this scenario and give the potential enhancement discussion.

Meanwhile, in the LS, RAN3 suggested two IEs to be reported from UE for the inter-RAT SHR from LTE to NR. The related information is listed below:

- Source LTE cell identity

- Target NR cell identity

It is an intuitive idea to include the two IEs into the existing SHR to support the inter-RAT HO from LTE to NR. For the first bullet, a new EUTRA source cell CGI should be introduced while for the bullet 2, we can directly reuse the existing target NR cell information.

For the inter-RAT SHR from NR to LTE, the following information was agreed:

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

For bullets c and e, both can also be applied to the inter-RAT SHR from LTE to NR. For bullet d, we can also reuse the existing IE defined in R17 to indicate T304 trigger for inter-RAT SHR from LTE to NR.

Moreover, as the aforesaid section 2.1.1, similar enhancements should be introduced. In other words, the target C-RNTI and the time information should be reported for inter-RAT SHR from LTE to NR.

Based on the above discussion, we summarize the potential enhancements for inter-RAT SHR from LTE to NR:

Proposal 3: For inter-RAT SHR from LTE to NR, below parameters is stored, reuse the existing IEs defined in R17 for intra-NR SHR:

- target NR cell information

- target NR C-RNTI

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition (T304) was met

e.	UE location Information

Proposal 4: A new EUTRA source cell CGI is introduced in inter-RAT SHR from LTE to NR.

Proposal 5: A new time information to indicate the time between HO command and SHR fetching in inter-RAT SHR from LTE to NR.

NR-DC SPR

There were some FFSs on the SPR scheme. We’d like to give our consideration here.

5	Network configures SPR configuration IE for the UE, with at least the following triggering conditions:

•	T310 triggering condition

•	T312 triggering condition

•	T304 triggering condition

5a: Other triggering conditions are FFS

5b: Values of the triggering conditions are FFS

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

a)	Source PSCell info (cell ID, measurement result)

b)	Target PScell info (cell ID, measurement result)

c)	Neighbour Cells info (cell ID, measurement result, CPAC Candidate cells flag)

d)	Success PSCell change/addition cause value (e.g., t304, t310, t312 cause, etc.)

f)	The time elapsed between the CPAC execution towards the target cell and the corresponding latest CPAC configuration received for the selected target cell

Agreements in RAN2#120 meeting:

4	Random access related information is included in SPR at least when the SPR is triggered due to T304 exceeds the configured threshold. Other conditions are FFS.

5a: Other triggering conditions are FFS

For Rel-17 SHR, it is worth noting that lots of triggering conditions were discussed. After fully discussion, T304/310/312 and DAPS related triggers are introduced. For the SPR, there is no DPAS configuration. Therefore, we believe the current agreements on T304/310/312 triggers are sufficient. The discussion on other triggering conditions is redundant and not needed.

Proposal 6: No other triggering condition is needed.

5b: Values of the triggering conditions are FFS

Current, the values of the SHR triggering conditions are relative thresholds in form of percentage. The UE compares the ratio of the elapsed time and the corresponding timer with the configured percentage threshold to decide whether to generate the SHR.

SuccessHO-Config-r17 ::=                SEQUENCE {

thresholdPercentageT304-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT310-r17             ENUMERATED {p40, p60, p80, spare5, spare4, spare3, spare2, spare1}      OPTIONAL, --Need R

thresholdPercentageT312-r17             ENUMERATED {p20, p40, p60, p80, spare4, spare3, spare2, spare1}         OPTIONAL, --Need R

sourceDAPS-FailureReporting-r17         ENUMERATED {true}                                                       OPTIONAL, --Need R

...

It is worth noting that the principle of SPR is similar to SHR and the SPR triggers are also based on the ones in SHR. Technically speaking, it is an intuitive idea to represent the triggers of SPR in term of percentage similar to SHR. Furthermore, it is also desirable to apply the defined values of SHR triggers for SPR triggers.

Proposal 7: SPR triggering conditions are configured in term of percentage and reuse the values defined for SHR.

7	UE logs at least the following information and measurements in the SPR IE (other information and measurements are FFS).

RAN3 has the following agreements on SPR forwarding scheme and the required information to be reported in SPR:

In case the SPR is retrieved in a “new node” (different from the node that sent the SPR configuration to the UE i.e., “old MN”), the SPR is always sent from the “new node” to the “old MN” which then forwards to the respective node(s) which should perform the SPR optimization.

To assist in the forwarding of SPR, UE may include the following in SPR

- CGI of the PCell which sent the SPR configuration (presence of this IE is to be discussed)

- WA: Indication whether the PSCell change was MN-initiated or SN-initiated (RAN3 should discuss how the UE knows whether the PSCell change as MN-initiated or SN-initiated and will check with RAN2 on the mechanism)

Based on the progress in RAN3, it is obvious to conclude that the CGI of PCell should be mandatory:

Proposal 8: The UE logs the PCell information where the SPR configuration is sent.

Generally, when the old MN receives the SPR of PSCell change cases, it should know whether the PSCell change was initiated by the itself or source SN before performing root cause analysis. For CPC case, there are different fields in the RRC Reconfiguration message for the execution conditions from MN and source SN. In this way, the UE can realize whether the CPC is initiated by MN or SN.

CondReconfigToAddMod-r16 ::=     SEQUENCE {

condReconfigId-r16               CondReconfigId-r16,

condExecutionCond-r16            SEQUENCE (SIZE (1..2)) OF MeasId                      OPTIONAL,    -- Need M

condRRCReconfig-r16              OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Cond condReconfigAdd

...,

[[

condExecutionCondSCG-r17         OCTET STRING (CONTAINING CondReconfigExecCondSCG-r17) OPTIONAL     -- Need M

]]

}

For classic inter-SN PSCell change, irrespective of MN-initiated or SN-initiated cases, the target SN generates the configuration container and forwards it to the MN. The MN transparently delivers it to the UE. For UE point of view, we believe it is impossible to differ the two cases. To assist the UE to know the initiating node for classic PSCell change, it is straightforward that an explicit indicator can be included in the RRC Reconfiguration message to inform UE whether the PSCell change is initiated by MN or source SN. Correspondingly, the UE can include the indicator in SPR to assist the MN to know whether the PSCell change was initiated by MN or source SN.

Proposal 9: Network sends the indicator to inform UE whether MN initiates the PSCell change.

Proposal 10: The UE logs indicator whether the PSCell change was MN-/SN-initiated.

Other conditions are FFS.

Technically speaking, for successful SPR, the UE will record the Random access related information in the SN RACH report. And RAN2 has agreed to include the Random access related information in case of T304 trigger. We cannot see any benefits to include the duplicated Random access related information for any other conditions.

Proposal 11: Random access related information is not included for any other conditions.

Conclusion

For Intra-system inter-RAT SHR, we have the following observations and proposals:

Observation 1: For inter-RAT SHR, the reception node directly forwards it to source NR node for inter-RAT HO from NR to LTE while to the target NR node for inter-RAT HO from LTE to NR, without UE context delivery between network nodes.

Observation 2: C-RNTI can be re-allocated if released.

Proposal 1: Introduce source C-RNTI in inter-RAT SHR from NR to LTE.

Proposal 2: Introduce time between reception of HO Command and inter-RAT SHR fetching in inter-RAT SHR from NR to LTE.

Proposal 3: For inter-RAT SHR from LTE to NR, below parameters is stored, reuse the existing IEs defined in R17 for intra-NR SHR:

- target NR cell information

- target NR C-RNTI

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition (T304) was met

e.	UE location Information

Proposal 4: A new EUTRA source cell CGI is introduced in inter-RAT SHR from LTE to NR.

Proposal 5: A new time information to indicate the time between HO command and SHR fetching in inter-RAT SHR from LTE to NR.

For NR-DC SPR, we have the following observations and proposals:

Proposal 6: No other triggering condition is needed.

Proposal 7: SPR triggering conditions are configured in term of percentage and reuse the values defined for SHR.

Proposal 8: The UE logs the PCell information where the SPR configuration is sent.

Proposal 9: Network sends the indicator to inform UE whether MN initiates the PSCell change.

Proposal 10: The UE logs indicator whether the PSCell change was MN-/SN-initiated.

Proposal 11: Random access related information is not included for any other conditions.

Reference

- R3-232140, LS on inter-RAT SHR and SPR