3GPP TSG-RAN WG2 Meeting #122	R2-2306305

Inchon Korea 22th -26th May, 2023

Source: 		   ZTE Corporation, Sanechips

Title:            Remaining issues on the FDM Solutions Enhancement

Agenda item:	7.10.2

Document for: 	Discussion and Decision

- Introduction

In the offline discussion of [Post121][655][IDC] Discussion on Leftover issues for IDC (xiaomi), several topics were discussed for the FDM solutions and companies have different views on the questions below:

Question 10: Do you think that the Rel-16 IDC ASN.1 framework of adding interferenceDirection and victimSystemType is reused for the Rel-18 IDC FDM assistance information? Question 11: Do you think that the value "whole" for affectedBandwidth-r18 and candidateBandwidth-r18 is needed? Question 12: Do you think that candidateBandwidth-r18 can be optional for the FDM configuration from the network, e.g. if the value "whole" for candidateBandwidth-r18 is not applicable?

In this paper, we share our further views on these topics.

- Discussion

2.1 victimSystem Type

During the offline discussion, there is a question on why there is no victim type for the single carrier case, in this chapter we’d like to share our understanding.

AffectedCarrierFreqRange-r18 ::=     SEQUENCE {

centerFreq-r18                 ARFCN-ValueNR,

affectedBandwidth-r18          ENUMERATED {mhz5, mhz10, mhz20, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100, mhz200, mhz300, mhz400, whole},

interferenceDirection-r18       ENUMERATED {nr, other, both, spare}

}

AffectedCarrierFreqRangeComb-r18 ::=   SEQUENCE {

affectedCarrierFreqRangeComb-r18         SEQUENCE (SIZE (2..FFS)) OF AffectedCarrierFreqRangeComb-r18    OPTIONAL,

victimSystemType-r18                     VictimSystemType-r16

}

In the LTE the victimSystemType was added for the UL CA case based on the RAN4 LS[1] as below:

RAN4 also realized that additional information of the victim system is certainly beneficial in order to minimize degradation of UL CA e.g. eNB can determine UL exact PRB combinations not causing interference. However, how to define the detailed signaling procedure is up to RAN2.

From the above description we can see that the victim type was used to give more exact frequency range information to the network.

Observation 1: The victim type was introduced based on RAN4 LS[1] with intention to give more exact frequency range information to the network.

However for the single carrier, with the FDM enhancement, the UE can report the actual affected frequency ranges to the network, so the Victim type seems not necessary. Furthermore for the single carrier case, the 3GPP carrier can be an aggressor or victim, then for the case with interferenceDirection-r18 = both, the aggressor or victim type maybe different, which would also increase the complexity. For the IMD interference, the actual affected frequency ranges was quite different. Take the Fig1 and Table 1 as an example, the IMD interference caused by frequency range combination of (1710~1785) and (4400~5000) include 2 parts, the first part is (2615~3290) and the second part is (6110~6785). With the victim type information, at least the network can further determine which part has interference to the non-3gpp system and then can adopt the corresponding mechanism.

Table 1: 2nd order IMD frequency limits

| Band | n3 |  | n79 |  |
| --- | --- | --- | --- | --- |
| UL frequency | fx_low: 1710 | fx_high:1785 | fy_low:4400 | fy_high:5000 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 2615 | 3290 | 6110 | 6785 |

Fig 2:2nd order IMD frequency Range

Proposal 1: The Rel-16 IDC ASN.1 framework of adding interferenceDirection and victimSystemType is reused (i.e. interferenceDirection for the single carrier and victimSystemType for the IMD) for the Rel-18 IDC FDM assistance information.

2.2 Bandwidth

For the bandwidth, based on the questions listed in the offline discussion, the key issue is whether the candidateBandwidth-r18 can be optional. Generally, there are 2 different understandings:

- Optional, and absence means whole frequency range.

- Mandatory, and the network can configure a maximum bandwidth for the case with whole frequency range

Obviously, there isn’t essential difference between these 2 options while the option 2 has less spec impact so the option 2 were preferred. Then for the maximum bandwidth setting, it can be left to the network implementation (e.g. based on the UE capability), no further specification work needed.

Proposal 2: The candidateBandwidth-r18 is mandatory, network can configure a maximum bandwidth for the case with whole frequency range.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: The victim type was introduced based on RAN4 LS [1] with intention to give more exact frequency range information to the network.

Proposal 1: The Rel-16 IDC ASN.1 framework of adding interferenceDirection and victimSystemType is reused (i.e. interferenceDirection for the single carrier and victimSystemType for the IMD) for the Rel-18 IDC FDM assistance information.

Proposal 2: The candidateBandwidth-r18 is mandatory, network can configure a maximum bandwidth for the case with whole frequency range.

- Reference

[1] R2-153048	Reply LS on 2UL interband CA problem of GNSS (Contact: DCM)