3GPP TSG-RAN WG2 Meeting #122	R2-2306313

Incheon, South Korea, 22– 26 May 2023

Agenda item:	7.9.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on multipath SL relay

WID/SID:	NR_SL_relay_enh-Core

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#121bis-e, RAN2 discussed the multi-path sidelink relay issues based on an input document from the rapporteur in [R2-2303857]. This contribution presents our views on the remaining FFS parts.

The agreements from the last meeting are captured as below:

Agreements:

The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition.

In case of duplication, PDCP control PDU only transmits on the primary RLC entity same as legacy.

Agreements:

Non-split SRB1 and 2 over indirect path is not supported in Scenario 2.

Split SRB1 and 2 are supported in Scenario 2 and primary path of the split SRB 1 and 2 is always on direct path.

If UE-UE link failure is detected on indirect path in Scenario 2, the remote UE can report UE-UE link failure to gNB over direct path.  Details of the reporting mechanism can be further discussed.

Agreements:

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2.

When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2.

-	when primary RLC entity of split SRB1 is on direct path

-	when non-split SRB1 is configured on direct path

Agreements:

The bearer type configuration is provided per SRB.  It is up to network implementation whether to configure SRB1 and SRB2 with same or different bearer types (within the bearer types that are supported).

FFS if there are cases where the configuration of non-split SRBs over indirect path is useful.

Agreements:

In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

Agreements:

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.  FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.  This agreement does not imply any conclusion on non-split SRB1 on indirect path.

A remote UE in multipath that is released to RRC_IDLE/RRC_INACTIVE can apply legacy cell/relay selection behaviour, thus moving to single-path operation on either path according to implementation.

# 2	Discussions

## 2.1	Where to send RRCReconfigurationComplete message.

In RAN2#121, it was agreed that:

Agreements:

For bringing the idle/inactive relay UE to RRC_CONNECTED, the legacy Rel-17 behaviour (Alt 1 in the proposal) is not disabled for indirect path addition when split SRB1 is configured.  A PC5-RRC trigger is specified at least for other cases.

FFS if a Rel-17 relay UE is supported for use with multi-path and how the above agreement is reflected in such a case.

Regarding this, one issue would be where to send the RRCReconfigurationComplete if the target relay UE is already in RRC_CONNECTED.

If the target relay UE is in RRC_CONNECTED, it is sensible to send the RRCReconfigurationComplete message via the direct path because there is no need of initiating RRC setup or resume procedure in the relay UE side. Thus, the direct path should be the baseline. Sending it to the indirect path introduces unnecessary delay because the target relay UE can send the RRCReconfigurationComplete message to the gNB only after the PC5 RRC connection is established and SL-RLC1 is reconfigured, for which another RRC reconfiguration procedure for the relay UE is required. One may consider it is simple to transmit the RRCReconfigurationComplete to the indirect path because the path is always fixed to the indirect path. In our view, however, it is not a good design principle to simplify the behaviour by sacrificing the performance, i.e., in terms of latency.

Observation 1: If the target relay UE is in RRC_CONNECTED, sending the RRCReconfigurationComplete message via the indirect path results in undesirable delay.

Proposal 1: If the target relay UE is in RRC_CONNECTED, the remote UE sends the RRCReconfigurationComplete message via the direct path.

In RAN2#121bis-e, RAN2 agreed that the remote UE sends the RRCReconfigurationComplete message to the direct path when (1) primary path of split SRB1 is on direct path or (2) non-split SRB1 is configured on direct path. Thus, to send the RRCReconfigurationComplete message to either the direct path or the indirect path depending on the target relay UE’s RRC state, one possible way is that the gNB sets the primary path properly. For instance, when adding an indirect path, the gNB sets the primary path of the split SRB0 to the direct path if the target relay UE is in RRC_CONNECTED. If the target relay UE is not in RRC_CONNECTED, the gNB sets the primary path of the split SRB0 to the indirect path.

Observation 2: The gNB can set the primary path depending on the RRC state of the target relay UE.

However, the gNB may not want to determine the primary path based on the target relay UE’s RRC state but rather based on e.g., the link quality. In this case, the gNB may need to indicate the path to send the RRCReconfigurationComplete on top of the primary path. Alternatively, the gNB may provide the RRC state of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message to the intended path according to the target relay UE’s RRC state.

Observation 3: In order not to associate the primary path and target relay UE’s RRC state, the gNB needs to indicate either the path to send the RRCReconfigurationComplete message explicitly or the RRC state of the target relay UE to the remote UE.

With the observations 2 and 3, we propose,

Proposal 2. To send the RRCReconfigurationComplete message to either a direct or an indirect path depending on the target relay UE’s RRC state, RAN2 discuss the following options:

- Option 1. The gNB sets the primary path of the split SRB0 based on the RRC state of the target relay UE.

- Option 2. The gNB configures a path for the remote UE to send the RRCReconfigurationComplete message via separate parameter than the primary path.

- Option 3. The gNB indicates the RRC state of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message regardless of the primary path.

## 2.2	Support of non-split SRB on the indirect path

It is FFS if there are cases where the configuration of non-split SRBs over indirect path is useful - It is useful when the indirect path quality is better than the direct path. Since we assume a single gNB in Rel-18, there is no technical issue that prevents configuring a non-split SRB1 on the indirect path but it may be related to modeling of the multi-path (MP) relay operation.

If the Dual Connectivity (DC) concept is applied to the MP relay operation, each path would belong to different cell groups. Thus, configuring the non-split SRB1 on the indirect path and the PCell on the direct path would imply that the SRB1 and the PCell belong to the different cell groups. This was not possible in DC since the UE has a single RRC connection based on the MN RRC and control of MN RRC is done via MCG SRB, e.g., SRB1. In this sense, taking the DC concept for the MP relay operation, non-split SRB1 can only be supported on the direct path, which reverts the RAN2 agreement in RAN2#119.

In Carrier Aggregation (CA) concept, on the other hand, both of direct and indirect paths would be considered part of the same cell group. Therefore, even if non-split SRB1 is configured on the indirect path while having the PCell on the direct, we can still consider them to belong to the same cell group. Accordingly, non-split SRB1 on the direct path can still be supported despite the PCell is on the direct path for the MP relay operation.

In Rel-18, DC or CA modelling would not make a big technical difference because single gNB is assumed. Using a DC model in Rel-18, however, may result in unoptimized operation because DC is basically designed for different gNBs case. In the meanwhile, using a CA model in Rel-18 may bring a forward compatibility issue when supporting the MP relay operation with two different gNBs in the future.

In our view, we have not identified any technical issue that requires reverting the previous agreement. Thus, we propose to keep the agreement as it is, i.e., to support non-split SRB on the indirect path.

Proposal 3: RAN2 confirm the agreement on the support of non-split SRB on the indirect path.

## 2.3	When the remote UE goes to RRC_IDLE/INACTIVE.

RAN2 discussed in which cases the relay UE goes to RRC_IDLE/INACTIVE while the remote UE is in RRC_CONNECTED, and how the multi-path can be handled based on [R2-2304303], and made some agreements:

WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE.

A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment.  FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE.  This agreement does not imply any conclusion on non-split SRB1 on indirect path.

Issue 1. Working assumption when the network explicitly releases the relay UE

The intent of the working assumption is that if the network is to send the relay UE to RRC_IDLE/INACTIVE, the relay UE doesn’t need to inform this to the remote UE because the necessary action can be done by the network, e.g., the network can release the multipath configuration related to the relay UE at the remote UE. To resolve the concerns that the WA may mandate the network behaviour, we can elaborate the WA as follows:

Modified WA: For a remote UE and a relay UE in RRC_CONNECTED, if the network releases the relay UE to RRC_IDLE or RRC_INACTIVE, the relay UE does not need to inform the remote UE of explicit release of RRC connection. It is up to network how to handle the multipath configuration related to this relay UE.

Proposal 4: RAN2 consider agreeing on the modified WA that For a remote UE and a relay UE in RRC_CONNECTED, if the network releases the relay UE to RRC_IDLE or RRC_INACTIVE, the relay UE does not need to inform the remote UE of its explicit release of RRC connection. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

Issue 2. When the relay UE performs handover

In Rel-17, upon reconfiguration with sync, the relay UE informs the remote UE of its handover via Notification message so that the remote UE initiates the RRC connection re-establishment because the gNB cannot reach the remote UE directly. However, in Rel-18 multi-path, the gNB can control the remote UE via the direct path or via the indirect path before performing handover of the relay UE. Thus, the relay UE doesn’t need to inform the remote UE of its handover. The handover case can be handled in the same way as explicit release of RRC connection of the relay UE in the issue 1.

Proposal 5: For a remote UE and a relay UE in RRC_CONNECTED, the relay UE does not inform the remote UE of its reception of an RRCReconfiguration including the reconfigurationWithSync. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

Issue 3. DataInactivityTimer

In Rel-14, DataInactivityTimer has been introduced to resolve the RRC state mismatch between the UE and the gNB when the RRCRelease is lost. The expiry of dataInactivityTimer allows the UE to go to RRC_IDLE by itself assuming that the gNB already considers the UE left RRC_CONNECTED. It means that, if dataInactivityTimer expires at the relay UE, it should be assumed that the gNB already sends the RRCRelease explicitly to the relay UE. In WA, RAN2 agreed that if the relay UE is explicitly released by the network, the network is expected to take some action to release the multi-path configuration of the concerned relay UE at the remote UE. Thus, it is likely that the remote UE is reconfigured by the gNB before expiry of DataInactivityTimer in the relay UE side. With this reason, it is not necessary for the relay UE to inform the remote UE of its expiry of dataInactivityTimer.

In addition, as per TS 38.321, the start or restart of dataInactivityTimer does not consider the MAC SDU reception or transmission over STCH/SCCH, meaning that the relay UE may unintentionally leave RRC_CONNECTED while receiving data from the remote UE but not sending data to the gNB. Thus, sensible implementation would be not to configure dataInactivityTimer to the relay UE, hence, further defining the UE behaviour for dataInactivityTimer would be considered as unnecessary optimization.

Proposal 6: For a remote UE and a relay UE in RRC_CONNECTED, the relay UE does not inform the remote UE of its expiry of dataInactivityTimer, if configured. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

## 2.4	RLF report

In MP relay operation, if the failure is detected on the indirect path, it is due to either PC5 link or Uu link of the relay UE. Thus, it may be beneficial to inform which link of the indirect path is problematic so that the gNB fixes the problematic link individually.

In addition, when the failure report is transmitted to the gNB, it is for fast link recovery, which can be either recovery of the failed path so that MP relay operation is continued or to revert to a single path operation. To facilitate the gNB decision, it would be beneficial for the gNB to acquire the latest buffer size information as well as the link quality since relying on the periodic or retransmission BSR may not be sufficient for the gNB to acquire the latest buffer size information at an appropriate time.

Proposal 7: RAN2 consider that buffer size information, problematic link/interface indication, and link quality to be reported when reporting the RLF.

In principle, the failure report is for fast link recovery and the gNB can even decide not to recover but to release the problematic path, in which case failure report is actually not necessary because the UE can still use the alive path even without releasing the problematic path. In this sense, we see some benefits of not reporting the RLF even if split SRB1 is available on one path while the RLF is detected on another path. Skipping RLF report based on the pre-configured condition would allow not performing unnecessary failure recovery procedure, which includes a couple of steps that may interfere continuous data transmission/reception on the alive path with no benefit.

Proposal 8: RAN2 discuss if the gNB can configure a condition not to send the RLF report even if the UE detects the RLF on one path while another path can be used for transmission of the RLF report.

## 2.5	Indirect path addition in Scenario 2

In RAN2#121bis-e, it was discussed how the remote UE reports the relay UE’s ID and agreed the following:

Agreements:

In scenario 2, if both remote and relay UE are in RRC_CONNECTED, the remote UE reports relay UE’s ID to gNB for indirect path addition.  Need for reporting in the idle/inactive case can be further discussed.  FFS what ID is used.

In RAN2#119bis-e, it was agreed that for multi-path Relay, support RRC_IDLE/RRC_INACTIVE target relay UE, for the path switching scenario where there is an addition of indirect path or a change of indirect path. As C-RNTI is available only for the RRC_CONNECTED UE, it was argued that report of C-RNTI would mandate the candidate relay UE to make the RRC connection, which seems to be against the agreement.

During the study phase, RAN2 reached to an agreement in RAN2#119e that the relation between remote UE and relay UE in scenario 2 is pre-configured or static. Having pre-configured or static relation between remote and relay UE would reduce the number of candidate relay UEs to a small number, such as 1. Furthermore, since the UE-to-UE link is non-3GPP interface, the remote/relay UE would have better knowledge than the gNB does. Therefore, it will be more reasonable and efficient for the remote UE to select the target relay UE, instead of reporting the non-3GPP condition to the gNB and letting the gNB choose the target relay UE.

Proposal 9: In scenario 2, the remote UE selects its target relay UE and reports the selected relay UE to the gNB.

When the remote UE selects the target relay UE by itself, the target relay UE may not be in RRC_CONNECTED. However, the relay UE can make the RRC connection before the remote UE reports the target relay UE’s ID given that RAN2 agreed to leave it to relay and remote UE implementation how to trigger the relay UE to initiate the RRC connection establishment procedure for scenario 2. Therefore, we see no need for reporting the ID of relay UE in RRC_IDLE or RRC_INACTIVE, and C-RNTI can be reported to the gNB.

Proposal 10: In scenario 2, the remote UE reports target relay UE’s C-RNTI to the gNB. How to make the target relay UE’s RRC connection (before the reporting the target relay UE’s C-RNTI) is up to implementation, as already agreed.

## 2.6	Support of CA and packet duplication over indirect path

It is not clear from the WID in [RP-223501] whether CA is applicable on the remote UE’s Uu link over the direct path and the relay UE’s Uu link over the indirect path. In our view, there is no reason to rule out CA on remote and relay UE’s Uu link.

Proposal 11. Carrier aggregation is applicable to the remote UE’s Uu link and relay UE’s Uu link.

Considering that reliability/throughput enhancement is one goal of sidelink multi-path, CA duplication over remote UE’s Uu link and relay UE’s Uu link is deemed necessary.

Proposal 12. CA duplication over remote UE’s Uu link and relay UE’s Uu link is supported.

Currently, CA duplication is performed at PDCP level. However, there is no PDCP in the relay UE. Therefore, in order to support CA duplication over relay UE’s Uu link, RAN2 needs to discuss how to perform the duplication without PDCP. For instance, the SRAP can perform the packet duplication.

Proposal 13. RAN2 discuss how to support CA duplication over relay UE’s Uu link for the relayed packet.

# 3	Conclusion

Observation 1: If the target relay UE is in RRC_CONNECTED, sending the RRCReconfigurationComplete message via the indirect path results in undesirable delay.

Proposal 1: If the target relay UE is in RRC_CONNECTED, the remote UE sends the RRCReconfigurationComplete message via the direct path.

Observation 2: The gNB can set the primary path depending on the RRC state of the target relay UE.

Proposal 2. To send the RRCReconfigurationComplete message to either a direct or an indirect path depending on the target relay UE’s RRC state, RAN2 discuss the following options:

- Option 1. The gNB sets the primary path of the split SRB0 based on the RRC state of the target relay UE.

- Option 2. The gNB configures a path for the remote UE to send the RRCReconfigurationComplete message via separate parameter than the primary path.

- Option 3. The gNB indicates the RRC state of the target relay UE to the remote UE so that the remote UE sends the RRCReconfigurationComplete message regardless of the primary path.

Proposal 3: RAN2 confirm the agreement on the support of non-split SRB on the indirect path.

Proposal 4: RAN2 consider agreeing on the modified WA that For a remote UE and a relay UE in RRC_CONNECTED, if the network releases the relay UE to RRC_IDLE or RRC_INACTIVE, the relay UE does not need to inform the remote UE of its explicit release of RRC connection. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

Proposal 5: For a remote UE and a relay UE in RRC_CONNECTED, the relay UE does not inform the remote UE of its reception of an RRCReconfiguration including the reconfigurationWithSync. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

Proposal 6: For a remote UE and a relay UE in RRC_CONNECTED, the relay UE does not inform the remote UE of its expiry of dataInactivityTimer, if configured. It is up to network to release the multipath configuration related to this relay UE at the remote UE.

Proposal 7: RAN2 consider that buffer size information, problematic link/interface indication, and link quality to be reported when reporting the RLF.

Proposal 8: RAN2 discuss if the gNB can configure a condition not to send the RLF report even if the UE detects the RLF on one path while another path can be used for transmission of the RLF report.

Proposal 9: In scenario 2, the remote UE selects its target relay UE and reports the selected relay UE to the gNB.

Proposal 10: In scenario 2, the remote UE reports target relay UE’s C-RNTI to the gNB. How to make the target relay UE’s RRC connection (before the reporting the target relay UE’s C-RNTI) is up to implementation, as already agreed.

Proposal 11. Carrier aggregation is applicable to the remote UE’s Uu link and relay UE’s Uu link.

Proposal 12. CA duplication over remote UE’s Uu link and relay UE’s Uu link is supported.

Proposal 13. RAN2 discuss how to support CA duplication over relay UE’s Uu link for the relayed packet.