3GPP TSG-RAN WG2 Meeting #122			     R2-2306316

Incheon, South Korea, 22-26 May, 2023

Agenda Item:	7.4.2.1 (NR_Mob_enh2-Core)

Source: 	LG Electronics Inc.

Title:         	Discussion on early TA acquisition and partial MAC reset

Document for: 	Discussion and Decision

Introduction

This contribution discusses follow-up issues about early TA acquisition and partial MAC reset.

Discussion

Early TA acquisition

RAN1 agreements in RAN2#112bis-e: For PDCCH ordered-RACH, if reception of RAR is not configured, UE autonomous re-transmission of PRACH is not allowed, regardless of the configuration of PreambleTransMax. When reception of RAR is configured, support RAR is received from serving cell at least in intra-DU case. When reception of RAR is configured, support RAR is received from serving cell in inter-DU case. - FFS: RA response window related issues For PDCCH ordered RACH mechanism in R18 LTM, when reception of RAR is configured, - the UE stores(remembers/maintains/handles) a TA for at least one candidate cell - storing(remembering/maintaining/handling) corresponding TAs for more than one candidate cell is up to UE capability - detailed number of candidate cell is up to UE capability

For “without RAR”, one open question is whether to support autonomous retransmission of preamble and RAN1 finally concludes not to support autonomous preamble retransmission as shown in above agreements. This means that the UE does not need to know the preamble is successfully received by target DU after one preamble transmission. If the transmitted preamble is not successfully received by the target DU, another PDCCH ordered RACH for candidate cell would be triggered by the target DU or the source DU. For this, the expected RAN2 impact on the MAC specification is that the MAC entity does not need to start RAR window and stop RACH procedure after one preamble transmission. Nevertheless, same CFRA preamble may be transmitted again by the subsequent PDCCH ordered RACH by the target DU. In this case, we need to know how to apply power ramping for this transmission, but several options are already discussed in RAN1 and it would be good to wait RAN1 conclusion first to avoid duplicated discussion for same issue.

Observation 1. For “without RAR”, as autonomous retransmission is not allowed regardless of the configuration of PreambleTransMax, the current RACH procedure may need to be modified.

Proposal 1. When “without RAR” is configured, the UE does not start RAR window and stop RACH procedure after one preamble transmission.

For “with RAR”, RAN2 already decided to support “with RAR from serving cell” only in Rel-18 and RAN1 also made related agreements in the last meeting. Now we can discuss further details on RAR and two options are discussed in RAN1 for receiving TA value of candidate cell.

- Option 1. Reuse legacy MAC RAR;

- Option 2. New MAC CE.

In option 1, as PDCCH ordered CFRA is only for acquiring TA value of candidate cell, information in legacy RAR other than Timing advance command is not needed. Thus, fields for UL grant and TC-RNTI would be ignored after receiving RAR. Some companies may argue that there may be CFRA preamble collision when the RAR is received from the source DU since the CFRA preamble is transmitted to the target cell but same CFRA preamble can be used in source cell. However, considering that RAN2 agreed to support shared preamble resource pool for candidate cells, the source DU would allocate a preamble to UEs in a source cell except those shared preamble resource pool for candidate cells and there should be no CFRA preamble collision. Another important point is how to calculate RA-RNTI to receive RAR from the source DU after transmitting CFRA preamble to the target DU. Since RA-RNTI is associated with the PRACH occasion in which the Preamble is transmitted, the source DU should know which PRACH occasion is used to transmit the preamble for candidate cell to the target DU. For this, the target DU should forward information about PRACH occasion in which preamble is received to the source DU and relevant TA value of candidate cell.

In option 2, on the other hand, the UE can acquire the absolute TA through Absolute TA command including candidate cell index scrambled with C-RNTI from source DU after transmitting a preamble to the target DU. In this case, the target DU should also forward information about successfully received preamble and relevant TA value of candidate cell to the source DU. Considering that anyway both options need information exchange between source DU and target DU, we slightly prefer to reuse legacy MAC RAR as much as possible.

Proposal 2. For “with RAR from source cell”, legacy MAC RAR is used to receive TA value for candidate cell. UL grant and TC-RNTI in RAR are discarded.

For the following FFS point in RAN2, when cell switch command includes TA value for the candidate cell, if the network knows TA value of the candidate cell, the simple and clear approach is just include the known TA value of candidate cell in a cell switch command. If “value 0” or “value indicating that the UE shall apply the TA of one source cell” can be included, there should be an additional step to compare the TA value of the candidate cell and TA value of source cell. We think that this is redundant step and just including TA value for candidate cell in the cell switch command should be sufficient.

Proposal 3. When TA value is included in the cell switch command, “value 0” and “value indicating that the UE shall apply the TA of one source cell” is not given to the UE.

- With the assumption that the UE will skip RACH in the target cell if a TA value is given in the cell switch command: It is FFS if the following TA values can be given to the UE: - Value 0, - Value indicating that the UE shall apply the TA of one source cell.

Partial MAC reset for intra-DU cell change

In RAN2#121 meeting, RAN2 made the following agreement and now we can discuss remaining parts of partial MAC reset.

No consensus to support HARQ continuation (and in order to resume discussion some new input may be needed, e.g. quantitative evidence of a serious problem).

Considering the company’s views in RAN2#121 meeting, we think that the following candidates can be considered to determine whether to skip or not during partial MAC reset:

- UL HARQ

- DL HARQ

- BSR

- logical channel Bj value

- UL time alignment

- Triggered Recommended bit rate query procedure

During intra-DU cell change, the main point is that SDAP, PDCP, RLC, and MAC is not changed, but the physical layer is changed because only cell is changed. Thus, at least physical layer or cell related parameters should not be skipped. From this perspective, UL HARQ and DL HARQ should not be skipped since the HARQ entity is defined per cell. Of course, some companies still insist to continue UL/DL HARQ to reduce data interruption. However, if HARQ related configurations in a new cell is different from the old cell, complex HARQ handling, e.g., cross-carrier HARQ retransmission and HARQ process ID mapping for CG/SPS, may be needed to continue HARQ. Especially, if CA is used, more complex handling would be required. In our view, it is not sure how much data interruption can be reduced through this HARQ continuing as sacrificing this complexity.

Considering that RLC and MAC are not changed and BSR/Bj is not related to physical layer, the triggered BSR and Bj can be maintained during intra-DU cell change since BSR is important information for scheduling at the network.

Regarding UL time alignment, RAN2 already agreed to support subsequent LTM. Considering that last serving cell can be a candidate cell after cell switching by LTM command, desirable UE behaviour should keep maintaining TA of serving cell after cell switching.

For Triggered Recommended bit rate query procedure, unlike BSR, it is not urgent information to the network and the priority of MAC CE for Recommended bit rate query is low in LCP procedure. Besides, given that cell quality may be changed a lot after cell change, it would be good to cancel Triggered Recommended bit rate query procedure during partial MAC reset for intra-DU cell change.

Proposal 4. For partial MAC reset during intra-DU cell change, the MAC entity shall

- maintain the triggered BSR and logical channel Bj value;

- flush the soft buffers for all DL HARQ processes;

- set the NDIs for all uplink HARQ processes to the value 0;

- cancel triggered Recommended bit rate query procedure.

- maintain the UL time alignment

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. For “without RAR”, as autonomous retransmission is not allowed regardless of the configuration of PreambleTransMax, the current RACH procedure may need to be modified.

Proposal 1. When “without RAR” is configured, the UE does not start RAR window and stop RACH procedure after one preamble transmission.

Proposal 2. For “with RAR from source cell”, legacy MAC RAR is used to receive TA value for candidate cell. UL grant and TC-RNTI in RAR are discarded.

Proposal 3. When TA value is included in the cell switch command, “value 0” and “value indicating that the UE shall apply the TA of one source cell” is not given to the UE.

Proposal 4. For partial MAC reset during intra-DU cell change, the MAC entity shall

- maintain the triggered BSR and logical channel Bj value;

- flush the soft buffers for all DL HARQ processes;

- set the NDIs for all uplink HARQ processes to the value 0;

- cancel triggered Recommended bit rate query procedure.

- maintain the UL time alignment