3GPP TSG-RAN WG2 Meeting #122	R2-2306321

# Incheon, Korea, 22 – 26 May 2023

Agenda item:		7.11.2.1

Source:	Intel Corporation

Title:	Control plane aspects for multicast reception in RRC_INACTIVE

Document for:		Discussion and Decision

# Introduction

In RAN2#121 meeting, RAN2 agreed on the following regarding PTM configuration for multicast reception in RRC_INACTIVE:

UE shall join in the multicast session before receiving multicast in RRC INACTIVE. If network finds it useful, the PTM configuration for the (single) serving cell can be configured to UE before the session activation, and UE stores the configuration. When session is activated, UE can receive multicast in INACTIVE state by applying the configuration without going back to RRC_CONNECTED, if not updated by MCCH after being configured. When network configures UE to receive multicast in INACTIVE state, RRCRelease message with suspendconfig can be used to deliver the PTM configuration. Other dedicated RRC messages will not be used to provide PTM configuration for MBS multicast for INACTIVE. We introduce a new MCCH logical channel for multicast in INACTIVE (different from broadcast MCCH) Multicast MCCH configuration is provided via new SIB. Optionally, Multicast MCCH configuration for the serving cell can also be provided in dedicated signalling. Understanding is we are not optimizing mobility case because of this. Serving cell will not provide the PTM configuration of neighbour cells from other gNBs. FFS whether the network can provide PTM configuration for intra-gNB cells.

In RAN2#121bis-e meeting, CP aspects of multicast reception in RRC_INACTIVE were progressed based on email discussions “[Post121][606][eMBS] Service continuity and notifications (ZTE)” and “[AT121bis-e][603][eMBS] Service continuity and notifications (ZTE)”.

In this contribution, we discuss control plane aspects for multicast reception in RRC_INACTIVE.

# Discussion

One open issue from RAN2#121 meeting is “FFS whether the network can provide PTM configuration for intra-gNB cells”. Providing PTM configuration for intra-gNB cells does not have the drawback of signalling exchange between gNBs. The potential benefit is the improved service continuity during cell reselection. However it should be noted that most resource efficient way for MBS services in intra-gNB scenario is to use Single Frequency Network (SFN), where the intra-gNB cells transmit the same service in the same time-frequency resources to boost SINR. Although UE is not aware of whether SFN is used or not, one possible UE implementation is that after cell reselection, UE tries to receive multicast service using PTM configuration in source cell when acquiring PTM configuration in the target cell. When SFN is used, UE can receive the multicast service using “old” PTM configuration (from source cell) after reselecting to target cell. Therefore service continuity can be ensured when UE reselects to an intra-gNB neighbor cell. In above discussion, there is no need to provide PTM configuration for intra-gNB cells.

Observation 1: When SFN is used among intra-gNB cells, there are no benefits of providing PTM configuration for intra-gNB cells.

There are some drawbacks of providing PTM configuration for intra-gNB cells: it increases signalling overhead in air interface, and requires additional standardization efforts e.g. on the validity check of the PTM configuration of non-serving cells.

Proposal 1: PTM configuration of intra-gNB neighbor cells are not provided.

Rel-17 MBS defines procedures related to change of MCCH information, e.g. MCCH modification period and notification of changes. The Rel-17 MBS procedure can be used for the change of PTM configuration.

Proposal 2: Rel-17 MBS procedures regarding change of MCCH information is used for PTM configuration update.

Regarding PTM configuration structure, it is helpful to review Rel-17 signalling structure of multicast. Rel-17 RRC structure for multicast MRB is scattered in several places, as in Table 1 below. Add/mod/release lists are used for PDCP, RLC, MAC & PHY configurations. Add/mod/release lists are delta signalling to modify the multicast MRB configurations stored in the UE. They do not work for multicast MCCH which is transmitted to a set of UEs, since a UE just starting multicast reception or just moving to the cell does not have previous lists. Therefore, multicast MCCH message should be self-contained without using add/mod/release list.

Table 1: Multicast MRB configuration in RRC

| Configuration | IE name | Parent IE |
| --- | --- | --- |
| TMGI, PDCP Configuration | MRB-ToAddMod-r17 | RadioBearerConfig >> mrb-ToAddModList-r17 |
| RLC configuration | RLC-BearerConfig | CellGroupConfig >> rlc-BearerToAddModList |
| MAC & PHY configuration | MBS-RNTI-SpecificConfig-r17 | CellGroupConfig >> MAC-CellGroupConfig >> g-RNTI-ConfigToAddModList-r17 |

Proposal 3: Multicast MCCH message is self-contained without using add/mod/release list.

Some companies propose to have multiple multicast MCCHs e.g. MCCH can be session specific. In Rel-17, RAN2 agreed that there is a single MCCH for broadcast. The concerns for multiple broadcast MCCHs are more power consumption for UEs monitoring multiple MCCHs as well as increased complexity. The concerns are also valid for multiple multicast MCCHs. In addition, Rel-17 RRC structure for multicast MRB is scattered in several places, as in Table 1 above. It is easier to group those IEs in one multicast MCCH. Some additional standardization efforts are needed to separate IEs into session specific multicast MCCHs.

Considering above aspects, it is proposed to have a single multicast MCCH per cell.

Proposal 4: Only single multicast MCCH is supported per cell.

# Conclusion

In this contribution, we discuss control plane aspects for multicast reception in RRC_INACTIVE. We have the following observation:

Observation 1: When SFN is used among intra-gNB cells, there are no benefits of providing PTM configuration for intra-gNB cells.

We propose the following:

Proposal 1: PTM configuration of intra-gNB neighbor cells are not provided.

Proposal 2: Rel-17 MBS procedures regarding change of MCCH information is used for PTM configuration update.

Proposal 3: Multicast MCCH message is self-contained without using add/mod/release list.

Proposal 4: Only single multicast MCCH is supported per cell.