3GPP TSG-RAN WG2 Meeting #122	R2-2306326

Incheon, Korea, May 22-26, 2023

Agenda item:	7.7.4.2

Source:	Samsung

Title:	Discussion on NTN Handover Enhancements

WID/SID:	NR_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# Introduction

In RAN2#121bis-e meeting, NR NTN handover enhancements were discussed regarding common signaling for HO command and RACH-less HO [1] [2]. In this document, we continue to discuss the HO enhancements on these aspects.

# Discussion

## Common (C)HO configuration

Common (C)HO configuration was discussed for the purpose of reducing overhead. To see how much overhead can be reduced, we consider the current RRC reconfiguration message including reconfigurationWithSync as the HO command.

ReconfigurationWithSync ::=         SEQUENCE {

spCellConfigCommon                  ServingCellConfigCommon                                     OPTIONAL,   -- Need M

newUE-Identity                      RNTI-Value,

t304                                ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

rach-ConfigDedicated                CHOICE {

uplink                              RACH-ConfigDedicated,

supplementaryUplink                 RACH-ConfigDedicated

}                                                                                               OPTIONAL,   -- Need N

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

daps-UplinkPowerConfig-r16      DAPS-UplinkPowerConfig-r16                                      OPTIONAL    -- Need N

]],

[[

sl-PathSwitchConfig-r17         SL-PathSwitchConfig-r17                                         OPTIONAL    -- Cond DirectToIndirect-PathSwitch

]]

}

–	ServingCellConfigCommon

The IE ServingCellConfigCommon is used to configure cell specific parameters of a UE's serving cell. The IE contains parameters which a UE would typically acquire from SSB, MIB or SIBs when accessing the cell from IDLE. With this IE, the network provides this information in dedicated signalling when configuring a UE with a SCells or with an additional cell group (SCG). It also provides it for SpCells (MCG and SCG) upon reconfiguration with sync.

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVINGCELLCONFIGCOMMON-START

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

uplinkConfigCommon                  UplinkConfigCommon                                                  OPTIONAL,   -- Need M

supplementaryUplinkConfig           UplinkConfigCommon                                                  OPTIONAL,   -- Need S

n-TimingAdvanceOffset               ENUMERATED { n0, n25600, n39936 }                                   OPTIONAL,   -- Need S

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

dmrs-TypeA-Position                 ENUMERATED {pos2, pos3},

lte-CRS-ToMatchAround               SetupRelease { RateMatchPatternLTE-CRS }                            OPTIONAL, -- Need M

rateMatchPatternToAddModList        SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPattern   OPTIONAL, -- Need N

rateMatchPatternToReleaseList       SEQUENCE (SIZE (1..maxNrofRateMatchPatterns)) OF RateMatchPatternId OPTIONAL, -- Need N

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

...,

[[

channelAccessMode-r16               CHOICE {

dynamic                             NULL,

semiStatic                          SemiStaticChannelAccessConfig-r16

}                                                                                                       OPTIONAL, -- Cond SharedSpectrum

discoveryBurstWindowLength-r16          ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5}                   OPTIONAL, -- Need R

ssb-PositionQCL-r16                     SSB-PositionQCL-Relation-r16                                    OPTIONAL, -- Cond SharedSpectrum

highSpeedConfig-r16                     HighSpeedConfig-r16                                             OPTIONAL  -- Need R

]],

[[

highSpeedConfig-v1700               HighSpeedConfig-v1700                                               OPTIONAL, -- Need R

channelAccessMode2-r17              ENUMERATED {enabled}                                                OPTIONAL, -- Cond SharedSpectrum2

discoveryBurstWindowLength-r17      ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot75, ms1, ms1dot25}  OPTIONAL, -- Need R

ssb-PositionQCL-r17                 SSB-PositionQCL-Relation-r17                                        OPTIONAL, -- Cond SharedSpectrum2

highSpeedConfigFR2-r17              HighSpeedConfigFR2-r17                                              OPTIONAL, -- Need R

uplinkConfigCommon-v1700            UplinkConfigCommon-v1700                                            OPTIONAL, -- Need R

ntn-Config-r17                      NTN-Config-r17                                                      OPTIONAL  -- Need R

]],

[[

featurePriorities-r17               SEQUENCE {

redCapPriority-r17                  FeaturePriority-r17                                             OPTIONAL, -- Need R

slicingPriority-r17                 FeaturePriority-r17                                             OPTIONAL, -- Need R

msg3-Repetitions-Priority-r17       FeaturePriority-r17                                             OPTIONAL, -- Need R

sdt-Priority-r17                    FeaturePriority-r17                                             OPTIONAL  -- Need R

}                                                                                                       OPTIONAL  -- Need R

]],

[[

ra-ChannelAccess-r17            ENUMERATED {enabled}                                                OPTIONAL -- Cond SharedSpectrum2

]]

}

-- TAG-SERVINGCELLCONFIGCOMMON-STOP

-- ASN1STOP

The IE ServingCellConfigCommon in ReconfigurationWithSync is cell-specific configuration common to UEs which contains information that can be typically acquired from system information. Therefore, the information included in ServingCellConfigCommon in ReconfigurationWithSync can be included in the common (C)HO configuration.

Proposal 1: Common (C)HO configuration includes at least ServingCellConfigCommon in ReconfigurationWithSync.

When the number of UEs to be handed over in a short duration is large, especially for quasi-earth-fixed cell, the overhead reduction can be significant by broadcasting the common configuration. For quasi-earth-fixed cell, the incoming cells to serving an area is predictable, there is no need to frequently broadcast/update the common configuration for the target cell. Normally the target cell to replace the source cell would be 1 or 2. Thus, common configuration for up to 2 potential target/candidate cells should be sufficient.

Proposal 2: Common (C)HO configuration includes up to 2 potential target/candidate cells.

The common (C)HO configuration can be provided by broadcast signaling, which can be easily supported by including the common (C)HO configuration in SIB. For quasi earth-fixed cell, basically all UEs in the source cell form a single group, there is no need to consider groupcast signaling for finer groups. Furthermore, groupcasting RRC configuration is not possible in the existing signaling framework, it will involve large spec impact. Therefore, broadcast signaling common (C)HO configuration is preferred.

Proposal 3: Common (C)HO configuration is provided by broadcast signaling.

For quasi-earth-fixed cell, as the incoming target cell is predictable, the source cell can broadcast the common configuration for the target cell in advance. When UE receives a dedicated RRCReconfiguration message triggering the HO to the target cell, UE performs HO procedure and applies the target cell configuration by combining the common configuration in SIB and UE dedicated configuration in RRCReconfiguration message. Compared to the current HO procedure, the additional steps are simply and straightforward, that are 1) storing the common configuration received in SIB; 2) combining the common configuration with UE dedicated configuration.

Proposal 4: The common HO configuration in SIB is acquired and stored before receiving HO command, and is combined with UE dedicated configuration when performing HO.

## RACH-less Handover

The general procedure for RACH-less HO has been agreed in the last meeting.

Agreement RAN2#121bis-e: RAN2 confirms the general UE procedure for NTN RACH-less HO 1. receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC) 2. start timer T304 for the target cell (RRC) 3. perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC) 4. start time alignment timer (MAC) 5. monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY) 6. send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY) 7. consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC) 8. stop timer T304 for the target cell. (RRC) FFS whether to release UL grant if pre-allocated after RACH-less HO completion FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

### UL synchronization

One open issue is whether N_TA is optional. In RAN2#121, the following agreement has been made for N_TA.

Agreement RAN2#121bis-e:

- In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW.

In LTE RACH-less HO, N_TA is always configured with either value 0 or the same value as a serving cell. This can be the same for NTN RACH-less HO.

Proposal 5: In RACH-less HO command, the parameter for N_TA is mandatory present which is either 0 or same value as the source cell.

For UL synchronization, as specified in TS 38.211 and TS 38.213, UE calculates TA, i.e., TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc, for the first UL transmission, where

- NTA,offset is configured by parameter n-TimingAdvanceOffset or a default value is used if not configured,

- NTA,adjcommon is configured by common TA parameters of the target cell,

- NTA,adjUE is computed based on UE location and ephemeris of the target cell.

- NTA is configured in RACH-less HO command for the target cell.

Proposal 6: In NTN RACH-less HO, UE calculates the TA value for the target cell using N_TA as specified in 38.211, 38.213, and pre-compensates the TA for initial UL transmission.

### UL carrier and BWP selection

In RACH-based HO, if supplementary UL carrier is configured, UE first selects UL carrier, as specified in TS 38.321 5.1.1 for Random Access procedure.

1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the signalled carrier. 1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier.

Similarly, for the initial UL transmission in RACH-less HO, if supplementary UL carrier is configured, UE has to first select UL carrier. For pre-allocated grant, if the pre-allocated grant is configured for the NUL carrier, UE selects NUL carrier to send the initial UL transmission; if the pre-allocated grant is configured for SUL carrier, UE selects SUL carrier to send the initial UL transmission. For dynamic grant, DCI includes the NUL/SUL indicator and UE selects the indicated carrier to send the initial UL transmission. If UL carrier is not explicitly signaled either for pre-allocated grant or for DG, and if SUL carrier is configured, UE selects UL carrier based on a RSRP threshold as in Random Access procedure.

Proposal 7: If SUL is configured and if the UL carrier for initial UL transmission is not explicitly signaled, select NUL or SUL carrier based on a RSRP threshold as in Random Access procedure.

For the initial UL transmission in RACH-less HO, UE needs to identify which UL BWP is used. For RACH-based HO, the UE performs CFRA on the BWP indicated by firstActiveUplinkBWP. Similarly, the BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission in RACH-less HO. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

Proposal 8: The BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

### TA report MAC CE

RAN1 has agreed that the initial UL transmission includes the RRCReconfigurationComplete message. Whether TA report MAC CE is included in the initial UL transmission in RACH-less HO has to be considered. In RACH-based HO, TAR MAC CE is included in Msg3/MsgA (or Msg5 if no enough UL grant for Msg3/MsgA) if ta-Report is enabled for reconfiguration with sync. Similar principle can be followed for RACH-less HO.

Proposal 9: If ta-Report is enabled for RACH-less HO, TA report MAC CE is included in the initial UL transmission.

### HARQ process for initial UL transmission

For the HARQ process for the initial UL transmission, a specific HARQ process ID can be specified, regardless of using pre-allocated grant or DG. HARQ process ID 0 can be used, same as Msg3/MsgA transmission in Random Access procedure. This simplifies the NW scheduling and UE does not need to derive HARQ process ID based on parameters.

Proposal 10: HARQ process ID 0 is used for initial UL transmission.

In Rel-17, HARQ mode A and B are introduced for UL, and HARQ mode of HARQ process 0 (i.e., Msg3/MsgA) scheduled by RAR is up to NW implementation. We think this is applicable to initial UL transmission in RACH-less HO.

Proposal 11: HARQ mode for the HARQ process of initial UL transmission is up to NW implementation.

Another issue to be considered is whether UE is allowed to transmit user data in the HARQ process of initial UL transmission before the successful completion of the RACH-less HO. We think before UE receiving NW confirmation of RACH-less HO completion, UE cannot confirm UL transmission can be reliably received by NW so it should not send any user data (i.e., not select the logical channel(s) corresponding to DRB(s)) before RACH-less HO completion.

Proposal 12: Before the successful completion of the RACH-less HO, the MAC entity shall not select the logical channel(s) corresponding to DRB(s) for the uplink grant for the HARQ process of initial UL transmission.

If DRX is configured, UE behavior for the HARQ process of initial UL transmission needs to be considered. The Active Time and UE behavior in Active time are specified as follows.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: - drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or - drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or - ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a). ---omitted--- 1> if a DRX group is in Active Time: 2> monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission; or 2> if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or 2> if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]: 3> if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled: 4> if the corresponding HARQ process is configured with HARQ feedback enabled: 5> set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value; 5> start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. 3> else: 4> start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback. NOTE 3: When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback. 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported; 3> stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process; 3> if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]: 4> start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> if this Serving Cell is configured with uplinkHARQ-Mode: 4> if the corresponding HARQ process is configured as HARQModeA: 5> set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value; 5> if drx-LastTransmissionUL is configured: 6> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 5> else: 6> start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 3> else: 4> if drx-LastTransmissionUL is configured: 5> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission. 4> else: 5> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission. 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. ---omitted---

As it has been agreed that if pre-allocate grant is not configured, UE monitors PDCCH for DG for initial UL transmission. Then, if DRX is configured, the Active time for Serving Cells in a DRX group should include the time between the start of T304 and the reception of first PDCCH that indicates the DG for initial UL transmission.

Proposal 13: If DRX is configured, the Active time for Serving Cells in a DRX group includes the time between the start of T304 and the reception of first PDCCH that indicates the DG for initial UL transmission.

### Release of pre-allocated grant

Another open issue is whether the pre-allocated grant is released or not after RACH-less HO completion. We think the same procedure as LTE RACH-less HO can be followed, i.e., release the pre-allocated grant once the RACH-less HO is completed. Since the UE behavior of using the pre-allocated grant in RACH-less HO procedure is different from that of using normal configured grant in connected mode, it makes more sense not to reuse the pre-allocated grant for subsequent UL transmissions after RACH-less HO completion.

Proposal 14: Release the pre-allocated grant once RACH-less HO is successfully completed.

# Conclusion

Based on the discussion, we have the following observations and proposals.

Common (C)HO configuration

Proposal 1: Common (C)HO configuration includes at least ServingCellConfigCommon in ReconfigurationWithSync.

Proposal 2: Common (C)HO configuration includes up to 2 potential target/candidate cells.

Proposal 3: Common (C)HO configuration is provided by broadcast signaling.

Proposal 4: The common HO configuration in SIB is acquired and stored before receiving HO command, and is combined with UE dedicated configuration when performing HO.

RACH-less HO

Proposal 5: In RACH-less HO command, the parameter for N_TA is mandatory present which is either 0 or same value as the source cell.

Proposal 6: In NTN RACH-less HO, UE calculates the TA value for the target cell using N_TA as specified in 38.211, 38.213, and pre-compensates the TA for initial UL transmission.

Proposal 7: If SUL is configured and if the UL carrier for initial UL transmission is not explicitly signaled, select NUL or SUL carrier based on a RSRP threshold as in Random Access procedure.

Proposal 8: The BWP indicated by firstActiveUplinkBWP is selected for the initial UL transmission. If the firstActiveUplinkBWP is not signaled, initialUplinkBWP is selected.

Proposal 9: If ta-Report is enabled for RACH-less HO, TA report MAC CE is included in the initial UL transmission.

Proposal 10: HARQ process ID 0 is used for initial UL transmission.

Proposal 11: HARQ mode for the HARQ process of initial UL transmission is up to NW implementation.

Proposal 12: Before the successful completion of the RACH-less HO, the MAC entity shall not select the logical channel(s) corresponding to DRB(s) for the uplink grant for the HARQ process of initial UL transmission.

Proposal 13: If DRX is configured, the Active time for Serving Cells in a DRX group includes the time between the start of T304 and the reception of first PDCCH that indicates the DG for initial UL transmission.

Proposal 14: Release the pre-allocated grant once RACH-less HO is successfully completed.

# Reference

[1] R2-2304248 Report of [AT121bis-e][108][NR NTN Enh] Common (C)HO configuration (Ericsson)

[2] R2-2304249 Report of [AT121bis-e][109][NR NTN Enh] RACH-less HO (Samsung)