3GPP TSG-RAN WG2 #122	R2-2306332

Incheon, South Korea, 22nd – 26th May 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on Cell barring for eRedCap

Document for:	Discussion

Agenda Item:	7.19.3

- Introduction

A new WID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved at the RAN#97-e meeting and revised at the RAN#98-e meeting [1]. The objectives of the WI are shown below.

The objective is to specify support for the following enhancements: Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2 and CT1 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99-e regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In this contribution, we describe our view on cell barring for Rel-18 eRedCap.

- Discussion

  - Cell barring

At the RAN2#121, following agreement was made[2];

- The NR MIB “cellBarred” bit applies to all UEs (Normal UEs, Redcap UEs and eRedcap UEs).

But unfortunately, at the RAN2#121-bis, there is no consensus about cell barring.

The Justification in the WID[1] states as follows;

Through the Rel-17 NR RedCap work item, 3GPP has established a framework for enabling reduced capability NR devices suitable for a range of use cases, including the industrial sensors, video surveillance, and wearables use cases, with requirements on low UE complexity and sometimes also on low UE power consumption.

To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, e.g., industrial wireless sensor network use cases, some further complexity reduction enhancements should be considered.

Therefore, it is expected that the market targeted by Rel-18 eRedCap is targeted at a different market than Rel-17 RedCap. In other words, it is supposed to be possible for operators to provide service only to Rel-18 eRedCap and not to accept Rel-17 RedCap.

Observation 1	It is supposed to be possible for operators to provide service only to Rel-18 eRedCap and not to accept Rel-17 RedCap.

In such cases, if Rel-17 RedCap UE cannot be excluded at Access control, it will cause unnecessary load on the Network side. Therefore, at least a mechanism to barring Rel-17 RedCap UEs and allow only Rel-18 eRedCap UEs is needed.

Proposal 1. 	At least a mechanism to barring Rel-17 RedCap UEs and allow only Rel-18 eRedCap UEs is needed.

Similarly for 1RX / 2RX barring, Rel-17 RedCap and Rel-18 eRedCap should be independently controllable as they are intended for different use cases.

On the other hand, if a consensus for discussion cannot be reached in RAN2, it appears that it would be better to focus on the point "whether the 1Rx/ 2Rx distinction is independently required for Rel-17 RedCap and Rel-18 eRedCap" and discuss it in the RAN plenary as in Rel-17. Since it is the scope of RAN2 regarding access control, even if we ask RAN1 for comments, we assume that we will not receive the desired answer if the technical issues are not clarified.

Proposal 2. 	If a consensus for discussion cannot be reached in RAN2, it would be better to focus on the point "whether the 1Rx/ 2Rx distinction is independently required for Rel-17 RedCap and Rel-18 eRedCap" and discuss it in the RAN plenary

- Summary and proposal

Observation 1	It is supposed to be possible for operators to provide service only to Rel-18 eRedCap and not to accept Rel-17 RedCap.

Proposal 1. 	At least a mechanism to barring Rel-17 RedCap UEs and allow only Rel-18 eRedCap UEs is needed.

Proposal 2. 	If a consensus for discussion cannot be reached in RAN2, it would be better to focus on the point "whether the 1Rx/ 2Rx distinction is independently required for Rel-17 RedCap and Rel-18 eRedCap" and discuss it in the RAN plenary

- References

[1] RP-223544, Enhanced support of reduced capability NR devices, Ericsson.

[2] R2-2301910, Report from eRedCap breakout session, Session chair (Ericsson)