3GPP TSG-RAN WG2 Meeting #122                                    R2-2306333

Incheon, Korea, May 22-26, 2023

Agenda item:		7.5.2 (NR_XR_enh-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on XR awareness

Document for:	Discussion and Decision

1. Introduction

This document shows our views on XR awareness.

2. Discussion

2.1 Need of UL traffic data arrival and End of the data burst

In RAN2#121bis e-meeting, the following agreements were made.

3. UE can report jitter information associated to UL XR traffic. How UE derives this jitter is left up to implementation (similarly as it is captured by SA2 for the jitter associated with the periodicity in DL. FFS what exactly is reported to the RAN (aim to have similar information as for DL). FFS on UL traffic data arrival reporting. FFS on whether EoDB signalling is needed.

In this section, we show our views on the FFS point of the need of UL traffic data arrival reporting and end of data burst signalling.

Let’s discuss the need of the UL traffic data arrival reporting first.

In our understanding, the UL traffic data arrival reporting is used to indicate when the UL burst traffic arrives. For a similar purpose, according to TS 23.501, the UE can optionally report the burst arrival time via TSCAI in order to indicate when the first packet of the data burst arrives for UL.

Table 5.27.2-1: TSC Assistance Information (TSCAI)

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two data bursts. |
| Burst Arrival Time (optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Survival Time (optional) | Survival Time, as defined in TS 22.261 [2], refers to the time period an application can survive without any data burst. |
| Burst Arrival Time Window (BAT Window) (optional) (NOTE 1) (NOTE 2) | Indicates the acceptable earliest and latest arrival time of the first packet of the data burst at either the ingress of the RAN (downlink flow direction) or the egress of the UE (uplink flow direction). |
| Capability for BAT adaptation (optional) (NOTE 1) | Indicates that the AF will adjust the burst sending time according to the network provided Burst Arrival Time offset (see clause 5.27.2.5). |
| N6 Jitter Information (optional) (NOTE 3) | Jitter information associated with the Periodicity in downlink (see clause 5.378.1). |
| Periodicity Range (optional) (NOTE 4) | It indicates that the AF will adjust the periodicity and provides the acceptable range, formulated as lower bound and upper bound of the Periodicity. |
| NOTE 1: Only one of the parameters (BAT Window or Capability for BAT adaptation) can be provided. NOTE 2: The parameter can only be provided together with Burst Arrival Time. NOTE 3: Only one of the parameters Burst Arrival Time or N6 Jitter Information may be provided for a given Traffic Flow. NOTE 4: The Periodicity Range can only be provided together with Periodicity when Burst Arrival Time and Burst Arrival Time Window are present. |  |

In other words, the UE reports the burst arrival time to the SMF via TSCAI and the SMF may inform the burst arrival time to the RAN. Thus, there is no reason to directly report the UL traffic arrival to the RAN from the UE. In addition, considering that it is always good to use the legacy procedure as much as possible, we think the UL traffic data arrival should be reported via TSCAI.

Proposal 1. The UL traffic data arrival is reported by BAT via TSCAI, same as legacy.

The next discussion is the need for the end of the data burst signalling.

In the RAN2#121bis e-meeting, it was proposed that the end of the data burst signalling should be indicated in order to terminate UE’s DRX active time early. More specifically, the UE triggers the scheduling request when data burst is started. Then, the UE transmits the MAC PDUs during the data burst period. After that, the UE starts the drx-InactivityTimer after transmitting the last MAC PDU even if there is no more data to be transmitted.

The UE does not need to monitor the PDCCH while running the drx-InactivityTimer if the last MAC PDU is successfully transmitted. This is because the UE does not need to receive a new UL grant for new transmission. In this case, if the end of the data burst is signalled to the network, the network indicates an indication to stop the drx-InactivityTimer. For this reason, some companies proposed that the end of the data burst should be signalled and they want to introduce a new signalling for end of the data burst.

However, we think that the power saving gain is very limited as shown in the above figure. This is because the long drx-InactivityTimer would not be configured for data burst. In addition, since the padding BSR with zero BS information can be transmitted in the last MAC PDU, the need for new signalling for the end of the data burst is questionable.

Of course, the padding BSR is not always contained in a MAC PDU if there is no room to contain the padding BSR in the MAC PDU. However, considering that the short drx-InactivityTimer can be configured, the PDCCH monitoring is not a critical problem while running short drx-InactivityTimer. For this reason, we do not see a strong reason to introduce a new signalling for the end of the data burst.

Thus, we think that the BSR with zero BS information is used to signal the end of the data burst for uplink. For this, we think the specification change is not needed.

Proposal 2. Do not introduce a new signalling for end of the data burst.

2.2 PDCP duplication based on the PDU set importance.

The PDU set importance is introduced to enable the differentiated handling of the PDU set. For example, it is assumed that the I-frame has the high importance than other frame. Since the P-frame and B-frame are decoded based on the I-frame, if the PDUs for I-frame are not successfully transmitted to the network, P-frame and B-frame do not need to be transmitted to the network. In other words, it is required that the transmission of the PDU set with the high importance should be more reliable than the transmission of the PDU set with the low importance.

Observation 1. The transmission of the PDU set with the high importance should be more reliable than the transmission of the PDU set with the low importance.

In addition, according to the latest agreements, all PDU sets associated with a single QoS flow would be transmitted through the RLC entities with the same priority, same as legacy. That is, the PDU sets belonging to a QoS flow are transmitted through the same RLC entity even if they have different importance. In this case, reliable transmission of the PDU set with high importance may not be ensured due to the transmission of the PDU set with low importance, e.g. Head-of-Line blocking.

Observation 2. Reliable transmission of the PDU set with high importance may not be ensured due to the transmission of the PDU set with low importance.

In order to ensure the reliable transmission of the PDU set with high importance, we can reuse the PDCP duplication introduced in Rel-15. However, if the PDCP duplication is applied to all PDU sets, the reliable transmission of the PDU set with high importance still may not be ensured due to the transmission of the PDU set with low importance.

In addition, considering that the PDCP duplication consumes much radio resource to perform duplicated transmission, applying PDCP duplication to high data rate such as XR traffic would increase huge signalling overhead

Observation 3. The use of the PDCP duplication for XR as it is may increase the signalling overhead significantly.

Based on above observations, the PDCP duplication can be reused with small enhancement in order to ensure reliable transmission of the PDU set with high importance while minimizing signalling overhead. More specifically, the PDCP entity applies the PDCP duplication only for the PDCP PDUs associated with PDU set with the high importance, and does not apply for the PDCP PDUs associated with PDU set with the low importance.

Proposal 3. In order to ensure reliable transmission of the PDU set with high importance within a DRB, the PDCP entity selectively duplicates the PDCP PDUs associated with PDU set with the high importance.

3.	Conclusion

Based on the above discussion, we made following observations and proposals.

Proposal 1. The UL traffic data arrival is reported by BAT via TSCAI, same as legacy.

Proposal 2. Do not introduce a new signalling for end of the data burst.

Observation 1. The transmission of the PDU set with the high importance should be more reliable than the transmission of the PDU set with the low importance.

Observation 2. Reliable transmission of the PDU set with high importance may not be ensured due to the transmission of the PDU set with low importance.

Observation 3. The use of the PDCP duplication for XR as it is may increase the signalling overhead significantly.

Proposal 3. In order to ensure reliable transmission of the PDU set with high importance within a DRB, the PDCP entity selectively duplicates the PDCP PDUs associated with PDU set with the high importance.