3GPP TSG-RAN WG2 Meeting #122                                                                        R2-2306338

Incheon, Korea, May 22nd- 26th 2023

Agenda item:	7.8.3

Source:	China Telecom

Title:	Flight path reporting enhancements for NR UAV

Document for:	Discussion and Decision

- Introduction

In the WID[1], one of the objectives is to specify the enhancements on measurement reports of UAV. During the previous meetings, the following agreements on measurement report for UAV were made [2]:

Agreements:

- Flightpath update indication in UAI is configurable by the network

- Maximum number of waypoints is set to 20 same as in LTE and number of waypoints is configurable by network as in LTE

- Flightpath information should be forwarded from source gNB to target gNB during handover. Send LS to RAN3 to check for feasibility [LS to RAN3 over email 307]

- As a baseline, we can consider a simple network control mechanism (e.g. a threshold(s)) that controls triggering the flightpath update indication in UAI. FFS if new threshold or the kind of threshold(s)

- As a baseline, single indication is used for both initial and updated flightpath available (i.e. same flag is used for initial and updated flight path indication.  FFS if further differentiation is needed if we decide to have delta signaling

In this paper, we mainly discuss the trigger condition for flight path update and the remaining issues on flight path report.

- Discussion

In RAN2#120 meeting, the following agreements on flight path reporting of NR support for UAV were made:

Agreements:

- A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message.   Flight path reporting uses at the UE Information request/response procedure as baseline.

- UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report.

- UAI message can also be used to indicate the UE has flight path availability.

In some cases, the UAV UE may change its flight path plan has changed and/or new flight path plan. From the perspective of network, this information should be informed to network. But the LTE mechanism does not support flight path updating during RRC_CONNECTION state. Accordingly, the UEAssistanceInformation (UAI) message is introduced to indicate flight path updating. UEInformationRequest and UEInformationResponse are reused to acquire the flight path.

Observation 1: The flight path updating indication can only be sent to the network via the UAI message.

After the introduction of the flight path update mechanism in NR UAV, when the UAV UE needs to report its flight path to the NW, it can send the indication to the gNB via two methods: RRC complete status messages or UAI message. In other words, the details of implementing "flight path update indication" via the UAI message are unclear. Therefore, the applicability of these two types of messages needs to be confirmed in RAN2.

Observation 2: It is necessary to clarify whether the initial flight path indication can only be sent to the gNB via RRC complete messages, and whether the flight path update indication can only be sent to the gNB via UAI message.

It was agreed in the last meeting that same flag is used for initial and updated flight path indication. But we consider that the network needs to have the ability to distinguish between the initial flight path indication and the updated flight path indication. And it was also agreed in the last RAN2 meeting to introduce the flight path information forwarding mechanism from the source gNB to the target gNB during handover. If such procedure is supported in RAN3, the target gNB needs to determine whether the current availability indication path plan in UE duplicates the flight path forwarded by the gNB. Therefore, we believe that it is necessary for the network to distinguish between initial and updated indications.

In our view, even if the same flag is used, the gNB can still determine whether the availability indication refers to the initial or updated flight path by the specific message in which the flags are included. For example, it could be specified as follows:

- When the indication flag is sent via RRC complete messages, it indicates that the currently available flight path is the initial flight path;

- When the indication flag is sent via UAI, the currently available flight path is the updated one.

Proposal 1: To agree that the initial flight path indication is sent to the network via the RRCSetupComplete, RRCResumeComplete, RRCReconfigurationComplete, or RRCReestablishmentComplete only.

The flight path updating indication was agreed to report via the UAI message. But the conditions for triggering such indication need to be further discussed. Considering that the network may sometimes not need flight path reporting or only need waypoints within coverage, it is reasonable for the network to configure the flight path update triggering conditions.

Proposal 2: To support the threshold triggered by flight updating indication to be configured by the network.

- Conclusions

Based on the discussion in the previous sections we propose the following:

Observation 1: The flight path updating indication can only be sent to the network via the UAI message.

Observation 2: It is necessary to clarify whether the initial flight path indication can only be sent to the gNB via RRC complete messages, and whether the flight path update indication can only be sent to the gNB via UAI message.

Proposal 1: To agree that the initial flight path indication is sent to the network via the RRCSetupComplete, RRCResumeComplete, RRCReconfigurationComplete, or RRCReestablishmentComplete only.

Proposal 2: To support the threshold triggered by flight updating indication to be configured by the network.

- References

- RP-213600 WI on NR Support for UAV (NR_UAV), RAN#94e, December 2021.

- R2-230xxxx Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online