3GPP TSG-RAN WG2 Meeting #122	R2-2306340

Incheon, Korea, May 22–26, 2023

Agenda item:	7.1.3

Source:	China Telecom

Title:	Consideration on remaining issues for NCR

Document for:	Discussion and Decision

- Introduction

During last few meetings, numerous agreements were achieved for NCR. In this document, we will further discuss the remaining issues for NCR and give some considerations.

- Discussion

- Wake up timer

It has been agreed that the gNB can release NCR-MT to RRC_IDLE state. However, how to trigger the NCR-MT back to RRC_CONNECTED state is undetermined. And there are two main options proposed in the last meeting.

- Opt 1: The gNB provides a wake-up timer to the NCR-MT in RRCRelease message. Upon timer expires, the NCR-MT proactively triggers RRC connection establishment.

- Opt 2: The RRC reconnection procedure is triggered by the OAM.

For opt 1, the timer is handled in AS layer, upon timer expires, the RRC of NCR-MT can send a notification to NAS of NCR-MT to trigger RRC connection establishment. In addition, the gNB can provide different wake-up timer values to support different situations. Compared to OAM-based solution, opt1 can provide more flexibility.

For opt 2, there may exist several issues. Firstly, the OAM traffic is transmitted to NCR via DRB, but the DRB for NCR is optional. Therefore, the OAM based solution may require further standardization efforts and limit the NCR implementation. Furthermore, the OAM is rather static, which cannot handle the dynamic configuration efficiently.

Proposal 1: A wake-up timer is introduced in RRCRelease message to indicate that when the NCR-MT should reconnect to the gNB.

- Backhaul beam monitoring

In the last meeting, whether the NCR-MT in RRC_INACTIVE state should perform the backhaul beam monitoring is undetermined. In the RAN2#120 meeting, it has agreed that ”After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB”. It implies that the NCR-Fwd needs to keep using the beam for backhual link indicated by the last configuration. However, how to monitor the quality of the backhaul link beam that indicated by the last configuration is undetermined. To resolve it, some companies propose that the gNB needs to perform backhaul link monitoring when NCR-MT is in RRC_INACTIVE. However, it will introduce much spec impact. In our view, the beam quality monitoring can be achieved by gNB implementation based on reciprocity of downlink and uplink beam. For example, once the gNB receives the uplink data from the UE, it can learn about the uplink beam quality and deduce the downlink backhaul link quality.

Proposal 2: NCR-MT in RRC_INACTIVE state may not perform backhaul beam monitoring in R18.

- Conclusions

The paper analyses the remaining issues related to NCR, and captures the following proposals:

Proposal 1: A new wake-up timer is introduced in RRCRelease message to indicate that when the NCR-MT should reconnect to the gNB.

Proposal 2: NCR-MT in RRC_INACTIVE state may not perform backhaul beam monitoring in R18.

- References

- Report of 3GPP TSG RAN WG2 meeting #121bis-e

- R2-2304420, Summary of [AT121bis-e][704][NCR] NCR RRC running CR (ZTE)