3GPP TSG-RAN WG2 Meeting #122	R2-2306341

Incheon, Korea, May 22–26, 2023

Agenda item:	7.18.2

Source:	China Telecom

Title:	Consideration on CP common aspects of MT-SDT

Document for:	Discussion and Decision

- Introduction

During the RAN2#121 meeting, numerous agreements were achieved for MT-SDT [1]. In this document, we further discuss the control plane common aspects for MT-SDT and give some considerations.

- Discussion

- Check for DVT

Upon reception of MT-SDT indication, the UE shall initiate the SDT procedure. However, whether the UE needs to check the data volume threshold for UL data is undetermined. Generally, there will be no UL data pending upon receiving the paging message of MT-SDT. However, the case that the UL data arrives at the same time cannot be ruled out. In this case, it would be beneficial to enable DVT check. If UL data exists and satisfies the DVT check, the SDT procedure in Rel-17 may be reused to reduce spec impact.

Proposal 1: After receiving the paging message for MT-SDT, the UE needs to check for DVT of UL data.

After receiving the paging message with MT-SDT indication, the UE needs to check the UL data in the buffer before sending the response. It has been agreed that the CG-SDT resource can be used to transmit the response. Therefore, the following two cases may need to be considered.

- Case 1: In case the UL data exists and DVT check is satisfied, SDT procedure in Rel-17 would be reused to reduce spec impact.

- If the CG-SDT resources are configured and valid, the CG-SDT is chosen to transmit the response message with new Resume Cause and the UL SDT data.

- Otherwise, if the RA-SDT resources are configured and criterias are met, the RA-SDT will be used to transmit the response message with new Resume Cause and UL SDT data.

- Otherwise, UE may use legacy RACH resource to transmit the RRCResumeRequest message with new Resume Cause.

- Case 2: If there is no UL data arrives or buffered, the following procedure would be considered.

- If the CG-SDT resources are configured and valid, the CG-SDT is chosen to transmit the response message with new Resume Cause.

- Otherwise, the UE can use legacy RACH procedure to transmit the reponse message with new Resume Cause.

In case 2, whether RA-SDT resource can be used to transmit the response message is undetermined. In our view, the network can indicate whether to use RA-SDT or legacy RACH resources to transmit the response message.

Proposal 2: If UL data exists, R17 SDT procedure can be reused to transmit the response message. If no UL data exists, CG-SDT can be used to transmit response, whether to use RA-SDT or legacy RACH resources can be indicated by the network.

- Paging message for MT-SDT

It has been agreed that the MT-SDT indication per UE is included as part of the paging record. However, whether other information can be included in paging message need further discussed.

- Indication on CG-SDT resource for paging response: It has been agreed that the CG-SDT can be used to transmit the response. As illustrated in the previous section, if CG-SDT resouces are available, it will be used to transmit the response. So, there is no need to introduce CG-SDT indication in paging message.

- Indication on RA-SDT resource for paging response: In our understanding, similar to CG-SDT, RA-SDT resource can also be used to transmit the response. However, some companies indicate that RA-SDT resouce is cell-specific, if there is no MO-SDT data, it would be better to avoid contention with MO-SDT UE that are trying RA-SDT. To compromise, it would be better to indicate whether the RA-SDT resource can be used to respond paging of MT-SDT. However, introducing RACH resource indication in paging message may have impact on RAN3. For instance, the paging message is produced by CU, but the RACH resource is allocated by DU. There may exist inconsistence between CU and DU. Therefore, it is not a good idea to transmit the RACH resource indication in paging message.

Proposal 3: For MT-SDT, there is no need to introduce CG-SDT resource or RA-SDT resource indication in the paging message.

- Resume cause

In case UE receives the MT-SDT indication in paging message but the conditions for MT-SDT are not fulfilled, the UE cannot trigger the SDT procedure. In this case, non-SDT resume procedure will be triggered. To help the network prioritize access attempts between UEs, it would be better to reuse the Resume Cause in the current spec. After the RRC connection is resumed, the network can transmit the MT-SDT data by legacy downlink data transmission procedure.

Proposal 4: If the conditions for MT-SDT are not fulfilled, UE will trigger legacy RRC Resume procedure and reuse the legacy Resume Cause.

- Conclusions

The paper analyses CP common aspects related to MT-SDT, and captures the following proposals:

Proposal 1: After receiving the paging message for MT-SDT, the UE needs to check for DVT of UL data.

Proposal 2: If UL data exists, R17 SDT procedure can be reused to transmit the response message. If no UL data exists, CG-SDT can be used to transmit response, whether to use RA-SDT or legacy RACH resources can be indicated by the network.

Proposal 3: For MT-SDT, there is no need to introduce CG-SDT resource or RA-SDT resource indication in the paging message.

Proposal 4: If the conditions for MT-SDT are not fulfilled, UE will trigger legacy RRC Resume procedure and reuse the legacy Resume Cause.

- References

- R2-2302401, Report of 3GPP TSG RAN WG2 meeting #121, Athens, Greece

- RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR