3GPP TSG-RAN WG2 Meeting #122	R2-2306344

Incheon, Korea, May 22–26, 2023

Agenda item:	6.1.1.1

Source:	China Telecom

Title:	Discussion on Security Issues for MO-SDT

Document for:	Discussion and Decision

Introduction

This paper aims to discuss the security issues on MO-SDT in Rel-17 and provides some proposals.

Discussion

In legacy non-SDT mechanism, the control plane connection between UE and the network shall be established first, and then UE shall need to active UP encryption and/or UP integrity protection for subsequent user plane data transmission based on the network configuration. For state transition from inactive to connected, the legacy UE derives the security keys for data transmission based on NCC value and target cell information when the RRCResume message received. This is a classical design solution for data transmission. During SDT procedure, UE needs to send the RRCResumeRequest signalling and UL data packet simultaneously. According to the agreements in RAN2#112 meeting [1], the following agreements on security handling for first UL data transmission were achieved:

6 	In case of RRC-based solution, for both RACH and CG based solutions, the CCCH message contains ResumeMAC-I generated using the stored security key for RRC integrity protection – i.e same as Rel-16.

7    For both RACH and CG based solutions, new keys are generated using the stored security context and the NCC value received in the previous RRCRelease message (i.e. same as legacy procedure) and these new keys are used for generating the data of DRBs that are configured for SDT.

As mentioned above, UL data packets transmission use the legacy procedure to perform security protection. Unfortunately, the stage 2 description for SDT in TS38.300 does not clearly address whether the security protection for UL data is needed. Moreover, all other related specifications, i.e., 38.331, 33.501, only specify the DRB security protection behaviour for legacy non-SDT mechanism. Therefore, in order to reflect the previous agreements, we propose to add stage 2 description on security protection for UL data packets transmission in TS38.300.

Proposal 1: to add stage 2 description on security protection for UL data packets transmission in TS38.300.

Proposal 2: to agree the corresponding CR [2].

Proposal

Based on the above analysis, we have the following proposal:

Proposal 1: to add stage 2 description on security protection for UL data packets transmission in TS38.300.

Proposal 2: to agree the corresponding CR [2].

Reference

- R2-2100001, Report of 3GPP TSG RAN WG2 meeting #112-e, Nov 2-13, 2020

- R2-2306345, Corrections to Security Issues for MO-SDT