3GPP TSG-RAN WG2 Meeting #122	R2- 2306352

Incheon, Korea, 22nd – 26th May, 2023

Agenda Item:	7.7.4.1.1

Source:	ETRI

Title:	Discussion on TN area information for the NTN-TN cell re-selection enhancements

Document for:	Discussion and Decision

1	Introduction

In the past meetings, the followings are agreed [1, 2].

RAN2 #121 agreements:

Agreements:

- TN coverage area information will be associated to the frequency information.

RAN2 #121-bis-e agreements:

Agreements:

1.	For signaling the TN coverage, the corresponding geographical area information is provided by broadcast signalling by the network via a list of (possibly overlapping) areas where each area is defined using center location coordinates + radius (where the area is meant to describe a group of cells, not just a single one). FFS on the SIB. FFS on whether additional information in dedicated signalling is needed/useful

Agreements via email – from offline 106:

- Area center location and its radius for TN coverage information is signalled using Ellipsoid-Point and radius separately. FFS if Rel-17 referenceLocation and distanceThresh are directly reused

- Decision on the size of TN coverage area list is postponed until more is known on the format of this information and how is it sent.

In this paper, we provide our views on NTN-TN cell re-selection enhancements, specifically, on how to describe TN areas based on the above agreements.

2	Discussion

2.1	Consideration on the circle-shaped TN area representation

In the previous meeting [2], RAN2 agreed to describe the TN coverage using a list of TN areas where a TN area is described by center location coordinates + radius, i.e., a circle. There will be various shapes of actual TN coverages which need to be described in terms of circle-shaped TN areas. Regardless of the shape, what is obvious is that accurate description of TN coverage is preferred. This is because a UE can avoid unnecessary measurements for TN.

Figure 1. (a) Example TN coverage, (b) example TN coverage description using a single circle confined inside the NTN cell coverage, and (c) example TN coverage using one circle with flexibility.

Figure 1 (a) shows an example TN coverage which is shaped as a cape. Note that a cape is a geographically common coastal landform. Let say a circle representing a TN area must be inside of the NTN cell coverage. Figure 1 (b) shows an example of TN coverage description using a single circle while satisfying this condition. In Figure 1 (c), NW is assumed to have flexibility in positioning the circle. Comparing with Figure 1 (b), Figure 1 (c) shows a more accurate description of the TN coverage.

Allowing a part of a circle-shaped TN area to exist outside of the NTN cell coverage increases the flexibility of NW in describing TN coverage, which enables more accurate description of the TN coverage.

A part or all of TN coverage area can overlap the NTN cell coverage. Accordingly, the center location of TN coverage area may be outside of the NTN cell coverage.

A circle representing a TN area may exist outside of the NTN cell coverage. Consequently, the radius of the circle may be much larger than the radius of the NTN cell coverage, which needs to be analysed. The analysis will be the basis for determining the required number of bits to be used for the radius of the circle.

The radius of a circle-shaped TN area may be larger than the radius of the NTN cell coverage, which needs to be analysed before determining the format of the radius.

The radius of a circle-shaped TN area may be larger than the radius of NTN cell coverage.

2.2	Flexible description of the TN coverage

Figure 2. TN coverage description examples illustrating TN area outside of the actual TN coverage.

The size of TN area list broadcasted through SIB is supposed to be limited. Furthermore, as shown in Figure 2, the circle-shaped TN area representation has its own limitation in accurate description of the TN coverage. The more the TN area outside of the actual TN coverage, the more unnecessary measurements for TN may occur.

Too coarse description of the TN coverage highly likely to be inevitable because the TN coverage is described in terms of circle-shaped TN areas and the size of TN area list broadcasted through SIB is limited.

Figure 3. Example of more detailed description of the TN coverage by using NTN-only area.

With the given size of TN area list, the description accuracy of the TN coverage can be significantly improved by introducing an NTN-only area. The NTN-only area refers to an area that does not belong to the TN coverage and belongs only to the NTN coverage. An NTN-only area overrides a TN area. That is, if an NTN-only area overlaps a TN area, the overlapped area is not a TN area but an NTN-only area.

Figure 3 is an example showing the benefit brought by the NTN-only area. Comparing with the examples in Figure 2 (b) and Figure 2 (c), the number of used circles remains the same, 3. However, the TN area outside of the actual TN coverage is significantly reduced. Consequently, introduction of the NTN-only area provides NW with more flexibility, which lead to higher accuracy in describing the TN coverage.

Representation of an NTN-only area can neither require a new information element format nor increase the size of TN area list in SIB. Since an NTN-only area can be represented by a TN area with no associated frequency, the information element format of a TN area can be shared with an NTN area. Accordingly, some elements in TN area list can be used for representing NTN-only areas, and thus the size of TN area list in SIB can remain the same.

Introduction of an NTN-only area enables NW to have more flexibility and higher accuracy in describing the TN coverage, where an NTN-only area refers to an area that does not belong to the TN coverage and belongs only to the NTN coverage.

Information for NTN-only areas is provided by broadcast signalling, where an NTN-only area refers to an area that does not belong to the TN coverage and belongs only to the NTN coverage.

TN area list can include NTN-only areas without increasing the size of the list.

3	Conclusions

In this paper, the following observations are made:

Observation 1	Allowing a part of a circle-shaped TN area to exist outside of the NTN cell coverage increases the flexibility of NW in describing TN coverage, which enables more accurate description of the TN coverage.

Observation 2	The radius of a circle-shaped TN area may be larger than the radius of the NTN cell coverage, which needs to be analysed before determining the format of the radius.

Observation 3	Too coarse description of the TN coverage highly likely to be inevitable because the TN coverage is described in terms of circle-shaped TN areas and the size of TN area list broadcasted through SIB is limited.

Observation 4	Introduction of an NTN-only area enables NW to have more flexibility and higher accuracy in describing the TN coverage, where an NTN-only area refers to an area that does not belong to the TN coverage and belongs only to the NTN coverage.

In this paper, we propose the followings:

Proposal 1	A part or all of TN coverage area can overlap the NTN cell coverage. Accordingly, the center location of TN coverage area may be outside of the NTN cell coverage.

Proposal 2	The radius of a circle-shaped TN area may be larger than the radius of NTN cell coverage.

Proposal 3	Information for NTN-only areas is provided by broadcast signalling, where an NTN-only area refers to an area that does not belong to the TN coverage and belongs only to the NTN coverage.

Proposal 4	TN area list can include NTN-only areas without increasing the size of the list.

References

R2-2301901, Report from Break-Out Session on NTN, IoT NTN and RedCap, RAN2 #121, Athens, Greece, 27th Feb. - 3rd Mar., 2023.

R2-2304201, Report from Break-Out Session on NR NTN and IoT NTN, RAN2 #121bis-e, Online, 17th - 26th Apr., 2023.