3GPP TSG RAN WG2 Meeting #122	R2-2306353

Incheon, Korea, May 22-26, 2023

Agenda Item:	7.5.4.1 (BSR Enhancements for XR)

Source: 	Samsung

Title: 	Discussion on BSR enhancements for XR

Document for:	Discussion & decision

Introduction

In the last RAN2 meeting, we discussed about whether new BS table is pre-defined or dynamically constructed, and how to construct the new BS table. In this paper, we propose more detailed operations on 1) new BS table, and 2) delay reporting.

New BS Table

Flexibility: The amount of XR traffic is highly dependent on the XR data rate, which can vary largely if, for example, corresponding frame rate changes, such that it is difficult to create one or more fixed BS tables that are tailored for all the possible XR data rates. Dynamic BS table can be adaptively constructed based on NW configuration, considering the XR traffic characteristics, thus more easily embracing various traffic characteristics, and providing higher grained BS information than fixed BS tables.

Scalability: The evolution of XR traffic model may continue, and possibly, introduce higher data rates. At that time, if the necessity of new fixed BS table(s) arises for the new traffic models, we may repeat the same discussion as now, which is inefficient from the technological evolution point of view. With dynamic BS table, we can cope with the future traffic models and data rates by reusing the entirely same philosophy of dynamic BS table.

Observation 1: Dynamic BS table is superior compared to fixed BS table from the flexibility and the scalability points of view.

From the UE side, a dynamic BS table can be constructed based on the NW configurations. The NW configurations can include configurable BS table parameters used in dynamic BS table construction, which can include Bmin, i.e., the minimum non-zero BS value, Bmax, i.e., the maximum BS value. Also, the new BS table includes N code points. As an example, if the code points are evenly distributed (i.e., uniform distribution), the difference between two adjacent BS values can be (Bmax-Bmin)/(N-1).

An example for the BS table constructed by Bmin and Bmax is given below, where the BS values corresponding to indices 0 and N-1, respectively, are configurable by NW.

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | ≤ Bmin | 8 | ≤ 102 | 16 | ≤ 442 | ... | ... |
| 1 | ... | 9 | ≤ 142 | 17 | ≤ 482 | ... | ... |
| ... | ... | 10 | ≤ 182 | 18 | ≤ 522 | ... | ... |
| ... | ... | 11 | ≤ 222 | 19 | ≤ 562 | ... | ... |
| ... | ... | 12 | ≤ 262 | 20 | ≤ 602 | ... | ... |
| ... | ... | 13 | ≤ 302 | 21 | ≤ 642 | ... | ... |
| ... | ... | 14 | ≤ 342 | 22 | ≤ 682 | N-2 | ... |
| ... | ... | 15 | ≤ 382 | 23 | ≤ 722 | N-1 | ≤ Bmax |

Note that the number of code points N of the new BS table can be implicitly determined by the length of the buffer size field of BSR, i.e., N = 2L given that the buffer size field is L-bit, thus no need to be configured by NW to reduce signalling overhead.

Proposal 1: New BS table is dynamically built based on NW configurations, which include configurable BS table parameters including minimum non-zero BS value and maximum BS value.

For a certain LCG able to refer new BS table(s), it may have more than one option when determining which BS table to refer. It is reasonable for UE to refer the best one providing the finest granularity for the buffer size, among multiple options, e.g., legacy BS tables and one or more new BS tables. Accordingly, we need a new BSR format to include information on which BS table is used.

Proposal 2: RAN2 is kindly asked to consider a new BSR format that can include information on which BS table is used.

Delay Reporting

The motivation behind the delay reporting is to assist timely UL scheduling for urgent XR traffic.

In addressing the above issue, a solution relying only on the buffer size information should be given a higher priority than that involving delay information, since the new BSR design becomes much simpler when only the buffer size information is of interest.

The candidate solution we can think of is UE can report the amount of buffered data whose remaining time satisfying certain condition, e.g., less than a certain threshold, to inform gNB of the amount of urgent data, where the urgency is defined by gNB with the remaining time threshold. By reporting the amount of urgent data, UE allows gNB to prioritise it in UL scheduling, thus facilitating the timely UL scheduling.

Proposal 3: In achieving the timely UL scheduling for urgent XR traffic, RAN2 is kindly asked to give higher priority to the solution relying only on the buffer size information than that involving delay information.

Proposal 4: In achieving the timely UL scheduling, RAN2 is kindly asked to consider that UE reports the amount of urgent data whose remaining time satisfying certain condition configured by gNB, e.g., less than a certain threshold.

If the outcome of the discussion tends to give higher weight on the delay reporting, we propose to report remaining time as the delay information. For the definition, remaining time can refer to the remaining PSDB of PDU set, or PDB of PDU if the concept of PDU set is not applicable. With only remaining time, NW may face ambiguity on how much UL resource should be allocated regarding the urgent traffic. It makes more sense to also report the amount of buffered data corresponding to the reported remaining time for the LCG that remaining time is reported.

Proposal 5: If it is agreed to report delay information, RAN2 is kindly asked to consider reporting remaining time, defined as the remaining PSDB/PDB of the PDU set/PDU. The amount of the buffered data corresponding to the reported remaining time can also be reported if necessary.

In both cases, i.e., reporting the amount of urgent data (Proposal 4) and reporting the remaining time (Proposal 5), we need a means of compromise to reduce signalling overhead, to avoid signalling overhead overwhelming the potential gain.

For example, since the arrival time of the buffered data can be different, so does the remaining time. If we report remaining time regarding all the buffered data per LCG/LCH, it may introduce multiple remaining time fields per LCG/LCH. As a means of compromise, we can consider reporting the shortest remaining time among the buffered data per LCG. Similarly, it seems necessary to discuss, at which level, the amount of urgent data should be reported. For example, it can be reported per LCG/LCH, or for all the LCGs, considering the signalling overhead and potential gain.

Proposal 6: RAN2 is kindly asked to discuss about, at which level, the remaining time and/or the amount of urgent data should be reported, considering signalling overhead and potential gain.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: Dynamic BS table is superior compared to fixed BS table from the flexibility and the scalability points of view.

Proposal 1: New BS table is dynamically built based on NW configurations, which include configurable BS table parameters including minimum non-zero BS value and maximum BS value.

Proposal 2: RAN2 is kindly asked to consider a new BSR format that can include information on which BS table is used.

Proposal 3: In achieving the timely UL scheduling for urgent XR traffic, RAN2 is kindly asked to give higher priority to the solution relying only on the buffer size information than that involving delay information.

Proposal 4: In achieving the timely UL scheduling, RAN2 is kindly asked to consider that UE reports the amount of urgent data whose remaining time satisfying certain condition configured by gNB, e.g., less than a certain threshold.

Proposal 5: If it is agreed to report delay information, RAN2 is kindly asked to consider reporting remaining time, defined as the remaining PSDB/PDB of the PDU set/PDU. The amount of the buffered data corresponding to the reported remaining time can also be reported if necessary.

Proposal 6: RAN2 is kindly asked to discuss about, at which level, the remaining time and/or the amount of urgent data should be reported, considering signalling overhead and potential gain.