3GPP TSG-RAN WG2 Meeting #122	 R2-2306357

Incheon, Korea, May 22 – 26, 2023

Source:	KDDI Corporation

Title:	Discussion on RACH-less HO for mIAB

Tdoc Type:	Discussion

Agenda Item:	7.12.2.1 Connected mode

Document for:  Discussion and decision

Release:       Rel-18

1	Introduction

In this contribution, we will discuss and share our views on RACH-less HO for mIAB.

2	Discussion

In last RAN2 meeting, RAN2 agree to discuss RACH-less HO as follows.

Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB). RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration: - Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details. - Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command). RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

We think that LTE RACH-less mechanism can be a base line for RACH-less HO for mIAB. With this mechanism, preallocated uplink grant and TA information is indicated RRC message with RACH-Skip-r14. So, we think that this method can be also adopted to 5G.

TS36.331

RACH-Skip-r14 ::=					SEQUENCE {

targetTA-r14					CHOICE {

ta0-r14							NULL,

mcg-PTAG-r14						NULL,

scg-PTAG-r14						NULL,

mcg-STAG-r14					STAG-Id-r11,

scg-STAG-r14					STAG-Id-r11

},

ul-ConfigInfo-r14				SEQUENCE {

numberOfConfUL-Processes-r14			INTEGER (1..8),

ul-SchedInterval-r14			ENUMERATED {sf2, sf5, sf10},

ul-StartSubframe-r14			INTEGER (0..9),

ul-Grant-r14					BIT STRING (SIZE (16))

}																OPTIONAL	-- Need OR

}

ul-Grant Indicates the resources of the target PCell/PSCell to be used for the uplink transmission of PUSCH [23], clause 8.8.

ul-SchedInterval Indicates the scheduling interval in uplink, see TS 36.321 [6], clause 5.20. Value in number of sub-frames. Value sf2 corresponds to 2 subframes, sf5 corresponds to 5 subframes and so on.

ul-StartSubframe Indicates the subframe in which the UE may initiate the uplink transmission, see TS 36.321 [6], clause 5.20. Value 0 corresponds to subframe number 0, 1 correponds to subframe number 1 and so on. The subframe indicating a valid uplink grant according to the calculation of UL grant configured by ul-StartSubframe and ul-SchedInterval, see TS 36.321 [6], clause 5.20, is the same across all radio frames.

numberOfConfUL-Processes The number of configured HARQ processes for preallocated uplink grant, see TS 36.321 [6], clause 5.20. This field is applicable if a UE is configured with asynchronous HARQ, otherwise it shall be ignored.

Proposal1: Regarding RACH-less HO for mIAB, RAN2 agree the followings.

- gNB can indicate preallocated uplink grant by RRC message.

- gNB can indicate the same TA as the source cell by RRC message.

With regard to the information for preallocated uplink grant, we think three information, configured HARQ process, time reference slots for a pre-allocated PUSCH transmission, UL grant information (UL Grant 27 bits field specified in TS38.321) are necessary at least. We understand that in the case of PUSCH scheduled by RAR UL grant, UE derives reference slot n for the PUSCH transmission based on the corresponding PRACH transmission timing. But, in the RACH-less HO case, since without PRACH transmission, alternative information is needed for UE to derive reference slot n.

TS38.213

With reference to slots for a PUSCH transmission scheduled by a RAR UL grant, if a UE receives a PDSCH with a RAR message ending in slot n for a corresponding PRACH transmission from the UE, the UE transmits the PUSCH in slot n+k2+∆+2μ∙Kcell,offset, where k2 and ∆ are provided in [6, TS 38.214] and Kcell,offset is provided by CellSpecific_Koffset; otherwise, if not provided, Kcell,offset=0.

Proposal2: RAN2 agree that gNB indicates the following information for preallocated uplink grant

- configured HARQ process

- reference slots for a pre-allocated PUSCH transmission

- UL grant information (UL Grant 27 bits field specified in TS38.321)

Once we agree on RAN2 aspects, then we believe that RAN2 should ask RAN1 to check the above RAN2 agreements, since this also impacts on RAN1 specs.

Proposal3: RAN2 agree to ask RAN1 to check whether the above RAN2 agreements are feasible.

One important thing that we should discuss is whether RACH-less HO is dedicated to mIAB or can be applied to other use cases. In our view, since RACH-less HO is not supported in NR, so from operators’ perspective it’s good to consider the same scenarios which are addressed by LTE RACH-less HO.

Proposal4: RAN2 agree RACH-less HO can be applied to other use case rather than mIAB.

For beam information, we prefer to adopt Option 1: (Explicit approach). Even those we can assume the same beam as the source will be used for the target cell also, but with explicit approach is future-proof. We are also fine with Option 1: (Explicit approach) having a note in a spec mentioning that in release 18 specs, gNB will indicate the same beam as the source.

Proposal5: RAN2 agree to adopt Option 1: (Explicit approach) Explicit beam information is included in HO command.

RAN2/1 are now having a similar discussion on contents of cell switch command under WI Further NR mobility enhancements. Even though, the discussion is for intra-CU, and cell switch command is conveyed by MAC CE, but we still believe that the beam information field itself can be similar one. Specifically in RAN#112bis-e, RAN1 agreed that “1 joint or 1 pair of UL and DL unified TCI State index for the target Cell” is included in the cell switch command. So, we may include a similar information in RRC message for RACH less HO.

Proposal6: Regarding beam information of the target cell provided by RRC, RAN2 agree to reuse the contents included in cell switch command, which is now under mobility discussion.

Chair's notes RAN1#112bis-e

Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell - FFS: C-RNTI - FFS: the presence of each field (i.e. always present or configurable)

3	Conclusion

We make the following observations and proposals:

Proposal1: Regarding RACH-less HO for mIAB, RAN2 agree the followings.

- gNB can indicate preallocated uplink grant by RRC message.

- gNB can indicate the same TA as the source cell by RRC message.

Proposal2: RAN2 agree that gNB indicates the following information for preallocated uplink grant

- configured HARQ process

- reference slots for a pre-allocated PUSCH transmission

- UL grant information (UL Grant 27 bits field specified in TS38.321)

Proposal3: RAN2 agree to ask RAN1 to check whether the above RAN2 agreements are feasible.

Proposal4: RAN2 agree RACH-less HO can be applied to other use case rather than mIAB.

Proposal5: RAN2 agree to adopt Option 1: (Explicit approach) Explicit beam information is included in HO command.

Proposal6: Regarding beam information of the target cell provided by RRC, RAN2 agree to reuse the contents included in cell switch command, which is now under mobility discussion.