3GPP TSG-RAN WG2 Meeting #122e	R2-2306358

# Incheon, Korea, 22 - 26 May, 2023

Agenda Item:    7.13.7

Source:         Xiaomi

Title:         	Discussion on the “LS on potential override of logged MDT reports upon moving from SNPN to PLMN” from RAN3 (R3-232118)

Document for:	Discussion and Decision

Introduction

In last meeting, RAN3 sent an LS to ask about how to avoid the loss of stored logged MDT reports during the mobility between different types of networks. Details as follows: [1]

RAN3 is currently working on ensuring SON/MDT support for non-public networks.

In this work, RAN3 noticed that logged MDT reports collected in one type of network, e.g. an SNPN, are lost when UE moves to a network of another type, e.g. a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type.

RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

In this contribution, we provide our consideration on the RAN3 LS and provide the draft LS reply in the annex.

Discussion

For the issue of resource waste due to non-real-time retrieval during the mobility between different types of networks, some companies provided their considerations on this issue in the previous meetings, but we had never discussed about this officially.

For the mobility between the SNPN and PLMN, as mentioned in the RAN3 LS, UE needs to deregister from the old network first. And in this case, the logged MDT configuration and the logged measurements information will be discarded without retrieval, as specified in TS 38.331 quoted below.

TS 38.331

The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or upon deregistration.

The UE shall:

1>	stop timer T330, if running;

1>	if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

Actually, this issue also exists for the existing logged MDT report in the case of mobility between PLMNs. Currently, for the mobility between PLMNs which are not the equivalent PLMNs, UE also needs to perform the deregistration process in the previous PLMN first, and then perform the registration process in the new PLMN. This can ensure that UE correctly receives services in the new PLMN and releases resources in the previous PLMN. In such case, the logged MDT report in the previous PLMN will also be discarded without retrieval when switching to the new PLMN.

As there is no enhancement mechanism on this in legacy and it is up to network implementation on the logged MDT report retrieval, the motivation of introducing the enhancement for the logged MDT report during the mobility between SNPN and PLMN needs to be clarified, otherwise, we can also leave it to the network implementation in such scenario.

Observation 1: The loss of stored logged MDT reports also exists for the mobility between PLMNs upon deregistration, and it is left to network implementation on the logged MDT report retrieval.

Proposal 1: The motivation to have the enhancement for logged MDT report for the mobility between the SNPN and PLMN needs to be further clarified by RAN3.

Proposal 2: RAN2 considers the draft LS reply to RAN3 in the annex.

Conclusions

In this contribution, we provide our analysis for RAN3 LS on the potential override of logged MDT reports upon moving from SNPN to PLMN, and have the observation and proposals as follows:

Observation 1: The loss of stored logged MDT reports also exists for the mobility between PLMNs upon deregistration, and it is left to network implementation on the logged MDT report retrieval.

Proposal 1: The motivation to have the enhancement for logged MDT report for the mobility between the SNPN and PLMN needs to be further clarified by RAN3.

Proposal 2: RAN2 considers the draft LS reply to RAN3 in the annex.

References

R2-2304628 LS on potential override of logged MDT reports upon moving from SNPN to PLMN (R3-232118; contact: Ericsson).

Annex: Draft LS reply

3GPP TSG-RAN WG2 Meeting #122e	R2-23xxxxx

Incheon, Korea, 22 - 26 May, 2023

Title:	[Draft] Reply LS on the potential override of logged MDT reports upon moving from SNPN to PLMN

Response to:	R2-2304628(R3-232118) “LS on potential override of logged MDT reports upon moving from SNPN to PLMN”

Release:	Release 18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

Name:                   <name>

E-mail Address:   <email>

1. Overall Description:

RAN2 would like to thank RAN3 for the LS on the potential override of logged MDT reports upon moving from SNPN to PLMN.

Regarding to the loss of stored logged MDT reports during the mobility between different types of networks, RAN2 discussed about the issue and observed that the same issue also exists for the mobility between PLMNs upon deregistration, and it is up to network implementation on the logged MDT report retrieval.

Based on the above, RAN2 kindly asks RAN3 to clarify the motivation to have the enhancement for logged MDT during mobility between SNPN and PLMN.

2. Actions:

To RAN WG3

ACTION:  RAN2 kindly asks RAN3 to take above into account and provide the feedback on the clarification accordingly.

3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #123	21 Aug – 25 Aug 2023			Toulouse, FR

TSG RAN WG2 Meeting #123-bis	09 Oct –13 Oct 2023			Xiamen , CN