3GPP TSG-RAN WG2 Meeting #122	R2-2306359

Incheon, South Korea, 22– 26 May 2023

Agenda item:	6.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	MBS Scell Reception

WID/SID:	NR_MBS-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#121bis-e we discussed MBS broadcast reception and it was concluded that .

Postpone discussion on whether the change is needed for plmn-Index with MBS broadcast reception on SCell in Rel-17. If we agree to fix it, we do this in ASN.1 BC way.

# 2	Discussion

For broadcast reception on SCell the UE acquires the MCCH on SCell, but the UE does not receive SIB1 of the SCell. In case plmn-Index is used for a PLMN/SNPN on MCCH on SCell, then the UE cannot determine correct PLMN/NPN necessarily for  the TMGI.

Observation 1: It is possible that for SCell UE is not required to acquire SIB1 and thus getting mapping of. plmn-Index of TMGI to PLMN/NPN may not be possible

If the TMGI only addresses PLMNs then it is always possible to explicitly signal also PLMN identity and thus there is no issues for that case but NPN use case this is nto possible.

Observation 2: It is possible to avoid the problem by using explicit PLMN identity signaling in MCCH but this is not possible for NPN use scenario as there is no expliciti NPN signaling in the NPN

So if we need to support the scenario where scell reception of broadcast may occur and plmn-index mapping is not guaranteed to be same as in PCell we have at least following options to solve the issue:

- UE receiving MBS from SCell where TMGI is using plmn-Index coding would receive SIB1 of the SCell to learn the proper mapping

- NW provides SIB1 of SCell to the UE e.g. only to UEs indicating interest to receive MBS form SCell (this would be similar as currently SIB20 provision for SCell)

- NW provides mapping of SCell PLMN-indexes to actual PLMN/SNPN.

- NW will use in such a scenario explicit coding of PLMN and NPN – Add explicit NPN identity to he TMGI coding

All of the options are NBC changes as the UE behaviour is changed. Additionally options 2-4 need to change ASN.1.

Option 1 would be easiest from ASN.1 point of view but most impactful to UEs as in some scenarios UE would need to acquire SCell SIB1.

Observation 3: Option 1 is easiest solution from ASN.1 point of view ( no changes) but it would require UEs to acquire SIB1 from SCells  and if UE receives MBS broadcast from SCell and NW uses plmn-Index in MCCH for TMGI

To avoid requirement to read SIB1 we could simply add SIB1 provision for SCells as optional to avoid UEs to read SIB1 similarly to SIB20 provision for SCell. Already in R15 we had SIB1 delivery for PCell added and we could follow similar structure:

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Cond DRX-Config2

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

sCellSIB1-r17                    SetupRelease [ ScellSib1-r17 }

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

SCellSIB1-r17 ::= OCTET STRING (CONTAINING SIB1)

One may consider that overhead is too big in this scenario as one would only need PLMN/NPN identity list. That would reduce some overhead although maybe that is not really a big issue in dedicated signaling. Also problem with this approach is that whenever any changes to PLMN/NPN/SNPN coding are done one would need to change coding. To circumvent this it is easiest likely just to provide then part of SIB1 e..g. CellAccessRelatedInfo that include all the required information and also possible extension introduced in future:

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

sCellSIB1-CellAccess-r17                    SetupRelease [ ScellSib1-CellAccess-r17 }

]]

}

SCellSIB20-r17 ::= OCTET STRING (CONTAINING SystemInformation)

SCellSIB1-CellAccess-r17 ::= OCTET STRING (CONTAINING CellAccessRelatedInfo)

Observation 4: To avoid SIB1 reading of SCell one could provide either Scell SIB1 or e.g. cellAccessRelatedInfo part of SIB1 to the UE

Another method is to to include explicit NPN identity in the TMGI coding as then NW can always be sure regardless of scenario that UE has correct understanding of used identity. This would also simplify the handling between gNBs as one could always rely on using explicit identity if needed. This way one could avoid any ambiquity about using plmn-Index if there is such a scenario.

TMGI-r17 ::=                     SEQUENCE {

plmn-Id-r17                      CHOICE {

plmn-Index                       INTEGER (1..maxPLMN),

explicitValue                    PLMN-Identity

},

serviceId-r17                    OCTET STRING (SIZE (3))

}

TMGI-v17xy ::=                     SEQUENCE {

explicitValue-v17xy                  NPN-Identity

}

}

Then this TMGI-v17xy would be added to the MBS-SessionInfo-r17, MRB-ToAddMod-r17, Paging and MBS-SessionInfoList-r17 and whenever new TMGI-v17 is used the plmn-id-r17 would use v17xy as the identity (i.e. for NPN use case). From coding perspective it might be better to introduce new versions for e.g. MBS-SessionInfoList-r17 and PagingGroupList-r17. From ASN.1 changes point of view this is biggest change but will be also most future robust.

Observation 5: To avoid the problem of using plmn-Index in case of SCell one could add explicit NPN-Identity in the TMGI

Then of course one could question whether solving the issue is something that needs solving at all. As said for PLMN use case one can always use explicit PLMN identity signaling in the TMGI. This would not allow usage of plmn-Index although in some scenarios and thus adding some overhead. For MBS broadcast reception from SCell belonging to SNPN would be limited to scenario where one can use same mapping of plmn-Index in PCell and SCell.

Proposal 1: Discuss whether we need to support using plmn-index for SCell MBS broadcast reception and if this seen necessary which solution to adopt

.

# 3	Conclusion

Observation 1: It is possible that for SCell UE is not required to acquire SIB1 and thus getting mapping of. plmn-Index of TMGI to PLMN/NPN may not be possible

Observation 2: It is possible to avoid the problem by using explicit PLMN identity signaling in MCCH but this is not possible for NPN use scenario as there is no explicit NPN signaling in the NPN

So if we need to support the scenario where scell reception of broadcast may occur and plmn-index mapping is not guaranteed to be same as in PCell we have at least following options to solve the issue:

- UE receiving MBS from SCell where TMGI is using plmn-Index coding would receive SIB1 of the SCell to learn the proper mapping

- NW provides SIB1 of SCell to the UE e.g. only to UEs indicating interest to receive MBS form SCell (this would be similar as currently SIB20 provision for SCell)

- NW provides mapping of SCell PLMN-indexes to actual PLMN/SNPN.

- NW will use in such a scenario explicit coding of PLMN and NPN – Add explicit NPN identity to he TMGI coding

Observation 3: Option 1 is easiest solution from ASN.1 point of view ( no changes) but it would require UEs to acquire SIB1 from SCells  and if UE receives MBS broadcast from SCell and NW uses plmn-Index in MCCH for TMGI

Observation 4: To avoid SIB1 reading of SCell one could provide either Scell SIB1 or e.g. cellAccessRelatedInfo part of SIB1 to the UE

Observation 5: To avoid the problem of using plmn-Index in case of SCell one could add explicit NPN-Identity in the TMGI

Proposal 1: Discuss whether we need to support using plmn-index for SCell MBS broadcast reception and if this seen necessary which solution to adopt